Piper PA-28-200R Arrow — John_Peck_N98JG_Piper_PA-28R_Preflight_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Night/IFR Check Items

[ ]  Master Switch .... ON

AIRCRAFT LIGHTING

[ ]  Navigation Lights (3) .... ON & CHECK

[ ]  Fin Strobe .... ON & CHECK

[ ]  Wingtip Strobes .... ON & CHECK

[ ]  Landing Light .... ON & CHECK

[ ]  Pitot Heat .... ON & CHECK

[ ]  Master Switch .... OFF

Cabin

[ ]  Aircraft Status .... AIRWORTHINESS CERTIFICATE & REGISTRATION

[ ]  Hobbs & Tach .... VERIFY READINGS

[ ]  Control Wheel Locks .... REMOVE

[ ]  Ignition and Master Switches .... OFF

[ ]  Electrical Switches .... ALL OFF

[ ]  Avionics Master .... OFF

[ ]  Landing Gear Switch .... DOWN

[ ]  Master Switch .... ON

[ ]  Landing Gear Lights .... 3 GREEN

[ ]  Fuel Quantity Indicators .... CHECK QUANTITY

[ ]  Master Switch .... OFF

[ ]  Flaps .... DOWN

Right Wing

[ ]  Flap .... CHECK

[ ]  Aileron .... CHECK

[ ]  Wing Leading Edge .... CHECK

[ ]  Wing Tie Down .... DISCONNECT

[ ]  Fuel Vent Opening .... CLEAR

[ ]  Fuel Quantity .... CHECK LEVEL

[ ]  Fuel Tank Sump Drain .... TAKE SAMPLE

[ ]  Main Wheel Strut and Tire .... CHECK

[ ]  Landing Gear & Door .... CHECK

Left Wing

[ ]  Main Wheel Strut and Tire .... CHECK

[ ]  Landing Gear & Door .... CHECK

[ ]  Fuel Vent Opening .... CLEAR

[ ]  Left Wing Fuel Quantity .... CHECK LEVEL

[ ]  Left Wing Fuel Tank Sump Drain .... TAKE SAMPLE

[ ]  Wing Tie Down .... DISCONNECT

[ ]  Pitot/Static Tube .... REMOVE COVER, CHECK

[ ]  Wing Leading Edge .... CHECK

[ ]  Stall Warning Switch .... FREE

[ ]  Aileron .... CHECK

[ ]  Flap .... CHECK

Fuselage and Empennage

[ ]  Landing Gear Sensor .... REMOVE COVER, CHECK

[ ]  Fuselage .... CHECK CONDITION

[ ]  Tail Tie Down .... DISCONNECT

[ ]  Control Surfaces .... CHECK

[ ]  Baggage Door .... CLOSE AND SECURE

Nose

[ ]  Engine Oil .... CHECK LEVEL – 6 QT MIN

[ ]  Propeller and Spinner .... CHECK

[ ]  Landing light .... CHECK SECURITY

[ ]  Engine Inlets .... CHECK

[ ]  Nose Wheel Strut and Tire .... CHECK

[ ]  Landing Gear & Doors .... CHECK

[ ]  Fuel Strainer Drain .... DRAIN

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-200R Arrow/John_Peck_N98JG_Piper_PA-28R_Preflight_Checklist.txt

Rights: licensed

Checklist hash: 91cdf73fb1e1824ffd5784cef006ff5d88acbb7f31e753ebba8c1337fb664912

Verify against the aircraft's own approved documentation before use.