Piper PA-28-200R Arrow — John_Peck_N98JG_Piper_PA-28R_W&B_Automatic__Calculations

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Weight & Balance

[ ]  Licensed Empty Weight .... 1535.7

[ ]  Licensed Empty Weight Moment .... 132885

[ ]  Quarts Oil (8 Maximum) .... 8

[ ]  Quarts Oil Weight .... 15

[ ]  Quarts Oil Arm .... 29.5

[ ]  Quarts Oil Moment .... 442.5

[ ]  Pilot and Front Passenger Arm .... 85.5

[ ]  Pilot and Front Passenger Moment .... 0

[ ]  Rear Seat Passengers Arm .... 118.1

[ ]  Rear Seat Passengers Moment .... 0

[ ]  Gallons Fuel (50 Maximum) .... 50

[ ]  Gallons Fuel Weight .... 300

[ ]  Gallons Fuel Arm .... 95

[ ]  Gallons Fuel Moment .... 28500

[ ]  Baggage Arm .... 142.8

[ ]  Baggage Moment .... 0

[ ]  Total Weight and Moment .... 1850.7

[ ]  Total Moment .... 161827.5

[ ]  Resulting Center of Gravity .... 87.44123845031609

ENVELOPE

[ ]  CG Envelope Point .... 81

[ ]  Weight Envelope Point .... 1400

[ ]  CG Envelope Point .... 81

[ ]  Weight Envelope Point .... 1940

[ ]  CG Envelope Point .... 82

[ ]  Weight Envelope Point .... 2000

[ ]  CG Envelope Point .... 84

[ ]  Weight Envelope Point .... 2150

[ ]  CG Envelope Point .... 86

[ ]  Weight Envelope Point .... 2300

[ ]  CG Envelope Point .... 88

[ ]  Weight Envelope Point .... 2450

[ ]  CG Envelope Point .... 90

[ ]  Weight Envelope Point .... 2600

[ ]  CG Envelope Point .... 92

[ ]  Weight Envelope Point .... 2600

[ ]  CG Envelope Point .... 94

[ ]  Weight Envelope Point .... 2600

[ ]  CG Envelope Point .... 95.999

[ ]  Weight Envelope Point .... 2600

[ ]  CG Envelope Point .... 96

[ ]  Weight Envelope Point .... 1400

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-200R Arrow/John_Peck_N98JG_Piper_PA-28R_W&B_Automatic__Calculations.txt

Rights: licensed

Checklist hash: 5279b101e3590d45108402aed250621bb64a4fb89fc2b7742cfe3f3734615942

Verify against the aircraft's own approved documentation before use.