Piper PA-28-201R Arrow III — pa28r-201_chklist_landrum

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Cruise" item count differs between passes (15 vs 3); verify nothing was dropped.

content_gap: Section "Pre Start" item count differs between passes (24 vs 25); verify nothing was dropped.

Pre Start

[ ]  Seats and Belts .... ADJUST

[ ]  Passengers .... INSTRUCT

[ ]  Flaps .... UP

[ ]  Circuit Breakers .... CHECK

[ ]  Alternate Air .... OFF

[ ]  Parking Brake .... SET

[ ]  Prop .... HIGH RPM

[ ]  Gear .... DOWN

[ ]  Avionics .... OFF

[ ]  Master .... ON

[ ]  Gear Lights .... THREE GREEN

[ ]  Fuel Selector .... DESIRED TANK

[ ]  DAY Anti-collision Lts .... ON

[ ]  NIGHT Nav Lts .... ON

[ ]  Throttle .... ½ in OPEN

[ ]  Fuel Pump .... ON

[ ]  Mixture .... RICH THEN ICO

[ ]  Prop .... “CLEAR”

[ ]  Starter .... ENGAGE

[ ]  Mixture .... RICH

[ ]  Warmup .... 800-1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Radios .... ON

[ ]  Transponder .... STANDBY

Taxi

[ ]  Parking Brake .... OFF

[ ]  Strobes .... OFF FOR TAXI

[ ]  Taxi .... CHECK BRAKES

[ ]  Fuel Pumps .... OFF

Before Takeoff

[ ]  Parking Brake .... SET

[ ]  Propeller .... HIGH RPM

[ ]  Mixture .... RICH

[ ]  Throttle .... 2000

[ ]  Mags .... CHK (175 MAX, 50 Diff)

[ ]  Vacuum .... 4.8 – 5.1

[ ]  Oil Temp / Press .... CHECK

[ ]  Ammeter .... CHECK

[ ]  Annunciator Panel .... TEST

[ ]  Alternate Air .... CHECK

[ ]  Propeller .... EXERCISE

[ ]  Fuel Pump .... ON

[ ]  Fuel Pressure .... CHECK

[ ]  Throttle .... 1000

[ ]  Trim .... NEUTRAL

Takeoff

[ ]  Fuel Selector .... SET

[ ]  Fuel Pump .... ON

[ ]  Engine Guages .... CHECK

[ ]  Alt Air .... CLOSED

[ ]  Seat Backs .... ERECT

[ ]  Seatbelts .... FASTENED

[ ]  Mixture .... SET

[ ]  Propeller .... HIGH RPM

[ ]  Flaps .... SET

[ ]  Trim .... SET

[ ]  Controls .... FREE

[ ]  Instruments .... CHECK/SET

[ ]  Door .... LATCHED

Normal

[ ]  Flaps .... SET

[ ]  Trim .... SET

[ ]  Rotate .... 65 - 75

[ ]  Gear .... UP

After Takeoff

[ ]  Gear .... CHECK UP

[ ]  Flaps .... UP

[ ]  Climb .... 90

[ ]  Man. Pressure .... 25in

[ ]  Props .... 2500

[ ]  Fuel Pump .... OFF

[ ]  Engine Temp. .... CHECK

Cruise

[ ]  Man Pressure .... 23in

[ ]  Props .... 2300

[ ]  Mixture .... LEAN (10 gph)

V-SPEEDS

NOTE: Vso 55 KIAS

NOTE: Vs1 60 KIAS

NOTE: Vx 78 KIAS

NOTE: Vldmax 79 KIAS

NOTE: Vy 90 KIAS

NOTE: Vfe 103 KIAS

NOTE: Vlo 107 KIAS

NOTE: Vle 129 KIAS

NOTE: Va 118-96 KIAS

NOTE: Vno 146 KIAS

NOTE: Vne 183 KIAS

Descent

[ ]  Fuel Selector .... SET

[ ]  Seats/Belts .... ADJUST

[ ]  Fuel Pumps .... ON

[ ]  Mixture .... RICH

[ ]  Prop .... HIGH RPM

[ ]  Gear .... DOWN – 3GR

[ ]  Flaps .... SET

[ ]  Man Press .... 17-18 in

[ ]  Landing Light .... ON

NOTE: BCGUMPS

After Landing

[ ]  Flaps .... UP

[ ]  Trim .... NEUTRAL

[ ]  Landing Light .... OFF

[ ]  Taxi Light .... AS REQ

[ ]  Transponder .... OFF

[ ]  Mixture .... LEAN for TAXI

[ ]  Fuel Pump .... OFF

Shutdown

[ ]  Electrical Equip .... OFF

[ ]  Avionics .... OFF

[ ]  Prop .... HIGH RPM

[ ]  Throttle .... 1000 RPM

[ ]  Fuel Pump .... OFF

[ ]  Mixture .... IDLE CUTOFF

[ ]  Magneto .... OFF

[ ]  Master .... OFF

[ ]  Controls .... LOCKED

Initial

[ ]  Control Lock .... REMOVE

[ ]  Gear .... DOWN

[ ]  Master .... ON

[ ]  Gear Ind Lights .... CHECK

[ ]  Fuel Gauges .... CHECK

[ ]  Nav Lights .... CHECK

[ ]  Pitot Heat .... CHECK

[ ]  ELT .... CHECK

[ ]  Alternator Lt .... CHECK

[ ]  Cabin/Inst Lts .... CHECK

[ ]  Master .... OFF

[ ]  Static Drain .... DRAINED

[ ]  Trim .... NEUTRAL

[ ]  Storm Window .... OPEN

[ ]  Flaps .... FULL

[ ]  Windows .... CLEAN

[ ]  Papers .... ONBOARD

[ ]  Preflight .... PERFORM

HOT Start  [ABNORMAL]

[ ]  Seats and Belts .... ADJUST

[ ]  Passengers .... INSTRUCT

[ ]  Flaps .... UP

[ ]  Circuit Breakers .... CHECK

[ ]  Alternate Air .... OFF

[ ]  Parking Brake .... SET

[ ]  Prop .... HIGH RPM

[ ]  Gear .... DOWN

[ ]  Avionics .... OFF

[ ]  Master .... ON

[ ]  Gear Lights .... THREE GREEN

[ ]  Fuel Selector .... DESIRED TANK

[ ]  DAY Anti-collision Lts .... ON

[ ]  NIGHT Nav Lts .... ON

[ ]  Throttle .... ½ in

[ ]  Fuel Pump .... ON

[ ]  Mixture .... ICO

[ ]  Prop .... “CLEAR”

[ ]  Starter .... ENGAGE

[ ]  Mixture .... RICH

[ ]  Warmup .... 800-1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Radios .... ON

[ ]  Transponder .... STANDBY

Flooded Start  [ABNORMAL]

[ ]  Seats and Belts .... ADJUST

[ ]  Passengers .... INSTRUCT

[ ]  Flaps .... UP

[ ]  Circuit Breakers .... CHECK

[ ]  Alternate Air .... OFF

[ ]  Parking Brake .... SET

[ ]  Prop .... HIGH RPM

[ ]  Gear .... DOWN

[ ]  Avionics .... OFF

[ ]  Master .... ON

[ ]  Gear Lights .... THREE GREEN

[ ]  Fuel Selector .... DESIRED TANK

[ ]  DAY Anti-collision Lts .... ON

[ ]  NIGHT Nav Lts .... ON

[ ]  Throttle .... FULL OPEN

[ ]  Fuel Pump .... OFF

[ ]  Mixture .... ICO

[ ]  Prop .... “CLEAR”

[ ]  Starter .... ENGAGE

[ ]  Mixture .... RICH

[ ]  Warmup .... 800-1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Radios .... ON

[ ]  Transponder .... STANDBY

Engine Out Procedure  [EMERGENCY]

[ ]  Trim [MEMORY] .... 79 KIAS

[ ]  Locate Field (size, shape, surface, slope, surroundings) .... CHECK

[ ]  Turn .... TO FIELD

[ ]  Radio call .... MAKE

[ ]  Execute Power Off Landing .... EXECUTE

Engine Troubleshoot  [EMERGENCY]

[ ]  Fuel Flow .... CHECK

[ ]  Fuel Pump .... ON

[ ]  Fuel Selector .... SET

[ ]  Alternate Air .... ON

[ ]  Engine Gauges .... CHECK

NOTE: If Engine Still Dead Continue

Securing Engine  [EMERGENCY]

[ ]  Fuel Pump .... OFF

[ ]  Selector .... OFF

[ ]  Mixture .... ICO

[ ]  Magnetos .... OFF

[ ]  Master .... OFF

[ ]  Door .... CRACK OPEN

[ ]  Belts .... TIGHT

[ ]  Min possible airspeed .... MAINTAIN

Landing Gear Emergency Extension  [EMERGENCY]

[ ]  Gear Selector .... DOWN

[ ]  Master .... ON

[ ]  Circuit Breaker .... CHECK IN

[ ]  Nav Lights .... CHECK OFF

[ ]  Gear Bulbs .... CHECK

[ ]  Airspeed .... < 87 KIAS

[ ]  Manual Extn .... ARM & PULL

[ ]  Yaw Aircraft to Drop Gear .... YAW

[ ]  Gear Indicators .... 3 GREEN

Gear Up Landing  [EMERGENCY]

[ ]  Flaps .... AS DESIRED

[ ]  Proceed with Securing Engine .... PROCEED

Electrical Fire  [EMERGENCY]

[ ]  Master Switch [MEMORY] .... OFF

[ ]  Air Vents .... OPEN

[ ]  Cabin Heat .... OFF

NOTE: Land As Soon As Practical

Loss of Oil Pressure  [EMERGENCY]

[ ]  Prepare for Engine Out Procedure .... PREPARE

Engine Fire (Ground)  [EMERGENCY]

[ ]  Starter .... CRANK

[ ]  Mixtures .... ICO

[ ]  Throttles .... FULL OPEN

[ ]  Fuel Selectors .... OFF

[ ]  Fuel Pumps .... OFF

Engine Fire (Flight)  [EMERGENCY]

[ ]  Fuel Selector [MEMORY] .... OFF

[ ]  Mixture [MEMORY] .... ICO

[ ]  Prop .... FULL FWD

[ ]  Fuel Pump .... OFF

[ ]  Magnetos .... OFF

[ ]  Alternator .... OFF

[ ]  Cabin Heat .... OFF

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-201R Arrow III/pa28r-201_chklist_landrum.txt

Rights: licensed

Checklist hash: a6ba3b26bfefb68e305bbb13915a511ef482b00f9e2b2ff017f2c0626ad4bb97

Verify against the aircraft's own approved documentation before use.