Piper PA-28-235 Cherokee — Eric_Duensing_pa28_235_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE STARTING ENGINE

[ ]  Preflight inspection .... COMPLETE

[ ]  Seats, Seat Belts .... ADJUST AND LOCK

[ ]  Avionics Master .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Autopilot .... OFF

[ ]  Circuit Breakers .... CHECK IN

[ ]  Brakes .... TEST

[ ]  Fuel Selector Valve .... FULLEST TANK

STARTING ENGINE

[ ]  Carburetor heat .... OFF

[ ]  Throttle .... OPEN 1/4"

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Primer .... AS REQUIRED

[ ]  Primer .... LOCKED

[ ]  Mixture .... FULL RICH

[ ]  Propeller Area .... CLEAR

[ ]  Depress Starter (release when engine starts)

[ ]  Throttle .... 800 RPM

[ ]  Oil pressure .... CHECK

[ ]  Electric Fuel Pump .... OFF

[ ]  Mixture .... LEAN FOR TAXI

[ ]  Avionics Master .... ON

[ ]  Fan, heater and defroster .... ON as required

[ ]  Navigation, Cockpit, and Taxi Lights .... ON as required

[ ]  Anti-collision lights .... ON

[ ]  Transponder .... TEST, then SDBY

[ ]  Altimeter .... SET to setting given by ATC

[ ]  Heading Indicator .... Set with magnetic compass

[ ]  Taxiing .... CHECK brakes immediately after airplane begins moving

DURING TURNS WHILE TAXIING

[ ]  Turn Coordinator .... Indicates skidding turn in proper direction

[ ]  Heading Indicator .... Turns in proper direction

BEFORE TAKEOFF

[ ]  Cabin Door .... CLOSED AND LATCHED

[ ]  Flight controls .... FREE AND CORRECT

[ ]  Fuel Selector Valve .... FULLEST TANK

[ ]  Flaps .... CHECK

[ ]  Flaps .... UP

[ ]  Elevator and Rudder Trim .... SET FOR TAKEOFF

[ ]  Mixture .... FULL RICH

[ ]  Parking Brake .... SET

[ ]  Throttle .... 1800 RPM

[ ]  Magnetos .... Check (max drop 175 RPM, max diff. 50 RPM)

[ ]  Carburetor Heat .... ON (check for RPM to drop about 100 RPM and stay down)

[ ]  Carburetor Heat .... OFF (check for RPM to return to 1700)

[ ]  Electric Fuel Pump .... ON (check fuel pressure)

[ ]  Ammeter .... CHECK

[ ]  Oil Pressure and Temperature .... Normal operating

[ ]  Suction Gauge .... 4.5" Hg

[ ]  Throttle .... 800 RPM

[ ]  Flight instruments .... SET

[ ]  Attitude Indicator .... Check, Set for level flight

[ ]  Heading Indicator .... Recheck heading

[ ]  Vertical Speed Indicator .... Note indication of 0 fpm climb

[ ]  Airspeed Indicator .... indicates 0

[ ]  Clock .... CHECK and SET

[ ]  Pitot Heat .... CHECK

[ ]  Communication Radio .... Tower or CTAF

[ ]  Navigation Radio .... SET if flying cross-country

[ ]  Autopilot .... OFF

[ ]  Transponder .... ALT

[ ]  Window .... CLOSED

[ ]  Parking Brake .... OFF

NORMAL TAKEOFF

[ ]  Flaps .... 0°

[ ]  Throttle .... FULL OPEN

[ ]  Airspeed Indicator .... Indicates increasing speed

[ ]  Elevator Control .... Rotate at 65 MPH

CLIMB SPEEDS

[ ]  Best angle (Vx) .... 90 MPH

[ ]  Best rate (Vy) .... 100 MPH

[ ]  Normal .... 115 MPH

SHORT FIELD TAKEOFF

[ ]  Flaps .... 25° (2nd notch)

[ ]  Brakes .... HOLD

[ ]  Throttle .... FULL OPEN

[ ]  Brakes .... RELEASE

[ ]  Elevator Control .... Rotate at 60 MPH

[ ]  Climb Speed .... 90 MPH until obstacles cleared, then as appropriate

[ ]  Flaps .... RETRACT after obstacles cleared

CRUISE

[ ]  Electric Fuel Pump .... OFF

[ ]  Elevator and Rudder Trim .... ADJUST

[ ]  Power .... Adjust to 75% per Power Setting Table

[ ]  Mixture .... lean to peak EGT minus 125 degrees

BEFORE LANDING

[ ]  Seats, Seat Belts, .... ADJUST AND LOCK

[ ]  Electric Fuel Pump .... ON

[ ]  Fuel Selector Valve .... FULLEST TANK

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... ON (IF ICING IS INDICATED)

NORMAL LANDING

[ ]  Wing Flaps .... BELOW 115 MPH

[ ]  Approach Speed .... 90 MPH

BALKED LANDING

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... OFF

[ ]  Flaps .... RETRACT TO 25° IMMEDIATELY

[ ]  Airspeed .... 90 MPH

[ ]  Flaps .... RETRACT (after establishing a climb)

AFTER LEAVING RUNWAY

[ ]  Flaps .... RETRACT

[ ]  Carburetor Heat .... OFF

[ ]  Transponder .... SDBY

[ ]  Electric Fuel Pump .... OFF

SHUTDOWN

[ ]  Avionics Master .... OFF

[ ]  Electrical Equipment .... OFF (except anti-collision light)

[ ]  Throttle .... IDLE

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magneto Switch .... OFF (after propeller stops turning)

[ ]  Anti-collision lights .... OFF

[ ]  Master Switch .... OFF

[ ]  Vents .... CLOSED

[ ]  Control Wheel .... SECURED WITH SEAT BELT

[ ]  Tiedowns .... SECURED

[ ]  Flight Plan .... CLOSED

VUJ

[ ]  AWOS .... 128.175

[ ]  GROUND .... 121.75

[ ]  TOWER .... 126.275

FLAPS

[ ]  1ST .... 10 DEGREES

[ ]  2ND .... 25 DEGREES

[ ]  3RD .... 40 DEGREES

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-235 Cherokee/Eric_Duensing_pa28_235_Checklist.txt

Rights: licensed

Checklist hash: 38ce98a0328525f2fc3eec620c5f47a0a06628529688737538d95097a26be1d9

Verify against the aircraft's own approved documentation before use.