Piper PA-28-235 Cherokee — Eric_Duensing_pa28_235_wb

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Weight & Balance

NOTE: INSTRUCTIONS: FILL OUT ALL NUMBERS IN BLUE. BLACK NUMBERS GIVE YOU THE ANSWER.

NOTE: WEIGHT AND BALANCE — DO NOT DELETE THE DATA IN RED

ITEM / WEIGHT / ARM / MOMENT

[ ]  Basic Empty Weight .... 1530.76 lb, Arm 83.11, Moment 127220

[ ]  Pilot .... 200 lb, Arm 88.9, Moment 17780

[ ]  Co-Pilot .... 0 lb, Arm 88.9, Moment 0

[ ]  Rear Seats .... 0 lb, Arm 118.5, Moment 0

[ ]  Baggage (200 lb max) .... 0 lb, Arm 142, Moment 0

[ ]  Fuel (in gallons) .... 84 gal, 504 lb, Arm 95, Moment 47880

[ ]  TAKEOFF .... Weight 2234.76 lb, C.G. 86.30904437165512 in, Moment 192880

[ ]  LANDING .... Weight 1808.7600000000002 lb, C.G. 84.26214644286692 in, Moment 152410

[ ]  MAX GROSS WEIGHT .... 2900 lb

C.G. VS WEIGHT LIMITS

[ ]  Takeoff C.G.: 86.30904437165512 in at 2234.76 lb

[ ]  Landing C.G.: 84.26214644286692 in at 1808.7600000000002 lb

[ ]  Normal: 83 in / 1400 lb, 83 in / 2100 lb, 91.55 in / 2900 lb, 93 in / 2900 lb

[ ]  Utility: 83 in / 1400 lb, 83 in / 1950 lb, 83.7 in / 2020 lb, 93 in / 2020 lb, 93 in / 1400 lb

[ ]  Fuel (Taxi, Climb) .... 1 gal

[ ]  Fuel Burn .... 14 gal/hr

[ ]  Planned Flight Time .... 5 hrs

[ ]  Estimated Fuel Used .... 71 gal

[ ]  Estimated Fuel Remaining .... 13 gal

[ ]  Estimated Time Remaining .... 0.9285714285714286 hrs

Computations

TEMPERATURE

[ ]  Enter oC: .... 29

[ ]  Get oF: .... 84.2

DENSITY ALTITUDE

[ ]  Field Elevation — Departure .... 295

[ ]  Field Elevation — Arrival .... 295

[ ]  Altimeter Setting — Departure .... 29.95

[ ]  Altimeter Setting — Arrival .... 30.21

[ ]  Temp, deg F — Departure .... 86

[ ]  Temp, deg F — Arrival .... 86

[ ]  Std Temp at Field — Departure .... 56

[ ]  Std Temp at Field — Arrival .... 40.268

[ ]  Pressure Altitude — Departure .... 324.9999999999976

[ ]  Pressure Altitude — Arrival .... 584.9999999999991

[ ]  Density Altitude — Departure .... 2324.9999999999977

[ ]  Density Altitude — Arrival .... 3633.7999999999993

WEIGHT-ADJUSTED V-SPEEDS (KIAS)

[ ]  Vso — MGW .... 60

[ ]  Vso — Your weight .... 52.6705309218182

[ ]  1.2 x Vso — MGW .... 72

[ ]  1.2 x Vso — Your weight .... 63.204637106181835

[ ]  1.3 x Vso — MGW .... 78

[ ]  1.3 x Vso — Your weight .... 68.47169019836366

[ ]  Vx — MGW .... 90

[ ]  Vx — Your weight .... 79.0057963827273

[ ]  Best glide — MGW .... 80

[ ]  Best glide — Your weight .... 70.22737456242426

[ ]  Vy — MGW .... 100

[ ]  Vy — Your weight .... 87.78421820303033

[ ]  Vfe — MGW .... 115

[ ]  Vfe — Your weight .... 100.95185093348488

[ ]  Va — MGW .... 111

[ ]  Va — Your weight .... 97.44048220536367

[ ]  Vno — MGW .... 126

[ ]  Vno — Your weight .... 110.60811493581821

[ ]  Vne — MGW .... 166

[ ]  Vne — Your weight .... 145.72180221703036

Old W&B

[ ]  Basic Empty Weight .... Weight 1530.76, Arm 83.11, Moment 127221.4636

[ ]  Pilot .... 0

[ ]  Front Seat Passenger .... 0

[ ]  Rear Seat Passenger(s) .... 0

[ ]  Baggage (200lb Max) .... 0

[ ]  Fuel (gals x 6lbs.) .... 84

[ ]  Total .... 1614.76

[ ]  Total Moment / Total Weight = Center of Gravity in Inches

[ ]  Result .... 135350.1436 / 1614.76 = 83.82059476330849 (83-93)

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-235 Cherokee/Eric_Duensing_pa28_235_wb.txt

Rights: licensed

Checklist hash: cbd96a36ddb4178466ecd707449bcae23d809111c7119fb137a0e54c8e14c47c

Verify against the aircraft's own approved documentation before use.