Piper PA-32RT-300T Lance — Jason_Helmka_PA32_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Preflight Inspection

CABIN

[ ]  Hobbs/Tach .... Check

[ ]  AROW .... Check

[ ]  Master Switch .... On

[ ]  Fuel .... Check Quantity

[ ]  Lights .... Check

[ ]  Flaps .... 10°

[ ]  Pitot Heat .... Test if IFR

[ ]  Master Switch .... Off

[ ]  Fuel Sump Lever .... Short push

RIGHT WING

[ ]  Flap, Aileron, Wing Tip .... Check

[ ]  Leading Edge .... Check

[ ]  Fuel Sump .... 1 Sample

[ ]  Fuel Level .... Check

[ ]  Tie Down .... Remove

[ ]  Landing Gear .... Check

[ ]  Fuel Sump .... 2 Sample

NOSE

[ ]  Windshield .... Clean

[ ]  Alternator Belt .... Check

[ ]  Prop & Spinner .... Check

[ ]  Nose Wheel & Gear .... Check

[ ]  Oil .... Check

[ ]  Air intake scoop .... Check

LEFT WING

[ ]  Leading Edge .... Check

[ ]  Landing Gear .... Check

[ ]  Fuel Sump .... 3 Sample

[ ]  Tie Down .... Remove

[ ]  Fuel Level .... Check

[ ]  Fuel Sump .... 4 Sample

[ ]  Pitot Tube .... Check

[ ]  Flap, Aileron, Wing Tip .... Check

EMPENNAGE

[ ]  Rear Door .... Secure

[ ]  Tie Down .... Remove

[ ]  Surfaces .... Check

[ ]  Controls .... Check

[ ]  Antennas .... Checks

[ ]  Lights .... Check

Starting Engine

[ ]  Seat Track/Back .... Lock

[ ]  Avionics .... Off

[ ]  Beacon .... On

[ ]  Brakes .... Set

[ ]  Throttle .... Crack open

[ ]  Prop .... Foward

[ ]  Master/ALT .... On

[ ]  Fuel Pump .... On

[ ]  Mixture .... Rich until 10 GPH flow then close

[ ]  Starter .... Engage

[ ]  Mixture .... Full rich when engine starts

[ ]  Throttle .... 700 RPM

[ ]  Oil Pressure .... Check

[ ]  Fuel Pump .... Off

[ ]  Fuel Pressure .... Check

Taxi

[ ]  Seat Belts/Harness .... On

[ ]  Flaps .... Up

[ ]  Avionics .... On

[ ]  Transponder .... 1200 or assigned

[ ]  ATIS .... Receive

[ ]  Altimeter .... Set

Engine Run-Up

[ ]  Brakes .... Set

[ ]  Fuel .... Fullest Tank

[ ]  Trim .... Neutral

[ ]  Flight Controls .... Free & Correct

[ ]  Annunciator Lights .... Check

[ ]  Instruments .... Set

[ ]  Mixture .... Full Rich

[ ]  Throttle .... 2000 RPM

[ ]  Mags .... Check L & R

[ ]  Prop .... Cycle

[ ]  Alternate air .... Check

[ ]  Vacuum .... Check

[ ]  Fuel Pressure .... Check

[ ]  Amps .... Check

[ ]  Oil Pressure .... Check

[ ]  Oil Temperature .... Check

[ ]  Throttle .... Idle

Pre-Takeoff

[ ]  Flaps .... 25°

[ ]  Mixture .... Best Power

[ ]  Fuel Pump .... On

[ ]  Pitot Heat .... As Req

[ ]  Doors/Windows .... Closed

[ ]  Door Seals .... Presurise

[ ]  Airconditioner .... Off

[ ]  Landing Light .... On

[ ]  Brakes .... Release

Takeoff

[ ]  Throttle .... Full

[ ]  Oil Pressure .... Check

[ ]  Rotate .... 70 KIAS

[ ]  Vy .... 87 KIAS

[ ]  Gear .... UP befor 106

Climb

[ ]  Airspeed .... 87 KIAS

[ ]  Flaps Up .... Reduce to 25

[ ]  Mixture .... 75% power

[ ]  Throttle .... 25"

[ ]  Prop .... 2500 RPM

[ ]  Fuel Pump .... Off

[ ]  Landing Lights .... Off

[ ]  CHT .... Below 350

Cruise

[ ]  Power .... 22" to 25"

[ ]  Prop .... 2400 to 2500 RPM

[ ]  Mixture .... 65% power

[ ]  Instruments .... Check

[ ]  Fuel .... Switch 30 mins

Descent

[ ]  Mixture .... Increase

[ ]  Fuel .... Fullest Tank

[ ]  ATIS .... Receive

[ ]  Instruments .... Set

Before Landing

[ ]  Landing Light .... On

[ ]  Seat Belts/Harness .... On

[ ]  Mixture .... Rich

[ ]  Prop .... Foward

[ ]  Fuel Pump .... On

[ ]  Fuel .... Fullest Tank

[ ]  Landing Gear .... Down

Normal Landing

[ ]  Flaps .... 25 to 40

[ ]  Airspeed .... 80 KIAS

[ ]  GUMP check .... On final

[ ]  Touch down .... 70 KIAS

Go Around

[ ]  Power .... Full

[ ]  Landing gear .... Positive rate UP

[ ]  Flaps .... 10 then positive rate UP

After Landing

[ ]  Flaps .... Up

[ ]  Fuel Pump .... Off

[ ]  Landing Light .... Off

[ ]  Pitot Heat .... Off

[ ]  Trim .... Neutral

[ ]  Door Seals .... Release

Shutdown

[ ]  Avionics .... Off

[ ]  Mixture .... Idle Cut-Off

[ ]  Mags .... Off

[ ]  Master/ALT .... Off

[ ]  Lights .... Off

[ ]  Hobbs/Tach .... Record

[ ]  Yoke .... Secure

[ ]  Tie Downs .... Secure

Emergency Procedures  [EMERGENCY]

ENGINE FAILURE DURING TAKEOFF

[ ]  Aircraft Control .... Maintain

[ ]  Airspeed .... 92 KIAS

[ ]  Fuel Selector .... Off

[ ]  Mixture .... Idle Cut-Off

[ ]  Fuel Pump .... Off

[ ]  Flaps .... Down

[ ]  Landing Gear .... Down

[ ]  Master .... Off

[ ]  Mags .... Off

[ ]  Door .... Unlatch

ENGINE FAILURE IN FLIGHT

[ ]  Airspeed .... 92 KIAS

[ ]  Landing Site .... Pick

[ ]  Fuel Selector .... Switch Tanks

[ ]  Fuel Pump .... On

[ ]  Mixture .... Rich

[ ]  Alternate Air .... Open

[ ]  Mags .... Check All

[ ]  Master .... On

NO TIME FOR RESTART

[ ]  Airspeed .... Maintain 92 KIAS

[ ]  Landing Site .... Pick

[ ]  Transponder .... 7700

[ ]  Radio .... 121.50 Declare Emergency

[ ]  ELT .... On

[ ]  Seatbelts/Harness .... On

[ ]  Flaps .... As Req

[ ]  Landing gear .... If terrain allows

[ ]  Mixture .... Idle Cut-Off

[ ]  Master .... Off

[ ]  Mags .... Off

[ ]  Door .... Unlatch

[ ]  Short Final .... 75 KIAS

[ ]  Body .... Protect

Electrical Fire In Flight  [EMERGENCY]

[ ]  All Elec Devices .... Off

[ ]  Master .... Off

[ ]  Vents, Cabin Heat & Air .... Off

[ ]  If Fire Out Master .... On Only If Critical

[ ]  Turn On One Ess Elec Device at a Time .... Check

[ ]  Reset Circuit Breakers .... Only If Critical

Landing Gear Failure  [EMERGENCY]

[ ]  Gear Selector .... Down

[ ]  Master Switch .... On

[ ]  Circuit Breakers .... Check

[ ]  Radio Lights .... Off

[ ]  Gear Indicator Bulbs .... Check

IF NOT DOWN

[ ]  Airspeed .... Below 87 KIAS

[ ]  Gear Selector .... Down

[ ]  Override Lever .... Engaged (While Fish tailing)

[ ]  Emergency Gear Lever .... Hold Down

NOTE: Hold lever down for at least 10 sec.

All Other Emergencies  [EMERGENCY]

NOTE: ALL OTHER EMERGENCIES REFER TO POH

V-Speeds

[ ]  Vr (Rotation Speed) .... 65 KIAS

[ ]  Vx (Best Angle) .... 87 KIAS

[ ]  Vy (Best Rate) .... 92 KIAS

[ ]  Va (Maneuvering Speed) .... 106-132 KIAS

[ ]  Vs (Stall Speed) .... 66 KIAS

[ ]  Vso (Stall- Full Flaps) .... 61 KIAS

[ ]  Vfe (Max Flaps Extended) .... 106 KIAS

[ ]  Vle (Max landing gear ext) .... 129 KIAS

[ ]  Vno (Max Normal Operating) .... 148 KIAS

[ ]  Vne (Never Exceed) .... 191 KIAS

[ ]  Vglide (Best Glide) .... 92 KIAS

[ ]  Max Crosswind .... 17 KIAS

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-32RT-300T Lance/Jason_Helmka_PA32_checklist.txt

Rights: licensed

Checklist hash: da333c606ec125f41855089221f9ac57724399a9a331d814808f766f3a3d0e2f

Verify against the aircraft's own approved documentation before use.