Piper PA-32RT-300T Lance — zoan_harclerode_PA32RT-300T-Lance_Checklist_Normal

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "COLD WEATHER STARTING (BELOW 20°)" item count differs between passes (17 vs 16); verify nothing was dropped.

content_gap: Section "CRITICAL NUMBERS" item count differs between passes (3 vs 11); verify nothing was dropped.

content_gap: Section "STARTING WITH EXTERNAL POWER SOURCE" item count differs between passes (9 vs 8); verify nothing was dropped.

content_gap: Section "TAKEOFF" item count differs between passes (14 vs 13); verify nothing was dropped.

BEFORE STARTING ENGINE

[ ]  Parking brakes .... SET

[ ]  Propeller .... FULL INCREASE RPM

[ ]  Fuel selector .... DESIRED TANK

[ ]  Cowl flaps .... OPEN

NOTE: Time & tank

STARTING ENGINE WHEN COLD

[ ]  Throttle .... 1/2" OPEN

[ ]  Master switch .... ON

[ ]  Electric fuel pump .... ON

[ ]  Mixture .... PRIME – THEN IDLE CUT-OFF

[ ]  Starter .... ENGAGE

[ ]  Mixture .... FULL RICH

[ ]  Throttle .... ADJUST

[ ]  Oil pressure .... CHECK

COLD WEATHER STARTING (BELOW 20°)

NOTE: Use of preheat should be considered if available. Rotate engine through 10 blades during preflight inspection.

CAUTION: Insure magneto & master switches are "OFF" before rotating engine manually.

[ ]  Master switch .... OFF

[ ]  External power .... CONNECTED

[ ]  Mixture .... RICH

[ ]  Propeller .... FORWARD

[ ]  Throttle .... OPEN 1/2"

[ ]  Fuel boost pump .... ON FOR 5-6 SEC.

[ ]  Fuel boost pump .... OFF

[ ]  Engage starter .... ROTATE ENGINE THROUGH 10 BLADES

[ ]  Fuel boost pump .... ON FOR 5-6 SEC.

[ ]  Fuel boost pump .... OFF

[ ]  Starter .... ENGAGE

WHEN ENGINE FIRES,

[ ]  Fuel boost pump .... ON

[ ]  External power .... DISCONNECT

[ ]  Master switch .... ON

STARTING ENGINE WHEN HOT

[ ]  Throttle .... 1/2" OPEN

[ ]  Master switch .... ON

[ ]  Electric fuel pump .... ON

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Starter .... ENGAGE

[ ]  Mixture .... ADVANCE

[ ]  Throttle .... ADJUST

[ ]  Oil pressure .... CHECK

STARTING ENGINE WHEN FLOODED

[ ]  Throttle .... OPEN FULL

[ ]  Master switch .... ON

[ ]  Electric fuel pump .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Starter .... ENGAGE

[ ]  Mixture .... ADVANCE

[ ]  Throttle .... RETARD

[ ]  Oil pressure .... CHECK

STARTING WITH EXTERNAL POWER SOURCE

[ ]  Master switch .... OFF

[ ]  All electrical equipment .... OFF

[ ]  Terminals .... CONNECT

[ ]  External power plug .... INSERT IN FUSELAGE

PROCEED WITH NORMAL START

[ ]  Throttle .... LOWEST POSSIBLE RPM

[ ]  External power plug .... DISCONNECT FROM FUSELAGE

[ ]  Master switch .... ON – CHECK AMMETER

[ ]  Oil pressure .... CHECK

WARM-UP

[ ]  Throttle .... 1000 TO 1200 RPM

TAXIING

[ ]  Parking brakes .... RELEASE

[ ]  Chocks .... REMOVED

[ ]  Taxi area .... CLEAR

[ ]  Throttle .... APPLY SLOWLY

[ ]  Prop .... HIGH RPM

[ ]  Brakes .... CHECK

[ ]  Steering .... CHECK

GROUND CHECK

[ ]  Parking brakes .... SET

[ ]  Propeller .... FULL INCREASE

[ ]  Throttle .... 2000 RPM

[ ]  Magnetos .... MAX DROP 175 RPM – MAX DIFF. 50 RPM

[ ]  Vacuum .... 5.0" HG. ± .1

[ ]  Oil temp .... CHECK

[ ]  Oil pressure .... CHECK

[ ]  Air conditioner .... CHECK

[ ]  Annunciator panel .... PRESS-TO-TEST

[ ]  Propeller .... EXERCISE – THEN FULL INCREASE

[ ]  Alternate air .... CHECK

NOTE: Engine is warm for takeoff when throttle can be opened without engine faltering.

[ ]  Electric fuel pump .... OFF

[ ]  Fuel flow .... CHECK

[ ]  Throttle .... RETARD

[ ]  Manifold pressure line .... DRAIN

BEFORE TAKEOFF

[ ]  Master switch .... ON

[ ]  Flight instruments .... CHECK

[ ]  Fuel selector .... PROPER TANK

[ ]  Engine gauges .... CHECK

[ ]  Alternate air .... CLOSED

[ ]  Seat backs .... ERECT

[ ]  Cowl flaps .... OPEN

[ ]  Mixture .... SET

[ ]  Prop .... SET

[ ]  Belts / harness .... FASTENED

[ ]  Empty seats .... SEAT BELTS SNUGLY FASTENED

[ ]  Flaps .... SET

[ ]  Trim tabs – rudder & elevator .... SET

[ ]  Controls .... FREE & CORRECT

[ ]  Doors .... LATCHED

[ ]  Air conditioner .... OFF

[ ]  Parking Brake .... RELEASE

TAKING THE RUNWAY

[ ]  Electric fuel pump .... ON

[ ]  Lights .... ON AS DESIRED

[ ]  Approach & Runway .... HAVE CLEARANCE & CLEAR VISUAL PATH

[ ]  Time & Tank .... RECORDED

[ ]  Transponder .... ON ALT

[ ]  Cowl flaps .... OPEN

[ ]  Flaps .... SET

[ ]  Trim .... SLIGHTLY AFT OF NEUTRAL

TAKEOFF

NORMAL

[ ]  Power .... MAX 36"/2700 RPM – 5 MINUTE LIMIT

[ ]  Accelerate to Vr .... 75 TO 85 KIAS

[ ]  Control wheel back pressure .... ROTATE TO CLIMB ATTITUDE

SHORT OR SOFT FIELD, OBSTACLE CLEARANCE

[ ]  Flaps .... SECOND NOTCH

[ ]  Trim .... SLIGHT AFT OF NEUTRAL

[ ]  Throttle .... FULL POWER PRIOR TO BRAKE RELEASE

[ ]  Accelerate to .... 65 TO 69 KIAS DEPENDING ON AIRCRAFT WEIGHT

[ ]  Control wheel back pressure .... ROTATE TO CLIMB ATTITUDE

[ ]  After breaking ground, accelerate to .... 65 TO 70 KIAS DEPENDING ON A/C WEIGHT

[ ]  Gear (Override Engaged) .... UP

[ ]  Accelerate to climb speed

[ ]  Flaps .... RETRACT SLOWLY

CLIMB

[ ]  Vy (3600 lb) (gear down) (F0) (T/O pwr) .... 89 KIAS

[ ]  Vy (3600 lb) (gear up) (F0) (T/O pwr) .... 93 KIAS

[ ]  Vy (3600 lb) (gear up) (F0) (Max. cont. pwr) .... 95 KIAS

[ ]  Vx (3600 lb) (gear down) (F0) (T/O pwr) .... 77 KIAS

[ ]  Vx (3600 lb) (gear up) (F0) (T/O pwr) .... 80 KIAS

[ ]  Vx (3600 lb) (gear up) (F0) (Max. cont. pwr) .... 82 KIAS

[ ]  En Route .... 105 KIAS

[ ]  Mixture .... RICH

[ ]  Electric Fuel Pump .... OFF @ SAFE ALTITUDE (ON ABOVE 14,000 FEET)

[ ]  Transition Altitude abv FL180 .... SET ALTIMETER 29.92

CRUISING

[ ]  Normal max power .... 81%

[ ]  Power .... SET PER POWER TABLE

[ ]  Mixture .... ADJUST

[ ]  Cowl flaps .... SET

CRITICAL NUMBERS

[ ]  CHT .... 150°F TO 435°F

NOTE: Oil Temps: Above 60°F OAT 180° - 245°; 30° - 90° 180° - 245°; 0° - 70° 170° - 225°; Below 10°F 160° - 210°

NOTE: Maneuvering Speeds (Va): 3600# max gross 132; 3242.5 125.75; 2885 119.5; 2527.5 113.25; 2170 107

DESCENT

[ ]  Mixture .... ABOVE 1350°F EGT

[ ]  Throttle .... ABOVE 15" MP

[ ]  Propeller .... CRUISE SETTING

[ ]  Altimeter .... SET TO STATION PRESSURE

[ ]  Oxygen .... OFF BELOW 10,000 FT

APPROACH & LANDING

[ ]  Fuel selector .... PROPER TANK

[ ]  Seat backs .... ERECT

[ ]  Belts / harness .... FASTEN

[ ]  Electric fuel pump .... ON

[ ]  Mixture .... SET

[ ]  Propeller .... SET FOR GO AROUND

[ ]  Cowl flaps .... OPEN

[ ]  Gear .... DOWN – 129 KIAS MAX.

[ ]  Flaps .... SET – WHITE ARC

[ ]  Air conditioner .... OFF

NORMAL TECHNIQUE

[ ]  Flaps .... AS REQUIRED

[ ]  Trim .... 95 KIAS

[ ]  Throttle .... AS REQUIRED

SHORT FIELD TECHNIQUE

[ ]  Flaps .... 40 – FULL FLAPS – 3RD NOTCH

[ ]  Trim .... 76 KIAS*

[ ]  Throttle .... AS REQUIRED

CAUTION: When C.G. is forward of 82 inches, use 90 KIAS approach speed.

GO-AROUND

[ ]  Propeller .... FULL INCREASE

[ ]  Throttle .... AS REQUIRED (NOT TO EXCEED 36" MP)

[ ]  Control wheel .... BACK PRESSURE TO ROTATE TO CLIMB ATTITUDE

[ ]  Gear .... UP

[ ]  Flaps .... RETRACT SLOWLY

[ ]  Trim .... AS REQUIRED

STOPPING ENGINE

[ ]  Flaps .... RETRACT

[ ]  Electric fuel pump .... OFF

[ ]  Air conditioner .... OFF

[ ]  Radios .... OFF

[ ]  Lights .... OFF AS REQUIRED

[ ]  Propeller .... FULL INCREASE

[ ]  Throttle .... FULL AFT

[ ]  Magnetos .... CHECK GROUND

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Master switch .... OFF

PARKING

[ ]  Parking brake .... SET AS REQUIRED

[ ]  Control wheel .... SECURE WITH SEAT BELT

[ ]  Flaps .... FULL UP

[ ]  Wheel Chocks & lock .... IN PLACE

[ ]  Tie downs .... SECURE

SPEED SHEET KIAS

NOTE: Take-off: normal F0 – F10 80 – 85

NOTE: Take-off: soft & short fld F25 65/69 – 80 – 93

NOTE: Vx: 77 (G↓ F↑ t/o pwr); 80 (G↑F↑t/o pwr); 82 (G↑F↑max cont pwr)

NOTE: Vy: 89 (G↓ F↑ t/o pwr); 93 (G↑F↑t/o pwr); 95 (G↑F↑max cont pwr)

NOTE: Close door 87 (vents - closed, storm window – open)

NOTE: Enroute Climb 105

NOTE: Max Gear Retract 106

NOTE: Max Flap Ext 109

NOTE: Max Gear Ext'g 129

NOTE: Vno 150

NOTE: Vne 191

NOTE: Va 132 – 107

NOTE: E descent: 10"mp – prop full fwd – cowl flaps closed - 109G↓F↓ - 1350° EGT

NOTE: BEST GLIDE 92

NOTE: E-gear down 87

NOTE: Downwind G↓ F10 110

NOTE: Base G↓ F25 105

NOTE: Apch – normal G↓ F40 95 (fwd c.g. 90)

NOTE: Over-the-fence **GEAR DOWN** 85

NOTE: Touch down 65

NOTE: Short Field power off 76

NOTE: Vso 52

NOTE: Vs 57

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-32RT-300T Lance/zoan_harclerode_PA32RT-300T-Lance_Checklist_Normal.txt

Rights: licensed

Checklist hash: 861014deee4a277e9ecdd82cf072ff60f6ecf7eb6fbdb44c7229c8d4764d2835

Verify against the aircraft's own approved documentation before use.