Piper PA-34-200 Seneca — jochman-clseneca

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Engine Failure  [EMERGENCY]

[ ]  Minimum Control Speed .... 80 MPH

[ ]  Blue Line Speed .... 105 MPH

[ ]  Mixture, Prop, Throttle .... FORWARD

[ ]  Flaps & Gear .... RETRACT

[ ]  Electric Fuel Pumps .... ON

[ ]  Identify .... DEAD FOOT = DEAD ENGINE

[ ]  Verify .... RETARD THROTTLE

[ ]  Propeller of Dead Engine .... FEATHER

[ ]  Mixture of Dead Engine .... IDLE CUT OFF

[ ]  Trim .... AS REQUIRED

[ ]  Bank .... 5º TO GOOD ENGINE

[ ]  Fuel Pump of Dead Engine .... OFF

[ ]  Magnetos of Dead Engine .... OFF

[ ]  Cowl Flaps of Dead Engine .... CLOSE

[ ]  Cowl Flaps of Good Engine .... AS NEEDED

[ ]  Alternator of Dead Engine .... OFF

[ ]  Electrical Load .... REDUCE

[ ]  Fuel to Dead Engine .... OFF

[ ]  Fuel to Good Engine .... ON or CROSSFEED

[ ]  Fuel Pump of Good Engine .... OFF

Restart Engine  [EMERGENCY]

[ ]  Mixture .... AS REQUIRED

[ ]  Fuel Boost Pump .... ON

[ ]  Fuel Selector .... CROSSFEED

[ ]  Magnetos .... SELECT L or R ONLY

[ ]  Alternate Air .... ON

Unfeathering Procedure  [EMERGENCY]

[ ]  Fuel Selector Dead Engine .... ON

[ ]  Fuel Pump on Dead Engine .... OFF

[ ]  Throttle .... OPEN ¼ INCH

[ ]  Prop .... FORWARD

[ ]  Mixture .... RICH

[ ]  Magneto Switches .... ON

[ ]  Starter .... ENGAGE, PROP WINDMILLS

[ ]  Throttle .... RETARD, ENGINE WARM

IF ENGINE DOES NOT START

[ ]  Prime w/Fuel Pump .... 3 SECONDS

[ ]  Repeat Steps .... STEPS 6, 7, 8

[ ]  Alternator .... ON

Before Engine Start

[ ]  Seats .... ADJUSTED

[ ]  Seat Belts, Harness .... FASTENED

[ ]  Parking Brake .... SET

[ ]  Circuit Breakers .... IN

[ ]  Radios .... OFF

[ ]  Cowl Flaps .... OPEN

[ ]  Alternate Air .... OFF

[ ]  Alternators .... ON

Starting Engines

[ ]  Mixture Controls .... IDLE CUT-OFF

[ ]  Throttle Controls .... OPEN ½ INCH

[ ]  Propeller Controls .... FORWARD

[ ]  Master Switch .... ON

[ ]  Magneto Switchs .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture Control .... RICH FOR 3 SEC.

[ ]  Propeller Area .... CLEAR

[ ]  Starter .... ENGAGE

[ ]  Mixture Control .... ADVANCE

[ ]  Oil Pressure .... CHECK

[ ]  Repeat Steps 8-11 for Other Engine .... AS REQUIRED

[ ]  Electric Fuel Pumps .... OFF

Hot Engine Start

[ ]  Mixture Controls .... IDLE CUT-OFF

[ ]  Throttle Controls .... OPEN ½ INCH

[ ]  Propeller Controls .... FORWARD

[ ]  Master Switch .... ON

[ ]  Magneto Switch .... ON

[ ]  Electric Fuel Pump .... OFF

[ ]  Propeller Area .... CLEAR

[ ]  Starter .... ENGAGE

[ ]  Mixture Control .... ADVANCE

[ ]  Oil Pressure .... CHECK

[ ]  Repeat Steps 7-10 for Other Engine .... AS REQUIRED

Flooded Engine Start

[ ]  Mixture Controls .... IDLE CUT-OFF

[ ]  Throttle Controls .... FULL FORWARD

[ ]  Propeller Controls .... FORWARD

[ ]  Master Switch .... ON

[ ]  Magneto Switch .... ON

[ ]  Electric Fuel Pump .... OFF

[ ]  Propeller Area .... CLEAR

[ ]  Starter .... ENGAGE

NOTE: When engine fires, retard throttle and advance mixture slowly.

Pre-Takeoff Check

[ ]  Parking Brake .... ON

ENGINE RUN-UP

[ ]  Mixture Controls .... FORWARD

[ ]  Propeller Controls .... FORWARD

[ ]  Throttle Controls .... 1500 RPM

[ ]  Propeller Controls .... CHECK FEATHER

[ ]  Throttle Controls .... 2000 RPM

[ ]  Propeller Governor .... CHECK

[ ]  Propeller Controls .... EXERCISE

[ ]  Propeller Controls .... FULL FORWARD

[ ]  Alternate Air Controls .... CHECK OFF/ON

[ ]  Engine Gauges .... GREEN

[ ]  Vacuum Gauge .... 4.5 to 5.2

[ ]  Magnetos .... CHECK

[ ]  Alternators .... CHECK

[ ]  Throttles .... 800-1000 RPM

[ ]  Fuel .... ON

[ ]  Alternators .... ON

[ ]  Altimeter .... SET

[ ]  Attitude Indicator .... SET

[ ]  Directional Gyro .... SET

[ ]  Clock .... WOUND & SET

[ ]  Mixtures .... SET

Pre-Takeoff Check - Continued

[ ]  Propellers .... FORWARD

[ ]  Friction Lock .... ADJUST

[ ]  Alternate Air .... OFF

[ ]  Cowl Flaps .... SET

[ ]  Seat Backs .... ERECT

[ ]  Wing Flaps .... SET

[ ]  Trim (Stabilator & Rudder) .... SET

[ ]  Seat Belts, Harnesses .... FASTENED

[ ]  Empty Seat Belts .... FASTENED

[ ]  Controls .... FREE & CORRECT

[ ]  Doors .... LATCHED

[ ]  Electric Fuel Pumps .... ON

[ ]  Pitot Heat .... AS REQUIRED

[ ]  Transponder .... ON

[ ]  Lights .... AS REQUIRED

Pre-Landing Check

[ ]  Seat Backs .... ERECT

[ ]  Seat Belts, Harnesses .... FASTENED

[ ]  Fuel Selectors .... ON

[ ]  Cowl Flaps .... AS REQUIRED

[ ]  Electric Fuel Pumps .... ON

[ ]  Mixture Controls .... RICH

[ ]  Propellers .... 2500 RPM

[ ]  Landing Gear .... DOWN

[ ]  Flaps .... AS REQUIRED

Post Landing Check

[ ]  Wing Flaps .... RETRACT

[ ]  Cowl Flaps .... OPEN

[ ]  Electric Fuel Pumps .... OFF

Shut Down

[ ]  Radios and Electric Equipment .... OFF

[ ]  Mixture Controls .... IDLE CUT-OFF

[ ]  Magneto Switches .... OFF

[ ]  Master Switch .... OFF

[ ]  Alternators .... OFF

[ ]  Parking Brake .... AS REQUIRED

Aircraft Performance Speeds

[ ]  VS0 – Landing configuration stall at gross weight, power off .... 69 MPH

[ ]  VS1 – Cruise configuration stall at gross weight, power off .... 76 MPH

[ ]  1.3 VS0 – Fence Speed .... 95 MPH

[ ]  1.5 VS0 – Approach Speed .... 105 MPH

[ ]  VX – Best Angle of Climb Speed .... 90 MPH

[ ]  VY – Best Rate of Climb Speed .... 105 MPH

[ ]  Normal Cruise .... 24 Hg 2400 RPM

[ ]  Slow Cruise .... 23 Hg 2300 RPM

[ ]  VFE – Max. Flap Extension .... 125 MPH

[ ]  VLE – Max. Gear Extension .... 150 MPH

[ ]  VLR – Max. Gear Retraction .... 125 MPH

[ ]  VA – Maneuvering Speed .... 133-146 MPH

[ ]  VNO – Top of Green Arc .... 190 MPH

[ ]  VNE – Red Line – Never Exceed .... 217 MPH

[ ]  VMC – Minimum Control Speed with Critical Engine Inoperative (Red Radial) .... 80 MPH

[ ]  VXSE – Best Angle of Climb Speed, One Engine Inoperative .... 90 MPH

[ ]  VYSE – Best Rate of Climb Speed, One Engine Inoperative .... 105 MPH

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-34-200 Seneca/jochman-clseneca.txt

Rights: licensed

Checklist hash: ea78004b180b5e22d1aba8d31797d995a9dc866405ec6aa52aefa9d6e19efb3a

Verify against the aircraft's own approved documentation before use.