Piper PA-34-200 Seneca — SenecaIIChecklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "CRUISE" item count differs between passes (5 vs 6); verify nothing was dropped.

content_gap: Section "ENGINE FAIL ABOVE 66 KIAS" item count differs between passes (9 vs 10); verify nothing was dropped.

content_gap: Section "FEATHER AND SECURE" item count differs between passes (12 vs 13); verify nothing was dropped.

PREFLIGHT CABIN

[ ]  Weight and Balance .... COMPLETE

[ ]  Documents .... CHECK AVAILABLE

[ ]  Airworthiness Certificate

[ ]  Registration

[ ]  Airplane Flight Manual

[ ]  Weight & Balance

[ ]  Landing Gear .... DOWN

[ ]  Avionics .... OFF

[ ]  Mixture .... IDLE-CUTOFF

[ ]  Cowl Flaps .... OPEN

[ ]  Trim Indicators .... NEUTRAL

[ ]  Flight Controls .... RELEASE CONTROL LOCK

[ ]  Master Switch .... ON

[ ]  Landing Gear .... 3 GREEN

[ ]  Fuel Quantity .... CHECK

[ ]  Flaps/Lights .... CHECK OPERATION

[ ]  Master/Ignition .... OFF

[ ]  Pitot Static Syst .... DRAIN

[ ]  Empty Seats .... SECURE

[ ]  Cross Feed .... DRAIN

EXTERNAL

[ ]  Cross Feed Drain .... CHECK CLOSED

RIGHT WING

[ ]  Flaps/Ailerons .... CHECK

[ ]  Tires/Brakes/Struts .... CHECK

[ ]  Rivets/Skin .... CHECK

[ ]  Landing/Nav Lights .... CHECK

[ ]  Fuel Quantity/Caps .... CHECK

[ ]  Oil (6-8qts) .... CHECK

[ ]  Prop/Spinner/Cowl Flap .... INSPECT

[ ]  Fuel Tank Drains .... CHECK

NOSE

[ ]  Windshield .... CHECK CLEAN

[ ]  Nose Gear/Land Light .... INSPECT

[ ]  Brake Reservoir .... CHECK LEVEL

[ ]  Doors/Fwd Baggage .... CLOSED – LOCKED

LEFT WING

[ ]  Engine/Landing Gear .... SAME RIGHT

[ ]  Stall Warning Vane .... INSPECT/CHECK

[ ]  Pitot Vane .... INSPECT

EMPANAGE

[ ]  Baggage Door .... SECURED

[ ]  Static Ports (L&R) .... CLEAR

[ ]  Dorsal Air Scoop .... FREE/CLEAR

[ ]  Empennage/Stabalator .... INSPECT

[ ]  Antennas .... SECURE

BEFORE ENGINE START/ START

[ ]  Seats/Belts .... ADJUSTED

[ ]  Radios .... OFF

[ ]  Parking Brake .... SET

[ ]  Circuit Breakers .... CHECK IN

[ ]  Mixture .... RICH/AS REQ

[ ]  Prop controls .... FULL FORWARD

[ ]  Throttles .... OPEN ½

[ ]  Alternate Air .... OFF

[ ]  Fuel Selectors .... ON

[ ]  Master Switch .... ON

[ ]  Magnetos .... ON

[ ]  Alternators .... ON

[ ]  Aux. Fuel Pump .... OFF

[ ]  Anti Collision Light .... ON

[ ]  Prop Area .... CLEAR

[ ]  Primer (depress for 5 sec) .... AS REQ

[ ]  Starter (30 sec max) .... ENGAGE

[ ]  Oil Pressure (w/in 30 sec) .... CHECK

[ ]  Repeat for opposite Engine

[ ]  Alternators .... CHECK

[ ]  Gyro Pressure .... CHECK

TAXI

[ ]  Brakes .... CHECK

[ ]  Instruments .... CHECK

[ ]  Fuel Selectors .... X FEED

AIRSPEEDS - SAFE OPERATION

[ ]  Vmc .... 66 KIAS

[ ]  Vyse .... 89 KIAS

[ ]  Vxse .... 78 KIAS

[ ]  Vx .... 76 KIAS

[ ]  Va .... 136 KIAS

[ ]  Vne .... 195 KIAS

[ ]  Vfe 10° .... 138 KIAS

[ ]  Vfe 25° .... 121 KIAS

[ ]  Vfe FULL .... 107 KIAS

[ ]  Vlo Extension .... 129 KIAS

[ ]  Vlo Exteded .... 129 KIAS

[ ]  Vlo Retract .... 107 KIAS

BEFORE TAKEOFF

[ ]  Parking Brake .... SET

[ ]  Mixture/Prop .... FULL FORWARD

[ ]  Quadrant Friction .... ADJUSTED

[ ]  Throttles .... 1000 RPM

[ ]  Manifold Lines (below 25″) .... DRAIN

[ ]  Prop Feather (200-300 RPM drop) .... CHECK FORWARD

[ ]  Throttles .... 1900 RPM

[ ]  Prop Governor .... CYCLE/FORWARD

[ ]  Alternate Air Control .... CHECK/OFF

[ ]  Magneto Switches (150max/50w/in) .... CHECK

[ ]  Alternator Output (equal) .... CHECK

[ ]  Gyro Pressure (4.5-5.2″) .... CHECK

[ ]  Throttles .... 800 – 1000 RPM

[ ]  Fuel Selectors .... ON

[ ]  Fuel Pumps .... OFF/ AS REQ

[ ]  Flight Controls .... FREE/ CORRECT

[ ]  Annunciator Panel .... PRESS TO TEST

[ ]  ALT/AI/DG .... SET

[ ]  Engine Instruments .... IN THE GREEN

[ ]  Flaps/Cowl Flaps/Trim .... SET FOR TAKEOFF

[ ]  Doors/ Windows .... CLOSE/ LOCKED

[ ]  Lights/Transponder .... ON/AS REQ

NORMAL TAKEOFF

[ ]  Crew Brief .... COMPLETE

[ ]  Flaps .... UP

[ ]  Throttles .... NOT EXCEED 40″

[ ]  Engine Instruments .... GREEN

[ ]  Airspeed .... ALIVE

[ ]  Vr .... 71 KIAS

[ ]  Vy .... 89 KIAS

[ ]  Gear .... UP POS RATE

SHORT FIELD TAKEOFF

[ ]  Flaps .... UP/ 25°

[ ]  Vr .... 61 KIAS

[ ]  (Thru 50′) .... 69 KIAS

[ ]  Vy .... 89 KIAS

[ ]  Gear/ Flaps .... UP POS RATE

NOTE: * Speed at Vr will be below Vmc

CLIMB

[ ]  Power Set .... 31.5″ MP

[ ]  Props Set .... 2450 RPM

[ ]  Cyl Head Temp .... BELOW 460°

CRUISE

[ ]  Power/ Props .... SET CRUISE

[ ]  Mixture .... SET PWR/ ECON

[ ]  Cowl Flaps .... AS REQ

[ ]  Cyl Head Temp: High Power Cruise .... BELOW 435°

[ ]  Cyl Head Temp: Economy Cruise .... BELOW 400°

DECENT

[ ]  Mixture .... ENRICH

[ ]  Throttles/Props .... AS REQ

[ ]  Cowl Flaps .... CLOSED

APPROACH AND LANDING

[ ]  ATIS/ ASOS/ AWOS .... OBTAIN

[ ]  Brief/ Seat Belts .... CHECK

[ ]  Gear Warning Horn .... TEST

[ ]  Fuel Selectors .... ON

[ ]  Cowl Flaps .... AS REQ

[ ]  Aux Fuel Pumps .... OFF/ AS REQ

[ ]  Mixture Controls .... AS REQ

[ ]  Props .... FULL FORWARD

[ ]  Landing Gear <Vlo .... DOWN/ 3GREEN

[ ]  Flaps <Vfe .... EXTEND

[ ]  Normal Appr .... 85 KIAS

[ ]  Short Field .... 78 KIAS

GO AROUND /MISSED APPR

[ ]  Throttles .... CLIMB POWER

[ ]  Flaps .... 10°

[ ]  Alternate Air .... OFF

[ ]  Gear (at pos R.O.C) .... UP

[ ]  Flaps (@ 89 KIAS) .... RETRACT

[ ]  Climb .... VY 89 KIAS

AFTER LANDING

[ ]  Flaps .... UP

[ ]  Cowl Flaps .... OPEN

[ ]  Alternate Air .... OFF

[ ]  Mixture .... LEAN FOR TAXI

[ ]  Transponder .... OFF

[ ]  Strobes .... OFF

SECURE

[ ]  Heater .... OFF

[ ]  Avionics/ Nav Lights .... OFF

[ ]  Magneto Grounding .... CHECK

[ ]  Alternators .... OFF

[ ]  Mixture .... IDLE CUTOFF

[ ]  Master Switch .... OFF

[ ]  Magnetos .... OFF

ENGINE FAIL ABOVE 66 KIAS  [EMERGENCY]

[ ]  Maintain Aircraft Control .... PITCH 89 KIAS

[ ]  Rudder .... APPLY TOWARD GOOD ENGINE

[ ]  Mixture .... FULL FORWARD

[ ]  Props .... FULL FORWARD

[ ]  Throttles .... FULL FORWARD

[ ]  Flaps .... CHECK RETRACT

[ ]  Gear .... CHECK RETRACT

[ ]  IDENTIFY [MEMORY] .... DEAD FOOT

[ ]  VERIFY [MEMORY] .... RETARD THROTTLE

TROUBLESHOOT  [EMERGENCY]

[ ]  Fuel Flow .... CHECK

[ ]  Aux Fuel Pumps .... HI, OFF

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Selector .... CROSS FEED

[ ]  Alternate Air .... ON

[ ]  Mixture .... CHECK RICH

[ ]  Oil Pressure/ Temperature .... CHECK

[ ]  Magnetos .... L, THEN R

NOTE: *If engine does not start, proceed below

FEATHER AND SECURE  [EMERGENCY]

[ ]  Re-verify Dead Engine with Throttles

[ ]  Mixture .... IDLE CUTOFF

[ ]  Prop (> 800 RPM) .... FEATHER

[ ]  Fuel Pump .... OFF

[ ]  Fuel Selector .... CONSIDER X-FEED

[ ]  Fuel Quantity .... SUFFICIENT FOR CROSS FEED

[ ]  Cowl Flaps - Operating Engine .... OPEN

[ ]  Cowl Flaps - Dead Engine .... CLOSED

[ ]  Rudder Trim Toward Good Engine

[ ]  Magneto .... OFF

[ ]  Alternator .... OFF

[ ]  Electrical Load .... REDUCE

SINGLE ENGINE LANDING  [EMERGENCY]

NOTE: * Do Not extend Gear/ Flaps until landing is assured

NOTE: * Final Approach 91KIAS, Flaps 25° Max

EMERCENCIE DECENT  [EMERGENCY]

[ ]  Throttles .... CLOSE

[ ]  Props .... FULL FORWARD

[ ]  Gear (at 129 KIAS) .... DOWN

[ ]  Pitch .... 129 KIAS

ENGINE FIRE DURING FLIGHT  [EMERGENCY]

[ ]  Fuel Selector .... OFF

[ ]  Throttle .... CLOSE

[ ]  Prop .... FEATHER

[ ]  Mixture .... IDLE CUTOFF

[ ]  Heater/Defrost .... OFF

NOTE: * If terrain permits, Land immediately if fire continues.

MANUAL GEAR EXTENSION  [EMERGENCY]

NOTE: Check Following before extending gear:

[ ]  Circuit Beakers .... CHECK IN

[ ]  Master Switch .... ON

[ ]  Alternators .... CHECK

[ ]  Navigation Lights .... OFF (DAY)

NOTE: *To extend, reposition clip downward clear of knob and proceed as follows:

[ ]  Airspeed .... 85 KIAS

[ ]  Gear Selector .... DOWN POSITION

[ ]  Emer Gear Extend Knob .... PULL OUT

[ ]  Indicator Lights .... 3 GREEN

NOTE: *Leave emergency gear knob in the out position until after landing.

UFEATHERING PROP  [EMERGENCY]

[ ]  Fuel Selector .... ON

[ ]  Aux Fuel Pump .... OFF

[ ]  Throttle .... OPEN ¼

[ ]  Prop .... FULL FORWARD

[ ]  Mixture .... RICH

[ ]  Magneto .... ON

[ ]  Starter (Engage until prop windmills)

NOTE: *Prime if engine fails to start

[ ]  Power (until engine warm) .... REDUCE

[ ]  Alternator .... ON

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-34-200 Seneca/SenecaIIChecklist.txt

Rights: licensed

Checklist hash: 0946e60dbe46b3adc4290d314d5cd3ac07df09a54fdff72a1b16fce433f251f0

Verify against the aircraft's own approved documentation before use.