Piper PA-38-112 Tomahawk — Douglas_White_Dave_Jochman_Piper_PA38_Tomahawk_Checklist_

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Preflight Inspection

COCKPIT

[ ]  1. Control Lock .... REMOVE

[ ]  2. Ignition .... OFF

[ ]  3. Master Switch .... ON

[ ]  4. Fuel Gauges .... CHECK

[ ]  5. Alternator Warning Light .... CHECK

[ ]  6. Master Switch .... OFF

[ ]  7. Primary Flight Controls .... CHECK

[ ]  8. Flaps .... CHECK

[ ]  9. Static Drain .... DRAINED

[ ]  10. Windows .... CLEAN

[ ]  11. Baggage .... STOWED

[ ]  12. Required Papers .... ON BOARD

[ ]  13. Parking Brake .... ON

LEFT WING

[ ]  1. Surface Condition .... CHECK

[ ]  2. Flap and Hinges .... CHECK

[ ]  3. Aileron and hinges .... CHECK

[ ]  4. Wing Tip .... CHECK

[ ]  5. Lights .... CHECK

[ ]  6. Fuel Cap .... OPEN

[ ]  7. Fuel Quantity and Color .... CHECK

[ ]  8. Fuel Cap .... CLOSE and SECURE

[ ]  9. Fuel Vent .... CHECK

[ ]  10. Fuel Tank Sump .... DRAIN

[ ]  11. Pitot Head .... UNOBSTRUCTED

[ ]  12. Stall Warning .... CHECK

[ ]  13. Landing Gear and Tire .... CHECK

[ ]  14. Brake Block and Disk .... CHECK

[ ]  15. Chock and Tie Down .... REMOVED

NOSE SECTION

[ ]  1. Fuel Strainer .... DRAIN

[ ]  2. General Condition .... CHECK

[ ]  3. Prop and Spinner .... CHECK

[ ]  4. Air Inlets .... CLEAR

[ ]  5. Engine Compartment .... CHECK

[ ]  6. Oil .... CHECK QUANTITY

[ ]  7. Dipstick .... PROPERLY SEATED

[ ]  8. Alternator Belt .... CHECK TENSION

[ ]  9. Cowling .... CLOSED and SECURE

[ ]  10. Nose Gear Tire .... CHECK

[ ]  11. Nose Wheel Strut .... Proper Inflation (3")

[ ]  12. Windshield .... CLEAN

RIGHT WING

[ ]  1. Check as Left Wing .... CHECK

FUSELAGE (RIGHT SIDE)

[ ]  1. General Condition .... CHECK

[ ]  2. Antennas .... CHECK

[ ]  3. Side and Rear Window .... CHECK

[ ]  4. Static Vents .... UNOBSTRUCTED

EMPENNAGE

[ ]  1. General Condition .... CHECK

[ ]  2. Hinges and Attachments .... CHECK

[ ]  3. Tie Down .... REMOVED

FUSELAGE (RIGHT SIDE)

[ ]  1. Check as Left Side .... CHECK

Before Starting Engine

[ ]  1. Cabin Doors .... CLOSED AND LATCHED

[ ]  2. Overhead Latch .... ENGAGED

[ ]  3. Seats .... ADJUSTED AND LOCKED

[ ]  4. Seat Belts & Harnesses .... FASTENED

[ ]  5. Circuit Breakers .... IN

[ ]  6. Parking Brake .... SET

[ ]  7. Carburetor Heat .... FULL OFF

[ ]  8. Fuel Selector .... DESIRED TANK

Engine Start (COLD)

[ ]  1. Prime .... AS REQUIRED (LEAVE OUT)

[ ]  2. Throttle .... OPEN ½ INCH

[ ]  3. Master Switch .... ON

[ ]  4. Electric Fuel Pump .... ON

[ ]  5. Mixture .... FULL RICH

[ ]  6. Starter .... ENGAGE

[ ]  7. Primer .... PUSH IN SLOWLY

[ ]  8. Throttle .... ADVANCE SLIGHTLY

[ ]  9. Oil Pressure .... CHECK

[ ]  10. Electric Fuel Pump .... OFF

[ ]  11. Fuel Pressure .... CHECK

[ ]  12. Primer .... LOCKED

Engine Start (Hot)

[ ]  1. Throttle .... CRACKED

[ ]  2. Master Switch .... ON

[ ]  3. Electric Fuel Pump .... ON

[ ]  4. Mixture .... FULL RICH

[ ]  5. Starter .... ENGAGE

[ ]  6. Throttle .... ADJUST

[ ]  7. Oil Pressure .... CHECK

[ ]  8. Electric Fuel Pump .... OFF

[ ]  9. Fuel Pressure .... CHECK

Engine Start (Flooded)

[ ]  1. Throttle .... OPEN FULL

[ ]  2. Master Switch .... ON

[ ]  3. Electric Fuel Pump .... ON

[ ]  4. Mixture .... IDLE CUT-OFF

[ ]  5. Starter .... ENGAGE

[ ]  6. Mixture .... ADVANCE

[ ]  7. Throttle .... RETARD

[ ]  8. Oil Pressure .... CHECK

[ ]  9. Fuel Pressure .... CHECK

Starting Engine (External Power)

[ ]  1. Master Switch .... OFF

[ ]  2. All Electrical Equipment .... OFF

[ ]  3. Terminals .... CONNECT

[ ]  4. External Power Plug .... INSERT

[ ]  5. Proceed with Normal Start .... START

[ ]  6. Throttle .... LOWEST RPM

[ ]  7. External Power Plug .... DISCONNECT

[ ]  8. Master Switch .... ON – CHECK AMMETER

[ ]  9. Oil Pressure .... CHECK

Warm Up

[ ]  1. Throttle .... 800 to 1200 RPM

Taxiing

[ ]  1. Intercom .... ON

[ ]  2. Radios .... ON

[ ]  3. Taxi Area .... CLEAR

[ ]  4. Brakes .... CHECK

[ ]  5. Steering .... CHECK

Ground Check

[ ]  1. Brakes .... SET

[ ]  2. Throttle .... 1800 RPM

[ ]  3. Magnetos .... 175 Max/ 50 Diff.

[ ]  4. Vacuum .... 5.0" Hg ± .1

[ ]  5. Oil Temp .... CHECK

[ ]  6. Oil Pressure .... CHECK

[ ]  7. Carburetor Heat .... CHECK

[ ]  8. Throttle .... RETARD

Before Takeoff

[ ]  1. Master Switch .... CHECK ON

[ ]  2. Flight Instruments .... CHECK

[ ]  3. Fuel Selector .... PROPER TANK

[ ]  4. Mixture .... SET

[ ]  5. Electric Fuel Pump .... ON

[ ]  6. Carburetor Heat .... OFF

[ ]  7. Engine Gauges .... CHECK

[ ]  8. Static Source .... NORMAL

[ ]  9. Seats .... LATCHED IN TRACK

[ ]  10. Seat Backs .... ERECT

[ ]  11. Belts/Harness .... FASTENED

[ ]  12. Empty Seat Belt .... FASTENED

[ ]  13. Flaps .... SET

[ ]  14. Trim Tab .... SET

[ ]  15. Controls .... FREE

[ ]  16. Doors .... CLOSED and LATCHED

[ ]  17. Overhead Latch .... ENGAGED

[ ]  18. Transponder .... ON ALT

Normal Takeoff

[ ]  1. Flaps .... SET

[ ]  2. Trim Tab .... SET

[ ]  3. Accelerate .... 53 KIAS

[ ]  4. Control Wheel .... Back Pressure to Rotate to Climb Attitude

Short Field Takeoff, Obstacle Clearance

[ ]  1. Flaps .... 21° (first notch)

[ ]  2. Accelerate .... 53 KIAS

[ ]  3. Control Wheel .... Back Pressure to Rotate to Climb Attitude

[ ]  4. Accelerate (until over obstacle) .... 61 KIAS

[ ]  5. Accelerate (after over obstacle) .... 70 KIAS

[ ]  6. Flaps .... RETRACT SLOWLY

Short Field Takeoff, No Obstacle

[ ]  1. Flaps .... 21° (first notch)

[ ]  2. Accelerate .... 53 KIAS

[ ]  3. Control Wheel .... Back Pressure to Rotate to Climb Attitude

[ ]  4. Accelerate (after breaking ground) .... 70 KIAS

[ ]  5. Flaps .... RETRACT SLOWLY

Soft Field Takeoff, Obstacle Clearance

[ ]  1. Flaps .... 21° (first notch)

[ ]  2. Accelerate and lift off nose as soon as possible.

[ ]  3. Lift off at slowest possible airspeed.

[ ]  4. Accelerate just above ground .... 61 KIAS to climb past obstacle height

[ ]  5. Accelerate (after over obstacle) .... 70 KIAS

[ ]  6. Flaps .... RETRACT SLOWLY

Soft Field Takeoff, No Obstacle

[ ]  1. Flaps .... 21° (first notch)

[ ]  2. Accelerate and lift off nose as soon as possible.

[ ]  3. Lift off at slowest possible airspeed.

[ ]  4. Accelerate just above ground best rate of climb speed .... 70 KIAS

[ ]  5. Flaps .... RETRACT SLOWLY

Climb

[ ]  1. VX – Best Angle of Climb Speed .... 61 KIAS

[ ]  2. VY – Best Rate of Climb Speed .... 70 KIAS

[ ]  3. Electric Fuel Pump .... OFF

Cruising

[ ]  1. Normal Max. Power .... 75%

[ ]  2. Power .... SET PER TABLE

[ ]  3. Mixture .... ADJUST

[ ]  4. Fuel Selector .... SWITCH AS NEEDED

Approach and Landing

[ ]  1. Fuel Selector .... PROPER TANK

[ ]  2. Seat Backs .... ERECT

[ ]  3. Belts/Harness .... FASTEN

[ ]  4. Electric Fuel Pump .... ON

[ ]  5. Mixture .... SET

[ ]  6. Flaps .... SET (89 KIAS MAX)

[ ]  7. Trim .... 70 KIAS

[ ]  8. Final Approach Speed .... 67 KIAS

Stopping Engine

[ ]  1. Flaps .... RETRACT

[ ]  2. Electric Fuel Pump .... OFF

[ ]  3. Radios .... OFF

[ ]  4. Magnetos .... CHECK GROUND

[ ]  5. Throttle .... FULL AFT

[ ]  6. Mixture .... IDLE CUT-OFF

[ ]  7. Magnetos .... OFF

[ ]  8. Master Switch .... OFF

[ ]  9. Intercom .... OFF

Parking

[ ]  1. Parking Brake .... SET

[ ]  2. Control Wheel .... SECURED WITH BELTS

[ ]  3. Flaps .... FULL Up

[ ]  4. Wheel Chocks .... IN PLACE

[ ]  5. Tie Downs .... SECURE

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-38-112 Tomahawk/Douglas_White_Dave_Jochman_Piper_PA38_Tomahawk_Checklist_.txt

Rights: licensed

Checklist hash: f130c6950ea4232e658c0c5586eadb22ce27c102d37194d4c9974174bc7f2072

Verify against the aircraft's own approved documentation before use.