Piper PA-38-112 Tomahawk — jochman-cltmhawk

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Preflight Inspection — Cockpit" item count differs between passes (13 vs 14); verify nothing was dropped.

content_gap: Section "Preflight Inspection — Left Wing" item count differs between passes (15 vs 16); verify nothing was dropped.

content_gap: Section "Preflight Inspection — Nose Section" item count differs between passes (12 vs 13); verify nothing was dropped.

Aircraft Performance Speeds

[ ]  VS0 – Landing configuration stall at gross weight, power off

[ ]  VS1 – Cruise configuration stall at gross weight, power off

[ ]  VR

[ ]  VR (Short/Soft)

[ ]  VREF

[ ]  VREF (Short/Soft)

[ ]  VX – Best Angle of Climb Speed

[ ]  VY – Best Rate of Climb Speed

[ ]  VFE – Max. Flap Extension

[ ]  VA – Maneuvering Speed (1670#)

[ ]  VA – Maneuvering Speed (1277#)

[ ]  VNO – Top of Green Arc

[ ]  VNE – Red Line – Never Exceed

[ ]  VL/D

[ ]  Max. Demonstrated X-wind

Aircraft Specifications

[ ]  Maximum Gross Weight

[ ]  Basic Empty Weight (N2398C)

[ ]  Useful Load (N2398C)

[ ]  Fuel Capacity (useable)

[ ]  Fuel Consumption

[ ]  Cruise Speed

Preflight Inspection — Cockpit

[ ]  Control Lock .... REMOVE

[ ]  Ignition .... OFF

[ ]  Master Switch .... ON

[ ]  Fuel Gauges .... CHECK

[ ]  Alternator Warning Light .... CHECK

[ ]  Master Switch .... OFF

[ ]  Primary Flight Controls .... CHECK

[ ]  Flaps .... CHECK

[ ]  Static Drain .... DRAINED

[ ]  Windows .... CLEAN

[ ]  Baggage .... STOWED

[ ]  Required Papers .... ON BOARD

[ ]  Parking Brake .... ON

Preflight Inspection — Left Wing

[ ]  Surface Condition .... CHECK

[ ]  Flap and Hinges .... CHECK

[ ]  Aileron and hinges .... CHECK

[ ]  Wing Tip .... CHECK

[ ]  Lights .... CHECK

[ ]  Fuel Cap .... OPEN

[ ]  Fuel Quantity and Color .... CHECK

[ ]  Fuel Cap .... CLOSE and SECURE

[ ]  Fuel Vent .... CHECK

[ ]  Fuel Tank Sump .... DRAIN

[ ]  Pitot Head .... UNOBSTRUCTED

[ ]  Stall Warning .... CHECK

[ ]  Landing Gear and Tire .... CHECK

[ ]  Brake Block and Disk .... CHECK

[ ]  Chock and Tie Down .... REMOVED

Preflight Inspection — Nose Section

[ ]  Fuel Strainer .... DRAIN

[ ]  General Condition .... CHECK

[ ]  Prop and Spinner .... CHECK

[ ]  Air Inlets .... CLEAR

[ ]  Engine Compartment .... CHECK

[ ]  Oil .... CHECK QUANTITY

[ ]  Dipstick .... PROPERLY SEATED

[ ]  Alternator Belt .... CHECK TENSION

[ ]  Cowling .... CLOSED and SECURE

[ ]  Nose Gear Tire .... CHECK

[ ]  Nose Wheel Strut .... Proper Inflation (3”)

[ ]  Windshield .... CLEAN

Preflight Inspection — Right Wing

[ ]  Check as Left Wing

Preflight Inspection — Fuselage (Right Side)

[ ]  General Condition .... CHECK

[ ]  Antennas .... CHECK

[ ]  Side and Rear Window .... CHECK

[ ]  Static Vents .... UNOBSTRUCTED

Preflight Inspection — Empennage

[ ]  General Condition .... CHECK

[ ]  Hinges and Attachments .... CHECK

[ ]  Tie Down .... REMOVED

Preflight Inspection — Fuselage (Left Side)

[ ]  Check as Left Side

Before Starting Engine

[ ]  Cabin Doors .... CLOSED AND LATCHED

[ ]  Overhead Latch .... ENGAGED

[ ]  Seats .... ADJUSTED AND LOCKED

[ ]  Seat Belts & Harnesses .... FASTENED

[ ]  Circuit Breakers .... IN

[ ]  Parking Brake .... SET

[ ]  Carburetor Heat .... FULL OFF

[ ]  Fuel Selector .... DESIRED TANK

Engine Start (COLD)

[ ]  Prime .... AS REQUIRED (LEAVE OUT)

[ ]  Throttle .... OPEN ½ INCH

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... FULL RICH

[ ]  Starter .... ENGAGE

[ ]  Primer .... PUSH IN SLOWLY

[ ]  Throttle .... ADVANCE SLIGHTLY

[ ]  Oil Pressure .... CHECK

[ ]  Electric Fuel Pump .... OFF

[ ]  Fuel Pressure .... CHECK

[ ]  Primer .... LOCKED

Engine Start (Hot)

[ ]  Throttle .... CRACKED

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... FULL RICH

[ ]  Starter .... ENGAGE

[ ]  Throttle .... ADJUST

[ ]  Oil Pressure .... CHECK

[ ]  Electric Fuel Pump .... OFF

[ ]  Fuel Pressure .... CHECK

Engine Start (Flooded)

[ ]  Throttle .... OPEN FULL

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Starter .... ENGAGE

[ ]  Mixture .... ADVANCE

[ ]  Throttle .... RETARD

[ ]  Oil Pressure .... CHECK

[ ]  Fuel Pressure .... CHECK

Starting Engine (External Power)

[ ]  Master Switch .... OFF

[ ]  All Electrical Equipment .... OFF

[ ]  Terminals .... CONNECT

[ ]  External Power Plug .... INSERT

[ ]  Proceed with Normal Start

[ ]  Throttle .... LOWEST RPM

[ ]  External Power Plug .... DISCONNECT

[ ]  Master Switch .... ON – CHECK AMMETER

[ ]  Oil Pressure .... CHECK

Warm Up

[ ]  Throttle .... 800 to 1200 RPM

Taxiing

[ ]  Intercom .... ON

[ ]  Radios .... ON

[ ]  Taxi Area .... CLEAR

[ ]  Brakes .... CHECK

[ ]  Steering .... CHECK

Ground Check

[ ]  Brakes .... SET

[ ]  Throttle .... 1800 RPM

[ ]  Magnetos .... 175 Max/ 50 Diff.

[ ]  Vacuum .... 5.0” Hg ± .1

[ ]  Oil Temp .... CHECK

[ ]  Oil Pressure .... CHECK

[ ]  Carburetor Heat .... CHECK

[ ]  Throttle .... RETARD

Before Takeoff

[ ]  Master Switch .... CHECK ON

[ ]  Flight Instruments .... CHECK

[ ]  Fuel Selector .... PROPER TANK

[ ]  Mixture .... SET

[ ]  Electric Fuel Pump .... ON

[ ]  Carburetor Heat .... OFF

[ ]  Engine Gauges .... CHECK

[ ]  Static Source .... NORMAL

[ ]  Seats .... LATCHED IN TRACK

[ ]  Seat Backs .... ERECT

[ ]  Belts/Harness .... FASTENED

[ ]  Empty Seat Belt .... FASTENED

[ ]  Flaps .... SET

[ ]  Trim Tab .... SET

[ ]  Controls .... FREE

[ ]  Doors .... CLOSED and LATCHED

[ ]  Overhead Latch .... ENGAGED

[ ]  Transponder .... ON ALT

Normal Takeoff

[ ]  Flaps .... SET

[ ]  Trim Tab .... SET

[ ]  Accelerate .... 53 KIAS

[ ]  Control Wheel, Back Pressure to Rotate to Climb Attitude

Short Field Takeoff, Obstacle Clearance

[ ]  Flaps .... 21° (first notch)

[ ]  Accelerate .... 53 KIAS

[ ]  Control Wheel, Back Pressure to Rotate to Climb Attitude

[ ]  Accelerate (until over obstacle) .... 61 KIAS

[ ]  Accelerate (after over obstacle) .... 70 KIAS

[ ]  Flaps .... RETRACT SLOWLY

Short Field Takeoff, No Obstacle

[ ]  Flaps .... 21° (first notch)

[ ]  Accelerate .... 53 KIAS

[ ]  Control Wheel, Back Pressure to Rotate to Climb Attitude

[ ]  Accelerate (after breaking ground) .... 70 KIAS

[ ]  Flaps .... RETRACT SLOWLY

Soft Field Takeoff, Obstacle Clearance

[ ]  Flaps .... 21° (first notch)

[ ]  Accelerate and lift off nose as soon as possible.

[ ]  Lift off at slowest possible airspeed.

[ ]  Accelerate just above ground to 61 KIAS to climb past obstacle height.

[ ]  Accelerate (after over obstacle) .... 70 KIAS

[ ]  Flaps .... RETRACT SLOWLY

Soft Field Takeoff, No Obstacle

[ ]  Flaps .... 21° (first notch)

[ ]  Accelerate and lift off nose as soon as possible.

[ ]  Lift off at slowest possible airspeed.

[ ]  Accelerate just above ground best rate of climb speed

[ ]  Flaps .... RETRACT SLOWLY

Climb

[ ]  VX – Best Angle of Climb Speed .... 61 KIAS

[ ]  VY – Best Rate of Climb Speed .... 70 KIAS

[ ]  Electric Fuel Pump .... OFF

Cruising

[ ]  Normal Max. Power .... 75%

[ ]  Power .... SET PER TABLE

[ ]  Mixture .... ADJUST

[ ]  Fuel Selector .... SWITCH AS NEEDED

Approach and Landing

[ ]  Fuel Selector .... PROPER TANK

[ ]  Seat Backs .... ERECT

[ ]  Belts/Harness .... FASTEN

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... SET

[ ]  Flaps .... SET (89 KIAS MAX)

[ ]  Trim .... 70 KIAS

[ ]  Final Approach Speed .... 67 KIAS

Stopping Engine

[ ]  Flaps .... RETRACT

[ ]  Electric Fuel Pump .... OFF

[ ]  Radios .... OFF

[ ]  Magnetos .... CHECK GROUND

[ ]  Throttle .... FULL AFT

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

[ ]  Intercom .... OFF

Parking

[ ]  Parking Brake .... SET

[ ]  Control Wheel .... SECURED WITH BELTS

[ ]  Flaps .... FULL Up

[ ]  Wheel Chocks .... IN PLACE

[ ]  Tie Downs .... SECURE

Weight and Balance Worksheet

[ ]  Basic Empty Weight — Weight 1180, Arm 73.3, Moment 86,439

[ ]  Pilot — Arm 85.5

[ ]  Co-Pilot — Arm 85.5

[ ]  Baggage #1 (100 lb Max) — Arm 115.0

[ ]  Fuel Gal. (30 Max) — Arm 75.4

[ ]  Total (M.G.W.=1670 lbs)

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-38-112 Tomahawk/jochman-cltmhawk.txt

Rights: licensed

Checklist hash: 14ac6790ef5a001d653c555e62f36f46bf5aeb02c5551209baca599051af6ac7

Verify against the aircraft's own approved documentation before use.