Piper PA-38-112 Tomahawk — Joris_De_Naeyer_Piper_pa38_dewouw

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Important

NOTE: This checklist is not an authoritative document. Full reference must be made to the individual aircraft flight Manual/Pilot Operating handbook as amended. Current ANO and AICs and Pilot Order book/Flying School Syllabus, local procedures, may also be relevant.

Pre-Flight Inspection

[ ]  Aircraft Documents — Documentation complete and satisfactory, including valid Certificate of Maintenance if required.

[ ]  Weather — Local, en-route and destination forecasts and actuals.

[ ]  Flight Planning — Current AIP/Flight Guide, NOTAMS, Amendments and bulletins. PPR, Daylight remaining, etc.

[ ]  Fuel — Sufficient to destination and alternates, Contingency.

[ ]  Charts — Current and sufficient coverage for flight.

[ ]  Weight & Balance — Load (passengers, baggage, fuel) within limits.

[ ]  Performance — Take-off, en-route and range, landing.

[ ]  Passengers — Comfort and safety briefing, emergency procedures.

[ ]  Pilot — Licence and experience current and valid. Medical certificate valid, fitness for flight.

[ ]  Survival Equipment — For over water flight - dinghy accessible, lifejackets worn but not inflated, Flares, Transceiver, ELT.

[ ]  Book-Out — Departure ATC, airfield movements book, aircraft tech. log.

[ ]  Flight Plan — File for IFR and International Flights, flight across water and sparsely populated areas.

Pre-Flight Action Of Pilot In Command — Approaching Aircraft

[ ]  Check — Access to taxiways, obstructions, level surface, for fuel / oil spillages.

[ ]  Remove — ANY ice or frost from ALL aircraft surfaces.

[ ]  Remove — Tie downs and Towbar.

[ ]  Remove — External control locks.

[ ]  Remove — Pitot cover.

[ ]  Remove — Chocks.

Pre-Flight Action Of Pilot In Command — In Cabin

[ ]  Control Locks & Covers .... REMOVE AND STOW

[ ]  Parking Brake .... ON

[ ]  Magneto Switches .... CHECK OFF, KEY OUT

[ ]  Master Switch .... ON

[ ]  Stall Warner .... CHECK

[ ]  Pitot Heat .... CHECK, THEN OFF

[ ]  Anti-Collision Lights .... CHECK, THEN OFF

[ ]  Landing/Nav. Lights .... CHECK AS REQUIRED, THEN OFF

[ ]  Fuel .... ON - CHECK CONTENTS

[ ]  Master Switch .... OFF

[ ]  Throttle .... CLOSED

[ ]  Mixture .... IDLE CUTOFF

[ ]  Flaps .... LOWER 21° (1ST STAGE)

[ ]  Trimmer .... CHECK POSITION

[ ]  First Aid Kit .... IN POSITION - SECURE

[ ]  Fire Extinguisher .... IN POSITION - SECURE

Preliminary External — Left Wing

[ ]  Flap .... CONDITION LINKAGES

[ ]  Aileron — Condition, Linkages and Hinges .... FULL AND FREE MOVEMENT

[ ]  Wing Tip — Condition, Security, Navigation and Strobe lights .... CHECK

[ ]  Wing Surface .... CONDITION, UPPER AND LOWER

[ ]  Leading Edge — Dents, Stall Warner, Pitot Head checked .... CHECK

[ ]  Fuel Tank .... VISUALLY CHECK CONTENTS, CAP SECURE, FUEL DRAIN, VENT CLEAR

Preliminary External — Left Undercarriage

[ ]  Tyre — Condition, inflation, creep marks aligned .... CHECK

[ ]  Hydraulic Lines .... CONDITION, LEAKS

[ ]  Disc Brake .... CONDITION

[ ]  Leg and Fairing .... CONDITION

Preliminary External — Front Fuselage and Engine

[ ]  Left Cowling — Check brake fluid level, engine compartment, secure panel, fuel drain .... CHECK

[ ]  Windscreen .... CLEAN, OAT PROBE SECURE

[ ]  Nose Leg — Oleo extension, torque link, nuts and split pins .... CHECK

[ ]  Nose Wheel — Condition, inflation, creep marks aligned .... CHECK

[ ]  Front Cowling — Condition and security, intakes clear, landing light .... CHECK

[ ]  Propellor .... CONDITION, ESPECIALLY LEADING EDGE

[ ]  Right Cowling — Engine compartment, oil level, secure panel .... CHECK

External / 1 — Right Undercarriage

[ ]  Tyre — Condition, inflation, creep marks aligned .... CHECK

[ ]  Hydraulic Lines .... CONDITION, LEAKS

[ ]  Disc Brake .... CONDITION

[ ]  Leg and Fairing .... CONDITION

External / 1 — Right Wing

[ ]  Fuel Tank .... VISUALLY CHECK CONTENTS, CAP SECURE, FUEL DRAIN, VENT CLEAR

[ ]  Leading Edge .... CONDITION, CHECK FOR DENTS

[ ]  Wing Surface .... CONDITION, UPPER AND LOWER

[ ]  Wing Tip — Condition, Security, Navigation and Strobe lights .... CHECK

[ ]  Aileron — Condition, Linkages and Hinges .... FULL AND FREE MOVEMENT

[ ]  Flap .... CONDITION LINKAGES

External / 1 — Right Fuselage

[ ]  Door .... LATCH AND HINGES SECURE

[ ]  Windows .... CLEAN

[ ]  Skin .... CONDITION

[ ]  Antenna’s .... SECURE

[ ]  Static Vent .... CLEAR

External / 1 — Tail Unit

[ ]  Tail Fin — Condition and security, fairings, aerials .... CHECK

[ ]  Rudder — Conditions, linkages, nuts and split pins, navigation light .... CHECK

[ ]  Tailplane/Elevator .... CONDITIONS, LINKAGES

External / 2 — Left Fuselage

[ ]  Skin .... CONDITION

[ ]  Aerials .... SECURE

[ ]  Static Vent .... CLEAR

[ ]  Windows .... CLEAN

[ ]  Door .... LATCHES AND HINGES SECURE

External / 2 — Internal

[ ]  Seats .... ADJUSTED AND LOCKED

[ ]  Hatches & Harnesses .... ADJUSTED AND SECURE

[ ]  Parking Brake .... ON

[ ]  Radios .... OFF

[ ]  Instruments — Legible, serviceable, readings within limit .... CHECK

[ ]  Controls .... FULL AND FREE MOVEMENT, CORRECT SENSE

[ ]  Trimmer .... CHECK THROUGH FULL RANGE, SET NEUTRAL

[ ]  Flaps .... CHECK IN STAGES, UP

[ ]  Cabin Air Control .... CLOSED (OFF)

[ ]  Alternate Static .... OFF

[ ]  Carb. Heat .... FULL AND FREE MOVEMENT, SET COLD

[ ]  Throttle .... FULL AND FREE MOVEMENT, SET 0.5 CM OPEN

[ ]  Throttle Friction .... CHECKED AND LOOSE

[ ]  Mixture .... FULL AND FREE MOVEMENT, SET RICH

[ ]  Master Switch .... ON

[ ]  Circuit Breakers/Fuses .... IN/SECURE

[ ]  Fuel .... ON, TANK WITH LOWEST CONTENTS

[ ]  Fuel Pump .... ON, CHECK FUEL PRESSURE, OFF

[ ]  Primer .... PRIME AS REQUIRED AND LOCK

[ ]  Lookout .... GOOD LOOK ROUND, CALL 'CLEAR PROP'

[ ]  Magnetos .... KEYS IN, OPERATE STARTER

After Start

[ ]  RPM .... SET TO 1200

[ ]  Starter Warning Light .... OUT

[ ]  Oil Pressure .... RISING TO GREEN ARC WITHIN 30 SECS

[ ]  Ammeter .... CHARGING

[ ]  Suction .... REGISTERING

[ ]  Magnetos .... CHECK FOR DEAD CUT

[ ]  Instruments .... SET AS REQUIRED

[ ]  Radios .... TUNED AND CHECKED AS REQUIRED, TAXI CLEARANCE

Taxying

[ ]  Brakes .... CHECKED BEFORE TAXYING

[ ]  Rudder .... MOVEMENT AND STEERING CHECKED

[ ]  Instruments .... CHECK IN TURNS DI, COMPASS, TURN COORDINATOR, ATTITUDE INDICATOR

Power Checks

[ ]  Position .... INTO WIND, CLEAR ALL AROUND

[ ]  Parking Brake .... ON

[ ]  Fuel .... CHANGE TO FULLER TANK

[ ]  Oil Temp. and Press .... WITHIN LIMITS

[ ]  Magnetos .... CHECK

[ ]  RPM .... SET 2000 - BRAKES HOLDING

[ ]  Carb. Heat .... SET HOT, MAX DROP 100 RPM, SET COLD

[ ]  Magnetos .... CHECK LEFT & RIGHT, MAX DROP 175 RPM, MAX DIFF BTWN MAGS 50 RPM

[ ]  Suction .... 3" - 5"

[ ]  Ammeter .... CHARGING

[ ]  Oil temp. and Press .... WITHIN LIMITS

[ ]  RPM .... TO IDLE, 500 - 700 RPM, RESET 1200 RPM

Pre Take-Off Checks - Vital Actions

[ ]  T — Trimmer .... SET FOR TAKE OFF

[ ]  T — Throttle friction .... FINGER TIGHT

[ ]  M — Mixture .... RICH

[ ]  M — Magnetos .... ON BOTH, MASTER SWITCH ON

[ ]  P — Pitot heater .... AS REQUIRED

[ ]  P — Primer .... LOCKED

[ ]  F — Fuel .... ON TANK WITH HIGHEST CONTENTS, FUEL PUMP ON

[ ]  F — Flaps .... AS REQUIRED (SEE POH)

[ ]  I — Instruments .... CHECKED AND SET, GYROS, ALTIMETER, ENGINE T° & PRESS

[ ]  H — Hatches .... DOORS AND WINDOWS SECURE

[ ]  H — Harness .... SECURE

[ ]  C — Carb. Heat .... COLD

[ ]  C — Controls .... FULL AND FREE MOVEMENT

Take Off

[ ]  Lookout .... RUNWAY, APPROACH AND DEPARTURE PATHS VISUALLY CHECKED, ATC CLEARANCE

[ ]  RPM .... FULL POWER

[ ]  Engine .... TEMP & PRESS, STEADY WITHIN LIMIT

[ ]  Airspeed .... INCREASING

After Take-Off

[ ]  Flaps .... UP IF NECESSARY

[ ]  Engine .... TEMP / PRESS, STEADY WITHIN LIMIT

[ ]  Radios .... SET, ATC CLEARANCE AS NECESSARY

[ ]  Altimeter .... CHECK

[ ]  Fuel pump .... OFF ABOVE 1000' AGL

Pre Stall / Aerobatic Checks (H A S E L L)

[ ]  H — Height .... SUFFICIENT TO RECOVER BY 3000' AGL (IF SPINNING SEE POH)

[ ]  A — Airframe .... GYROS CAGED, BRAKES OFF, FLAPS AS REQUIRED (SEE POH)

[ ]  S — Security .... HATCHES AND HARNESSES TIGHT AND SECURE, NO LOOSE ARTICLES

[ ]  E — Engine .... TEMP / PRESS, CHECK FOR CARB ICE, MIXTURE RICH, FUEL PUMP ON

[ ]  L — Location .... CLEAR OF CLOUD, CONTROLLED AIRSPACE, BUILT UP AREAS AND AIRFIELDS

[ ]  L — Lookout .... 360° TURN TO CHECK FOR AIRCRAFT, ESPECIALLY BELOW

Continued Stall / Aerobatics (H E L L)

CONTINUED STALL / AEROBATICS (H E L L)

Cruise / Re-Join Checks (F R E D A)

[ ]  F — Fuel .... ON AND SUFFICIENT (INCLUDING GO-AROUND & DIVERSION), PUMP ON IF NECESSARY, TANKS EVEN

[ ]  R — Radio .... ON AND CORRECT FREQUENCY SET, JOINING INSTRUCTIONS IF REQUIRED

[ ]  E — Engine .... TEMP / PRESS, MIXTURE, CHECK FOR CARB ICE

[ ]  D — DI .... SYNCHRONIZED WITH COMPASS

[ ]  A — Altimeter .... QNH SET AS REQUIRED

Pre-Landing Checks

[ ]  B — Brakes .... OFF

[ ]  U — Undercarriage .... FIXED

[ ]  M — Mixture .... RICH

[ ]  F — Fuel .... ON FULLEST TANK, SUFFICIENT, FUEL PUMP ON

[ ]  F — Flaps .... AS REQUIRED

[ ]  P — Pitch .... FIXED

[ ]  I — Instruments .... ENGINE TEMP & PRESS CHECKED, ALTIMETER SET

[ ]  C — Carb. Heat .... CHECKED, RETURN COLD

[ ]  H — Hatches .... SECURE

[ ]  H — Harness .... SECURE AND FASTENED

Go-Around

[ ]  Throttle .... FULL POWER, CORRECT FOR YAW

[ ]  Carb. Heat .... COLD

[ ]  Flaps .... RETRACT IN STAGES AT SAFE HEIGHT

[ ]  Radio .... 'GO-AROUND' CALL, ATC INSTRUCTIONS

After Landing

[ ]  Vacate active runway and stop.

[ ]  Carb. Heat .... COLD (IF NECESSARY)

[ ]  Flaps .... UP

[ ]  Trimmer .... NEUTRAL

[ ]  Throttle Nut .... LOOSEN

[ ]  Fuel pump .... OFF

[ ]  Anti-collision lights .... OFF

[ ]  Electrics .... NON-ESSENTIAL OFF

[ ]  Radios .... NON-ESSENTIAL OFF

Shut Down

[ ]  Position .... INTO WIND, NOSEWHEEL STRAIGHT

[ ]  Parking brake .... ON

[ ]  RPM .... 1200 FOR 30 SECS

[ ]  Magnetos .... CHECK FOR DEAD CUT

[ ]  Radio .... OFF

[ ]  Throttle .... CLOSE

[ ]  Mixture .... IDLE CUT OFF (FULLY LEAN)

AFTER ENGINE STOPS

[ ]  Magnetos .... OFF, KEY OUT

[ ]  Electrics .... OFF

[ ]  Master Switch .... OFF

[ ]  Harness .... LEFT TIDY

[ ]  Hatches .... DOORS AND WINDOWS CLOSED

Fire On The Ground

[ ]  If taxying, stop clear of other aircraft, fuel trucks/stations, etc.

[ ]  Throttle .... CLOSED

[ ]  Mixture .... IDLE CUT OFF (FULLY LEAN)

[ ]  Fuel .... OFF, PUMP OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

[ ]  Brakes .... PARKING BRAKE ON

[ ]  Evacuate to a safe distance upwind, taking fire extinguisher.

Cabin Fire In The Air

[ ]  Master Switch .... OFF IF ELECTRICAL FIRE

[ ]  Electrical circuits .... OFF AS REQUIRED

[ ]  Fire extinguisher .... USE AS NECESSARY

[ ]  Forced landing procedure or diversion as applicable.

Engine Fire In The Air

[ ]  Throttle .... CLOSE

[ ]  Mixture .... IDLE CUT OFF (FULLY LEAN)

[ ]  Fuel .... OFF, FUEL PUMP OFF

[ ]  Magnetos .... OFF

[ ]  Cabin heater/Defrost .... OFF

NOTE: Forced landing (without power) procedure

WARNING: DO NOT ATTEMPT TO RESTART

Radio Failure

[ ]  Radio .... CHECK FREQUENCY, VOLUME, SQUELCH, CHECK AVIONICS SELECTOR/SWITCHES

[ ]  Headset .... CHECK PLUGS SECURE, CHANGE HEADSETS, TRY HAND MICROPHONE

[ ]  Electrics .... CHECK AMMETER, MASTER SWITCH, CIRCUIT BREAKERS - RESET ONCE ONLY

[ ]  Transponder .... SET 7600

[ ]  Speechless / Transmit blind / Non-radio procedure as appropriate.

Electrical Failure

[ ]  Electric load .... REDUCE (NON-ESSENTIAL ELECTRICS/RADIO ONLY)

[ ]  Field/Output Circuit Breaker .... CHECK / RESET

[ ]  Ammeter/Low voltage Warning light .... CHECK

[ ]  If NO OUTPUT — Reset Master Switch (off for 2 secs, then on) .... IF OUTPUT RESTORED — RESTORE ESSENTIAL ELECTRICS SINGLY

WARNING: IN THE EVENT OF REPEATED/CONTINUED ELECTRICAL FAILURE: select only essential electrical services, divert if applicable, note radio transmission makes a particularly heavy drain on the battery.

Engine Failure After Take-Off (EFATO)

[ ]  Lower nose to maintain best glide airspeed.

[ ]  Select landing area ahead.

[ ]  Use flap as necessary.

[ ]  Fuel .... OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

[ ]  Brief passengers/tighten harness/unlatch door as time permits.

WARNING: NEVER ATTEMPT TO TURN BACK

Engine Failure At Altitude (Forced Landing Without Power)

[ ]  Immediate actions.

[ ]  Attain and maintain best glide speed – TRIM.

[ ]  Assess surface wind.

[ ]  Select suitable landing area.

[ ]  Plan approach pattern.

[ ]  Check for cause of failure.

[ ]  Carb Heat .... ON

[ ]  Fuel .... CHANGE TANKS, CHECK SUFFICIENT, FUEL PUMP ON

[ ]  Mixture .... CHECK

[ ]  Primer .... LOCKED

[ ]  Magnetos .... ON BOTH

[ ]  Throttle .... CHECK

IF ENGINE DOES NOT RESTART, R/T MAYDAY CALL, COMMITTED CHECKS

[ ]  Fuel .... OFF

[ ]  Magnetos .... OFF

[ ]  Harness .... TIGHT

[ ]  Doors .... UNLATCHED

[ ]  Crew / passengers .... BRIEFED, AS TIME PERMITS

[ ]  Master Switch .... OFF

Ditching At Sea — Ditching Procedure (Without Power)

[ ]  Establish glide, head for coast line or any shipping area, check for causes of failure as time permits.

[ ]  Radio .... MAYDAY CALL

[ ]  Transponder .... 7700

[ ]  Harnesses .... TIGHT

[ ]  Doors .... UNLATCHED, DV WINDOW OPEN

[ ]  Crew / passengers .... BRIEFED

NOTE: Large swell / Light wind : land along swell, tail down stalled. Light swell / Strong wind : Landing into wind, tail down stalled.

[ ]  After ditching, use survival equipment.

WARNING: DO NOT INFLATE LIFE JACKETS IN CABIN

NOTE: Emergencies by their nature are not standard, and this checklist can only provide a guide to the appropriate actions - in an emergency: PILOT JUDGEMENT SHOULD DICTATE PILOT ACTIONS.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-38-112 Tomahawk/Joris_De_Naeyer_Piper_pa38_dewouw.txt

Rights: licensed

Checklist hash: a616c3328b1f20161c78fc0ceae3d117c719c0a75561672ae797eeb0d20b5326

Verify against the aircraft's own approved documentation before use.