Piper PA-44-180 Seminole — Ahmed_Nimer_Piper_44-180_simulator_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Taxi In And Engine Shut Down Checks" item count differs between passes (14 vs 16); verify nothing was dropped.

Warning

WARNING: THIS CHECK LIST IS FOR SIMULATION USE ONLY AND YOU CANT USE IT FOR REAL FLYING

Pre Start Checks

[ ]  Passneger Brief .... COMPLETED

[ ]  External Checks .... COMPLETED

[ ]  Door .... LATCH

[ ]  Belts, Shoulder Starps .... SECURE, CHECK

[ ]  Alternators .... ON

[ ]  Parking Brake .... ON

[ ]  Landing Gears .... DOWN

[ ]  Throttles .... CHECK/IDEL

[ ]  Propeller Controls .... CHECK/FULL FWD

[ ]  Mixtures .... CHECK/ICO

[ ]  Throttles Friction .... SET

[ ]  Carb Heat .... CHECK/OFF

[ ]  Cowl Flaps .... OPEN

[ ]  Trimmers .... NEUTRAL

[ ]  Fuel Selector .... ON

[ ]  Electrics .... OFF

[ ]  Circuit Breakers .... ALL IN

NOTE: YOU CAN NOT START THE ENGINES IF YOU DID NOT COMPLETE ALL OF THE PRE START PROCEDURES.

PRE START CHECK LIST COMPLETED

Starting Engines Checks

[ ]  Battery Master Switch .... ON

[ ]  Anti Collision Lights .... FIN

[ ]  3 Greens .... CHECK

[ ]  Throttles .... 1/4 IN OPEN

[ ]  Propellers .... FULL

[ ]  Mixtures .... RICH

[ ]  Fuel Pumps .... ON

[ ]  Primer .... OPERATE

[ ]  Propeller Area .... CLEAR

[ ]  Magnetos .... ON

[ ]  Starter .... ENGAGE

[ ]  Throttles .... 1000 RPM

[ ]  OIL PRESSURE .... CHCHK

[ ]  Alternators .... CHECK

[ ]  Suction .... CHECK

[ ]  Fuel Pumps .... OFF

[ ]  Fuel Pressure .... CHECK

NOTE: Repeat items From 7-17 For Each Engine In Turn.

NOTE: YOU CAN NOT TAXI IF YOU DID NOT COMPLETE ALL OF THE ENGINE START PROCEDURES.

ENGINE START CHECK LIST COMPLETED

Before Taxi Checks

[ ]  Radio Master & Radio Nav .... ON

[ ]  Heater & Fan .... SET

[ ]  Radio /Nav Eq .... SET/CHECK

[ ]  Transponder .... TEST/STBY

[ ]  Gryo Compass .... CHECK

[ ]  Flight Instruments .... CHECK

[ ]  Lights .... AS REQUIRED

[ ]  Taxi Clearance .... OBTAIN

[ ]  Altimeter .... SET QNH

NOTE: YOU CAN NOT TAXI IF YOU DID NOT COMPLETE ALL OF THE BEFORE TAXI PROCEDURES.

BEFORE TAXI CHECK LIST COMPLETED

Taxi Checks

[ ]  Area .... CLEAR

[ ]  Parking Brakes .... OFF

[ ]  Brakes .... CHECK

[ ]  Instruments .... CHECK

[ ]  Fuel Selectors .... X-FEED/ON

NOTE: TAXI TO THE RUNWAY MAX SPEED 20KT

NOTE: YOU CAN NOT TAXI IF YOU DID NOT COMPLETE ALL OF THE TAXI CHECKS PROCEDURES.

TAXI CHECK LIST COMPLETED

Ground Run Up Checks

[ ]  Parking Brakes .... ON

[ ]  Mixtures .... RICH

[ ]  Propellers .... FULL FWD

[ ]  Engine Instruments .... CHECK

[ ]  Throttles .... 1500 RPM

[ ]  Propellers .... FEATHER CHECK (max drop 500)

[ ]  Throttles .... 2000 RPM

[ ]  Magnetos .... CHECK (max drop 175, max dif 50)

[ ]  Propellers .... EXERCISE (max drop 300)

[ ]  Carb Heat .... CHECK/OFF

[ ]  Engine Instruments .... CHECK

[ ]  Warning Lights .... OUT

[ ]  Alternators .... CHECK

[ ]  Suction .... CHECK (4.8to5.2 In Hg)

[ ]  Throttles .... IDEL/Check(500to600RPM)

[ ]  Throttles .... 1000 RPM

[ ]  Throttles Friction .... SET

NOTE: YOU CAN NOT TAKE OFF IF YOU DID NOT COMPLETE ALL OF THE GROUND RUN UP PROCEDURES.

GROUND RUN UP CHECKS COMPLETED.

Before T/OFF Checks

[ ]  Fuel Contents .... CHECK

[ ]  Fuel Pumps .... ON

[ ]  Fuel Selectors .... ON

[ ]  Mixtures .... RICH

[ ]  Engine Instruments .... CHECK

[ ]  Warning Lights .... CHECK/TEST

[ ]  Flight Instruments .... CHECK

[ ]  Flight Controls .... CHECK/FREE MOVMENT

[ ]  Trimmers .... SET FOR T/OFF

[ ]  Carb Heat .... OFF

[ ]  Cowl Flaps .... OPEN

[ ]  Flaps .... CHECK/SET 1 NUTCH FOR T/OFF

[ ]  Landing Lights .... ON

[ ]  Navigation Lights .... ON

[ ]  Recognition Lights .... ON

[ ]  Transponder .... SET/ON

[ ]  Departure Clearance .... OBTAINED

[ ]  Rad Nav Eq .... SET FOR DEPARTURE

[ ]  Emergency Brief .... COMPLTETED

NOTE: When you Cleared Into runway

[ ]  Pitot heater .... AS REQUIRED

[ ]  Ventilation FAN .... OFF

NOTE: YOU CAN NOT TAKE OFF IF YOU DID NOT COMPLETE ALL OF THE BEFOR TAKE OFF PROCEDURES.

BEFORE TAKE OFF CHECKS COMPLETED.

Take Off Procedures Checks

[ ]  Throttles .... FULL

[ ]  Air Speed Indicator .... INCREASE

[ ]  Engine Instruments .... ALL IN GREEN

[ ]  Flight Instruments .... NORMAL INDECATION

[ ]  Air Speed 90KT .... ROTATE

[ ]  Brakes .... ON/OF

[ ]  Positive Rate Of Climb .... GEARS UP

NOTE: DO NOT TAKE OFF IF YOU DID NOT COMPLETE ALL OF THE TAKE OFF PROCEDURES.

TAKE OFF PROCEDURES COMPLETED.

After T/OFF And Climb Out Checks

[ ]  AT 500 AGL .... FLAPS UP

[ ]  AT 700 AGL .... POWER SET 45"MAP&24RPM

[ ]  Engine Instruments .... CHECK

[ ]  Landing Lights .... OFF

[ ]  Fuel Pumps .... OFF (at 1000AGL Or At TOC)

[ ]  Radio Nav Eq .... SET

[ ]  After 5000 FEET .... APPLY FULL POWER

NOTE: YOU MUST COMPLETE ALL OF THE CLIMB OUT CHECK BEFORE YOU REACH THE TOP OF CLIMB.

AFTER TAKE OFF AND CLIMB OUT CHECKS COMPLETED.

Cruise Checks

[ ]  Power .... SET

[ ]  Mixtures .... SET

[ ]  Fuel Pumps .... OFF

[ ]  Cowl Flaps .... CLOSED

[ ]  Engine Instruments .... CHECK/ALL IN GREEN

[ ]  Alternators .... CHECK

[ ]  Suction .... CHECK

[ ]  Flight Instruments .... CHECK

NOTE: YOU CANNT CONTINUE YOUR FLIGHT IF YOU DID NOT COMPLETE ALL OF THE CRUISE PROCEDURES.

CRUISE CHECKS COMPLETED

Descent Checks

[ ]  Fuel Contents .... CHECK

[ ]  Fuel Pumps .... ON

[ ]  Fuel Selectors .... ON

[ ]  Mixtures .... SET/RICH

[ ]  Engine Instruments .... NORMAL INDICATION

[ ]  Alternators .... CHECK

[ ]  Suction .... CHECK

[ ]  Flight Instruments .... CHECK/NORMAL

[ ]  Rad Nave Eq .... SET/CHECK

[ ]  Altimeter .... SET/CHECK

[ ]  Briefing .... COMPLETED

NOTE: If the OAT Above 10 C Open the Cowl Flaps

NOTE: YOU CAN NOT LAND IF YOU DID NOT COMPLETE ALL OF THE DESCENT PROCEDURES.

DESCENT CHECKS COMPLETED

Approach Checks

[ ]  Mixtures .... RICH

[ ]  Propeller .... SET

[ ]  Altimeter .... QNH/SET

[ ]  Transponder .... SET

[ ]  Landing Lights .... ON

[ ]  Cowl Flaps .... OPEN

NOTE: DO NOT LAND IF YOU DID NOT COMPLETE ALL OF THE APPROACH PROCEDURES.

APPROACH CHECKS COMPLETED.

Landing Checks

[ ]  Landing Gears .... Air speed WHITE ARC/DOWN

[ ]  3 Greens .... CHECK

[ ]  Flaps .... WHITE ARC/SET 2 NUTCH

[ ]  Mixtures .... RICH

[ ]  Propellers .... FULL

FINAL CHECKS

[ ]  Mixture .... RICH

[ ]  Propeller .... FULL

[ ]  3 Greens .... CHECK

[ ]  Landing Speed .... 75-80KT

[ ]  Touch Down .... MAINTAIN R/W CENTER LINE

[ ]  Breakes .... APPLY SLOWLY

APPROACH AND LANDING CHECKS COMPLETED

After Landing Checks

[ ]  Flaps .... UP

[ ]  Fuel Pumps .... OFF

[ ]  Carb Heart .... OFF

[ ]  Cowl Flaps .... OPEN

[ ]  Lights .... AS REQUIRED

[ ]  Pitot Hearter .... OFF

[ ]  Radio NAV Eq .... OFF (except the one in use)

AFTER LANDING CHECKS COMPLETED.

Taxi In And Engine Shut Down Checks

[ ]  Taxi In Clearance .... OBTAINED

[ ]  Lights .... AS REQUIRE

ENGINE SHUTDOWN CHECKS

[ ]  Parking Breaks .... ON

[ ]  Lights .... AS REQUIRED

[ ]  Throttles .... IDEL

[ ]  Mixtures .... SET/IDEL

[ ]  RADIO&NAV Eq&GPS .... OFF

[ ]  Alternators .... OFF

[ ]  Magnetos .... OFF

[ ]  Battery Master Switch .... OFF

[ ]  Lights .... OFF

[ ]  All Switches .... OFF

ENGINE SHUT DOWN CHECKS COMPLETED.

Closing Warning

WARNING: DO NOT USE THIS CHECK LIST FOR REAL FLYING BECAUSE ITS FOR SIMULATOR ONLY.

NOTE: Made By: Ahmed Nimer Jordan.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-44-180 Seminole/Ahmed_Nimer_Piper_44-180_simulator_checklist.txt

Rights: licensed

Checklist hash: 3d3b30c2b846136a0584049d07a3e27cf14a5b17af9595ed76b6469c657132c7

Verify against the aircraft's own approved documentation before use.