Piper PA-44-180 Seminole — jochman-clsemnle

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Engine Failure  [EMERGENCY]

[ ]  Minimum Control Speed .... 56 KIAS

[ ]  Blue Line Speed .... 88 KIAS

[ ]  Mixture, Prop, Throttle .... FORWARD

[ ]  Flaps & Gear .... RETRACT

[ ]  Electric Fuel Pumps .... ON

[ ]  Identify .... DEAD FOOT = DEAD ENGINE

[ ]  Verify .... RETARD THROTTLE

[ ]  Propeller of Dead Engine .... FEATHER

[ ]  Mixture of Dead Engine .... IDLE CUT OFF

[ ]  Trim .... AS REQUIRED

[ ]  Bank .... 5º TO GOOD ENGINE

[ ]  Fuel Pump of Dead Engine .... OFF

[ ]  Magnetos of Dead Engine .... OFF

[ ]  Cowl Flaps of Dead Engine .... CLOSE

[ ]  Cowl Flaps of Good Engine .... AS NEEDED

[ ]  Alternator of Dead Engine .... OFF

[ ]  Electrical Load .... REDUCE

[ ]  Fuel to Dead Engine .... OFF

[ ]  Fuel to Good Engine .... ON or CROSSFEED

[ ]  Fuel Pump of Good Engine .... OFF

Restart Engine  [EMERGENCY]

[ ]  Mixture .... AS REQUIRED

[ ]  Fuel Boost Pump .... ON

[ ]  Fuel Selector .... CROSSFEED

[ ]  Magnetos .... SELECT L or R ONLY

[ ]  Carburetor Heat .... ON

Unfeathering Procedure  [EMERGENCY]

[ ]  Fuel Selector Dead Engine .... ON

[ ]  Fuel Pump on Dead Engine .... ON

[ ]  Prop .... FORWARD

[ ]  Throttle .... 2 STROKES, OPEN ¼ INCH

[ ]  Mixture .... RICH

[ ]  Magneto Switches .... ON

[ ]  Starter .... ENGAGE, PROP WINDMILLS

[ ]  Throttle .... RETARD, ENGINE WARM

[ ]  If engine does not start

[ ]  Prime .... AS REQUIRED

[ ]  Repeat Steps .... STEPS 6, 7, 8

[ ]  Alternator .... ON

Before Engine Start

[ ]  Seats .... ADJUSTED

[ ]  Seat Belts, Harness .... FASTENED

[ ]  Parking Brake .... SET

[ ]  Circuit Breakers .... IN

[ ]  Radios .... OFF

[ ]  Cowl Flaps .... OPEN

[ ]  Carburetor Heat .... OFF

[ ]  Alternators .... ON

[ ]  Propeller Sync. .... MANUAL

Starting Engines

[ ]  Fuel Selector .... ON

[ ]  Mixture Controls .... RICH

[ ]  Throttle Controls .... OPEN ½ INCH

[ ]  Propeller Controls .... FORWARD

[ ]  Master Switch .... ON

[ ]  Magneto Switchs .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Prime .... AS REQUIRED

[ ]  Propeller Area .... CLEAR

[ ]  Starter .... ENGAGE

[ ]  Throttle .... ADJUST

[ ]  Oil Pressure .... CHECK

[ ]  Repeat Steps 8-12 for Other Engine .... COMPLETE

[ ]  Electric Fuel Pumps .... OFF

Hot Engine Start

[ ]  Throttle Controls .... OPEN ½ INCH

[ ]  Propeller Controls .... FORWARD

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture Controls .... RICH

[ ]  Magneto Switch .... ON

[ ]  Propeller Area .... CLEAR

[ ]  Starter .... ENGAGE

[ ]  Throttle .... ADJUST

[ ]  Oil Pressure .... CHECK

[ ]  Repeat Steps 7-10 for Other Engine .... COMPLETE

Flooded Engine Start

[ ]  Mixture Controls .... IDLE CUT-OFF

[ ]  Throttle Controls .... FULL FORWARD

[ ]  Propeller Controls .... FORWARD

[ ]  Master Switch .... ON

[ ]  Magneto Switch .... ON

[ ]  Electric Fuel Pump .... OFF

[ ]  Propeller Area .... CLEAR

[ ]  Starter .... ENGAGE

NOTE: When engine fires, retard throttle and advance mixture slowly.

[ ]  Oil Pressure .... CHECK

Pre-Takeoff Check

[ ]  Parking Brake .... ON

ENGINE RUN-UP

[ ]  Mixture Controls .... FORWARD

[ ]  Propeller Controls .... FORWARD

[ ]  Propeller Sync. .... MANUAL

[ ]  Throttle Controls .... 1500 RPM

[ ]  Propeller Controls .... CHECK FEATHER

[ ]  Throttle Controls .... 2000 RPM

[ ]  Propeller Governor .... CHECK

[ ]  Propeller Controls .... EXERCISE

[ ]  Propeller Controls .... FULL FORWARD

[ ]  Carburetor Heat .... CHECK

[ ]  Engine Gauges .... GREEN

[ ]  Vacuum Gauge .... 4.8 to 5.2

[ ]  Magnetos .... CHECK

[ ]  Alternators .... CHECK

[ ]  Throttles .... 800-1000 RPM

[ ]  Fuel .... ON

[ ]  Alternators .... ON

[ ]  Altimeter .... SET

[ ]  Attitude Indicator .... SET

[ ]  Directional Gyro .... SET

[ ]  Clock .... WOUND & SET

[ ]  Mixtures .... SET

Pre-Takeoff Check - Continued

[ ]  Propellers .... FORWARD

[ ]  Friction Lock .... ADJUST

[ ]  Carburetor Heat .... OFF

[ ]  Cowl Flaps .... SET

[ ]  Seat Backs .... ERECT

[ ]  Wing Flaps .... SET

[ ]  Trim (Stabilator & Rudder) .... SET

[ ]  Seat Belts, Harnesses .... FASTENED

[ ]  Empty Seat Belts .... FASTENED

[ ]  Controls .... FREE & CORRECT

[ ]  Doors .... LATCHED

[ ]  Electric Fuel Pumps .... ON

[ ]  Pitot Heat .... AS REQUIRED

[ ]  Transponder .... ON

[ ]  Lights .... AS REQUIRED

Takeoff and Landing Speeds

NORMAL T/O:

NOTE: 0º Flaps, VR=75 KIAS

NOTE: then VY =88 KIAS, Gear UP

SHORT FIELD T/O, NO OBSTACLE:

NOTE: 0º Flaps, Trim Set, Brakes ON

NOTE: Full Power before Brake Release, VR=70 KIAS

NOTE: 75 KIAS to 50’, then VY =88 KIAS. Gear UP

SHORT FIELD T/O, OBSTACLE CLEARANCE:

NOTE: 25º Flaps, Trim Set, Brakes ON

NOTE: Full Power before Brake Release, VR=63 KIAS

NOTE: 67 KIAS to 50’, Gear UP

LANDING SPEEDS:

NOTE: Traffic Pattern   100 KIAS

NOTE: Final        90 KIAS

NOTE: Lightly Loaded    80 KIAS

NOTE: Short Field       75 KIAS

Pre-Landing Check

[ ]  Seat Backs .... ERECT

[ ]  Seat Belts, Harnesses .... FASTENED

[ ]  Fuel Selectors .... ON

[ ]  Cowl Flaps .... AS REQUIRED

[ ]  Electric Fuel Pumps .... ON

[ ]  Mixture Controls .... RICH

[ ]  Propellers .... 2500 RPM

[ ]  Propeller Sync. .... MANUAL

[ ]  Landing Gear .... DOWN

[ ]  Flaps .... AS REQUIRED

Post Landing Check

[ ]  Wing Flaps .... RETRACT

[ ]  Cowl Flaps .... OPEN

[ ]  Electric Fuel Pumps .... OFF

[ ]  Carburetor Heat .... OFF

Shut Down

[ ]  Throttle .... AFT

[ ]  Radios and Electric Equipment .... OFF

[ ]  Mixture Controls .... IDLE CUT-OFF

[ ]  Magneto Switches .... OFF

[ ]  Alternators .... OFF

[ ]  Master Switch .... OFF

[ ]  Parking Brake .... AS REQUIRED

Aircraft Performance Speeds

NOTE: VS0 – Landing configuration stall at gross weight, power off    55 KIAS

NOTE: VS1 – Cruise configuration stall at gross weight, power off     57 KIAS

NOTE: 1.3 VS0 – Fence Speed    75 KIAS

NOTE: 1.5 VS0 – Approach Speed      84 KIAS

NOTE: VX – Best Angle of Climb Speed      82 KIAS

NOTE: VY – Best Rate of Climb Speed       88 KIAS

NOTE: Normal Cruise      24 Hg 2400 RPM

NOTE: Slow Cruise   23 Hg 2300 RPM

NOTE: VFE – Max. Flap Extension     111 KIAS

NOTE: VLE – Max. Gear Extension     140 KIAS

NOTE: VLR – Max. Gear Retraction    109 KIAS

NOTE: VA – Maneuvering Speed   135 KIAS

NOTE: VNO – Top of Green Arc   169 KIAS

NOTE: VNE – Red Line – Never Exceed       202 KIAS

NOTE: VMC – Minimum Control Speed with Critical Engine Inoperative (Red Radial)  56 KIAS

NOTE: VXSE – Best Angle of Climb Speed, One Engine Inoperative   82 KIAS

NOTE: VYSE – Best Rate of Climb Speed, One Engine Inoperative    88 KIAS

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-44-180 Seminole/jochman-clsemnle.txt

Rights: licensed

Checklist hash: eae3815072e08e52148ee059cbaf173931cde078aa8d787684c103c2fa7e3da5

Verify against the aircraft's own approved documentation before use.