Piper PA-44-180 Seminole — piper_seminole_procedures_mwaugh

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Takeoff

NORMAL TAKEOFF – FLAPS 0°

[ ]  Power 2000 RPM – Check Engine Gauges .... CHECK

[ ]  Rotate VR 75K – Takeoff VLOF 80K .... ROTATE

[ ]  Climb BLUELINE – 88K .... CLIMB

[ ]  No Runway – Positive Rate – GEAR UP .... GEAR UP

[ ]  500' AGL – 100K .... 100K

[ ]  1000' AGL – 24" / 2500 RPM – Takeoff Checklist .... TAKEOFF CHECKLIST

SHORT FIELD TAKEOFF – FLAPS 0°

[ ]  Hold Brakes .... HOLD

[ ]  Power 2000 RPM – Check Engine Gauges .... CHECK

[ ]  Release Brakes .... RELEASE

[ ]  Rotate VR 70K .... ROTATE

[ ]  Climb 75K to 50' .... CLIMB

[ ]  Climb BLUELINE – 88K .... CLIMB

[ ]  No Runway – Positive Rate – GEAR UP .... GEAR UP

[ ]  500' AGL – 100K .... 100K

[ ]  1000' AGL – 24" / 2500 RPM – Takeoff Checklist .... TAKEOFF CHECKLIST

Approach

NORMAL APPROACH (1 OR 2 ENGINE)

[ ]  Approach Checklist .... COMPLETE

[ ]  Downwind 100K (17") .... 100K

[ ]  GEAR DOWN / FLAPS 25° .... GEAR DOWN / FLAPS 25°

[ ]  Descend BLUELINE – 88K (15") .... DESCEND

[ ]  “GEAR DOWN – CHECKLIST” .... GEAR DOWN – CHECKLIST

[ ]  Base – “BLUELINE GUMP” .... BLUELINE GUMP

[ ]  400' AGL – “GEAR DOWN STABILIZED” .... GEAR DOWN STABILIZED

[ ]  Power Slow RETARD over RUNWAY END .... RETARD

[ ]  Touchdown 75K .... TOUCHDOWN

SHORT FIELD APPROACH

[ ]  Approach Checklist .... COMPLETE

[ ]  Downwind 100K (17") .... 100K

[ ]  GEAR DOWN / FLAPS 25° .... GEAR DOWN / FLAPS 25°

[ ]  Descend BLUELINE – 88K (15") .... DESCEND

[ ]  “GEAR DOWN – CHECKLIST” .... GEAR DOWN – CHECKLIST

[ ]  Base – “BLUELINE GUMP” .... BLUELINE GUMP

[ ]  600' AGL / FLAPS 40 / SPEED 75K .... FLAPS 40 / 75K

[ ]  400' AGL – “GEAR DOWN STABILIZED” .... GEAR DOWN STABILIZED

[ ]  Power Slow RETARD in FLARE .... RETARD

[ ]  Touchdown – FLAPS UP / MAXIMUM BRAKING .... TOUCHDOWN

Instrument Approaches

ILS APPROACH (1 OR 2 ENGINE)

[ ]  Approach Checklist .... COMPLETE

[ ]  100K to Intercept (17") .... 100K

[ ]  G/S Int. – GEAR DOWN / FLAPS 25° .... GEAR DOWN / FLAPS 25°

[ ]  G/S Intercept – “NO FLAGS” .... NO FLAGS

[ ]  G/S Descent – BLUELINE - 88K (2E - 15" / 1E - 20") .... BLUELINE 88K

[ ]  “GEAR DOWN – CHECKLIST” .... GEAR DOWN – CHECKLIST

[ ]  1000' AGL “BLUELINE GUMP” .... BLUELINE GUMP

[ ]  400' AGL “GEAR DOWN – STABILIZED” .... GEAR DOWN – STABILIZED

[ ]  100' ABOVE MINS “100 TO GO” .... 100 TO GO

[ ]  Power Slow RETARD over RUNWAY END .... RETARD

[ ]  Touchdown 75K .... TOUCHDOWN

NON-PRECISION APPROACH (1 OR 2 ENGINE)

[ ]  Approach Checklist .... COMPLETE

[ ]  100K Procedure Turn (17") .... 100K

[ ]  2E – 2Miles FAF GEAR DOWN / FLAPS 25° .... GEAR DOWN / FLAPS 25°

[ ]  1E – FAF GEAR DOWN / FLAPS 0° .... GEAR DOWN / FLAPS 0°

[ ]  FAF – “NO FLAGS” .... NO FLAGS

[ ]  FAF Descent – 88K (2E - 13" / 1E - 15") .... 88K

[ ]  “GEAR DOWN – CHECKLIST” .... GEAR DOWN – CHECKLIST

[ ]  1000' AGL “BLUELINE GUMP” .... BLUELINE GUMP

[ ]  100' ABOVE MINS “100 TO GO” .... 100 TO GO

[ ]  MDA – Level (2E - 20" / 1E – FULL) .... LEVEL

[ ]  1E - Leaving MDA – FLAPS 25° .... FLAPS 25°

[ ]  400' AGL “GEAR DOWN – STABILIZED” .... GEAR DOWN – STABILIZED

[ ]  Power Slow RETARD over RUNWAY END .... RETARD

[ ]  Touchdown 75K .... TOUCHDOWN

CIRCLE TO LAND (2 ENGINE ONLY)

[ ]  FAF – FLAPS 0° .... FLAPS 0°

[ ]  Turning Final, Leaving MDA – FLAPS 25° .... FLAPS 25°

Missed Approach

[ ]  Full Power – PITCH 88K .... FULL POWER / PITCH 88K

[ ]  Slow FLAPS 0° .... FLAPS 0°

[ ]  Positive Rate – GEAR UP .... GEAR UP

[ ]  500' AGL – 100K .... 100K

[ ]  1000' AGL - 24" / 2500 RPM – Takeoff Checklist .... TAKEOFF CHECKLIST

[ ]  HOLDING SPEED 100K .... 100K

Steep Turns

[ ]  SPEED 120K - 20" / 2300 RPM .... 120K

[ ]  50° BANK – POWER 23" - TRIM .... 23" / TRIM

VMC Demo

[ ]  Clearing Turns – 100K (17") .... 100K

[ ]  LEFT Throttle – IDLE .... IDLE

[ ]  RIGHT Throttle – FULL POWER .... FULL POWER

[ ]  5° BANK .... 5° BANK

[ ]  PITCH 1° / SEC – LIMIT RUDDER @ 77K .... PITCH 1°/SEC / LIMIT RUDDER

[ ]  LOSS OF DIRECTIONAL CONTROL or STALL WARNING .... RECOGNIZE

[ ]  PITCH DOWN – POWER TO IDLE .... PITCH DOWN / IDLE

[ ]  Slow POWER to FULL – 82K and then 88K .... FULL POWER / 82K / 88K

Stalls and Slow Flight

SLOW FLIGHT

[ ]  Clearing Turns – 100K (17") .... 100K

[ ]  GEAR DOWN / FLAPS 40° .... GEAR DOWN / FLAPS 40°

[ ]  GEAR DOWN Checklist .... COMPLETE

[ ]  LEVEL FLIGHT – 70K .... 70K

[ ]  CRUISE POWER .... SET

[ ]  FLAPS 0° / SPEED 82K .... FLAPS 0° / 82K

[ ]  Positive Rate – GEAR UP .... GEAR UP

[ ]  SPEED 88K .... 88K

STALL – POWER OFF

[ ]  Clearing Turns – 100K (17") .... 100K

[ ]  GEAR DOWN / FLAPS 40° .... GEAR DOWN / FLAPS 40°

[ ]  GEAR DOWN Checklist .... COMPLETE

[ ]  DESCENT – 80K .... 80K

[ ]  POWER to IDLE – Pitch to BUFFET .... IDLE / PITCH TO BUFFET

[ ]  PITCH DOWN – FULL POWER .... PITCH DOWN / FULL POWER

[ ]  FLAPS 0° / SPEED 82K .... FLAPS 0° / 82K

[ ]  Positive Rate – GEAR UP .... GEAR UP

[ ]  SPEED 88K .... 88K

STALL – POWER ON

[ ]  Clearing Turns – 100K (17") .... 100K

[ ]  POWER 13" / SPEED 80K .... 13" / 80K

[ ]  POWER 18" / Pitch to BUFFET .... 18" / PITCH TO BUFFET

[ ]  PITCH DOWN – FULL POWER .... PITCH DOWN / FULL POWER

[ ]  SPEED 82K .... 82K

[ ]  SPEED 88K .... 88K

Configuration Checklist

[ ]  Fuel Selectors – ON .... ON

[ ]  Cowl Flaps – OPEN .... OPEN

[ ]  Mixtures – RICH .... RICH

[ ]  Prop – HIGH .... HIGH

[ ]  Fuel Pumps - ON .... ON

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-44-180 Seminole/piper_seminole_procedures_mwaugh.txt

Rights: licensed

Checklist hash: c8393fdd41ca0d6af31c0ae7e96b3cbebac1f8b3c6f93b5bbafb5b3f3ee9419a

Verify against the aircraft's own approved documentation before use.