Piper PA-46-350P Malibu Mirage — Pete_Sandhu_Piper_PA46-350P_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

1 BEFORE STARTING ENGINE

[ ]  Preflight .... COMPLETE

[ ]  Paper & Documents .... ABOARD

[ ]  Chocks & Tow bar .... REMOVED

[ ]  Seat & Seats Belts .... SECURE

[ ]  Window Shades .... OPEN

[ ]  Circuit Breakers .... CHECK IN

[ ]  Induction Air .... CHECK then PRIMARY

[ ]  Cabin Altitude Selector .... SET

[ ]  Altitude Rate Control .... 9 O'clock Position

[ ]  Landing Gear Selector .... DOWN

[ ]  Cabin Pressurization Control .... CAGED

[ ]  Cabin Pressure Dump Switch .... NORMAL

[ ]  Avionics Radio Master .... OFF

[ ]  Ice Protection Equipment .... OFF

[ ]  Propeller .... FULL INCREASE

[ ]  Fuel Selector .... Desired Tank for Takeoff

[ ]  Mixture .... IDLE CUTOFF

[ ]  Hobbs .... RECORD

2 STARTING ENGINE

[ ]  Brakes .... SET

[ ]  Emergency Fuel Pump .... OFF

[ ]  Check Area .... CLEAR

[ ]  Battery Master (ExternalPower = off) .... ON

[ ]  Alternators (ExternalPower = off) .... ON

[ ]  Magnetos .... ON

[ ]  wait for Engine Instruments

[ ]  Fuel Computer .... SET

[ ]  Throttle .... HALF

[ ]  Mixture .... RICH (4 secs) then IDLE CUTOFF

NOTE: (if engine is hot, no prime required)

[ ]  Starter .... ENGAGE

[ ]  Mixture

[ ]  Oil Pressure (within 30 sec) .... GREEN

3 BEFORE TAXIING

[ ]  Alternators .... CHECK

[ ]  Throttle .... 1000-1200

[ ]  Mixture .... Lean for Taxi

[ ]  Fuel Boost Pump .... on for High Elevations

[ ]  External Lights .... as REQUIRED

[ ]  Aux. Fuel Pump .... ON

[ ]  Avionics Radio Master .... ON

[ ]  EFIS .... ON

[ ]  Wing Flaps .... CHECK

[ ]  Spoiler .... CHECK

[ ]  Annunciator Panel .... TEST

[ ]  ATIS .... OBTAIN

[ ]  Altimeters (3) .... SET

[ ]  Heading Bug .... Set to Departure Runway

[ ]  Environmental System .... As Desired

4 TAXIING

[ ]  Brakes .... CHECK

[ ]  Turning Instruments .... CHECK

5 BEFORE TAKEOFF & RUN–UP

[ ]  Flight Plan .... ENTER

[ ]  Flight Instruments .... SET

[ ]  Doors .... CLOSED (4 GREEN)

[ ]  Seats .... ADJUSTED AND LOCKED

[ ]  Armrests &Table .... STOWED

[ ]  Pressurization .... SET

[ ]  Flight Controls .... CHECK

[ ]  Electric Trim .... TEST & SET

[ ]  Rudder Trim .... SET

[ ]  Flaps .... 10 degrees (Normal Takeoff)

[ ]  Minimum Oil Temp for Run–Up .... 100°F

ENGINE RUN–UP

[ ]  Mixture .... RICH

[ ]  Throttle .... 1700 RPM

[ ]  Magentos (ea.max 150, diff.max 50) .... CHECK

[ ]  Propeller .... CHECK

[ ]  Engine Instruments .... GREEN

[ ]  Vacuum System (4.5“ to 5.2“) .... GREEN

[ ]  Throttle .... 1200 RPM

[ ]  Alternators (2) .... CHECK

[ ]  De–Ice / Anti–Ice .... CHECK

[ ]  Throttle .... 1000 RPM

[ ]  Autopilot .... TEST & OFF

[ ]  Annunciator Panel .... CHECK

[ ]  Stall Warning Horn .... TEST

[ ]  Manifold & Static Line .... DRAIN

6 CLEARED FOR TAKEOFF

[ ]  Emergency Fuel Pump .... ON

[ ]  Lights .... SET

[ ]  Ice Protection .... SET

[ ]  Air Conditioning .... OFF

7 TAKEOFF

[ ]  Heading Bug

[ ]  Throttle .... Slowly Apply Full Power (3 secs)

[ ]  Rotate .... 80 KIAS

[ ]  Best Angle –Vx .... 90 KIAS

[ ]  Best Rate –Vy .... 110 KIAS

[ ]  Positive Rate of Climb .... GEAR UP

NOTE: (Check Annunciator for Warnings)

AT SAFE ALTITUDE

[ ]  Flaps .... RETRACT

[ ]  Fuel Pump .... OFF

8 CRUISE CLIMB

[ ]  Manifold Pressure .... 35“

[ ]  Propeller Control .... 2400 RPM

[ ]  Mixture Control .... 32 GPH

[ ]  Airspeed .... 125 KIAS

[ ]  Pressurization / Altimeter .... SET

9 CRUISE

[ ]  Mixture .... Lean to 1550 TIT

[ ]  High Speed .... 30”/2400 RPM

[ ]  Economy .... 25”/2000 RPM

[ ]  Fuel Imbalance .... Max 10 Gallons

10 DESCENT

[ ]  Power slowly reduce 15 min before arrival

[ ]  Mixture .... CRUISE SETTING

[ ]  Propeller Control .... CRUISE SETTING

[ ]  Pressurization / Altimeter .... SET

11 APPROACH & BEFORE LANDING

[ ]  Seat & Seats Belts .... SET

[ ]  Fuel Selector .... PROPER TANK

[ ]  Landing Gear down locked .... 3 GREEN

[ ]  Mixture .... RICH

[ ]  Propeller .... FULL

[ ]  Aux. Fuel Pump .... ON

[ ]  Pressurization .... CHECK

[ ]  Ice Protection .... CHECK

[ ]  Toe Brake .... PUMP & TEST then lower feet

[ ]  Induction Air .... PRIMARY

[ ]  Airspeed .... 85 KIAS

12 AFTER LANDING & PARKING

[ ]  Ice Protection .... OFF

[ ]  Flaps / Spoiler .... RETRACT

[ ]  EFIS & Avionics Radio Master .... OFF

[ ]  Engine cool down .... TIT STABLE

[ ]  Mixture .... IDLE CUTOFF

[ ]  Magentos / Alternators .... OFF

[ ]  Battery Master .... OFF

[ ]  HOBBS .... RECORD

1 LIMITATIONS & SPEEDS KIAS

[ ]  V1 (decision speed at half TODA) .... 55

[ ]  Vde-ice .... 183

[ ]  VA (light - heavy) .... 103 - 135

[ ]  Vglide .... 90

NOTE: Angle of Bank ⌠ Vs (Vso): 10°⌠70 (60)   30°⌠76 (66)   45°⌠82 (75)   60°⌠98 (86)

[ ]  Basic Empty Weight .... 2997lbs

[ ]  Max Takeoff Weight .... 4300lbs

[ ]  Zero Fuel Weight .... 4100lbs

[ ]  Max Landing Weight .... 4100lbs

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-46-350P Malibu Mirage/Pete_Sandhu_Piper_PA46-350P_checklist.txt

Rights: licensed

Checklist hash: dcbcd3c6ae408af0ab4353725cf54d4906d084cde4a439f80ab25c1df516df63

Verify against the aircraft's own approved documentation before use.