Piper PA-46 Malibu — Daniel_Draxler_PA46_cheklist1b

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Before Starting Engine

[ ]  Preflight .... COMPLETE

[ ]  Paper & Documents .... ABOARD

[ ]  Control Locks & Tow bar .... REMOVED

[ ]  Seat & Seats Belts .... SET

[ ]  Circiut Breakers .... CHECK

[ ]  Induction / Alternate Air .... CHECK

[ ]  Landing Gear Selector .... DOWN

[ ]  Avionics Radio Master .... OFF

[ ]  Mixture, Prop, Throttle .... SET

Starting Engine

[ ]  Brakes .... SET

[ ]  Battery Master (ExternalPower = off) .... ON

[ ]  Alternators (ExternalPower = off) .... ON

[ ]  Magnetos .... ON

[ ]  Prime, Fuel Pump LH/RH check .... 6 SEC

[ ]  Starter .... ENGAGE

[ ]  Oil Pressure within 30 sec .... GREEN

Before Taxiing & Taxi Check

[ ]  Mixture .... SET

[ ]  Propeller .... SET

[ ]  Induction / Alternate Air .... PRIMARY

[ ]  Avionics Radio Master .... ON & SET

[ ]  Wing Flaps .... CHECK

[ ]  Spoiler .... CHECK

[ ]  Brakes .... CHECK

[ ]  Turning Instruments .... CHECK

[ ]  Annunciator Panel .... TEST

Before Takeoff & Run–Up

[ ]  Flight Instruments, Clocks .... SET

[ ]  Autopilot .... TEST & OFF

[ ]  Doors & Windows .... CLOSED

[ ]  Pressurization .... SET

[ ]  Flight Contorls .... CHECK

[ ]  Trim Tabs .... CHECK & SET

[ ]  Minimum Oil Temp for Run–Up .... 100°F

ENGINE RUN–UP

[ ]  Throttle .... 1700 RPM

[ ]  Magetos (ea.max 150, diff.max 50) .... CHECK

[ ]  Propeller .... CHECK

[ ]  Engine Instruments .... GREEN

[ ]  Vacuum System (4.5" to 5.2") .... GREEN

[ ]  Throttle .... 1200 RPM

[ ]  Alternators .... CHECK

[ ]  De–Ice / Anti–Ice .... CHECK

[ ]  Throttle .... 1000 RPM

[ ]  Manifold & Static Line .... DRAIN

[ ]  Aux. Fuel Pump .... AS REQUIRED

[ ]  Lights .... SET

Takeoff

[ ]  Fuel Selector .... PROPER TANK

[ ]  Flaps .... SET

[ ]  Ice Protection .... SET

[ ]  Gyro / Txpdr / Altimeter .... CHECK & SET

[ ]  Throttle .... FULL POWER

[ ]  Rotate .... 77 KIAS

[ ]  Best Angle – Vx .... 90 KIAS

[ ]  Best Rate – Vy .... 110 KIAS

Cruise Climb

[ ]  Manifold Pressure .... 34" – 35"

[ ]  Propeller Control .... 2400 RPM

[ ]  Mixture Control (35 UsGal/h) .... FULL RICH

[ ]  Airspeed .... 125 KIAS

[ ]  Aux. Fuel Pump .... LOW (IF REQ'D)

[ ]  Induction / Alternate Air .... SET

[ ]  Pressurization / Altimeter .... SET

Descent

[ ]  Mixture .... MIN. 1500°F TIT

[ ]  Propeller Control .... CRUISE SETTING

[ ]  Pressurization / Altimeter .... SET

[ ]  ATIS & Appr. Briefing .... CHECK

[ ]  Power .... AS REQ'D

Approach & Before Landing

[ ]  Fuel Selector .... PROPER TANK

[ ]  Seat & Seats Belts .... SET

[ ]  Aux. Fuel Pump .... AS REQUIRED

[ ]  Landing Gear down locked .... 3 GREEN

[ ]  Flaps / Spoiler .... AS REQUIRED

[ ]  Toe Brakes .... PUMP & TEST

[ ]  Induction / Alternate Air .... PRIMARY

[ ]  Airspeed .... 85 KIAS

After Landing & Parking

[ ]  Ice Protection & Txpdr .... OFF

[ ]  Flaps / Spoiler .... RETRACT

[ ]  Avionics Radio Master .... OFF

[ ]  Engine cool down .... 800°F TIT, CHT GREEN

[ ]  Engine (after 3-4min) .... SHUT DOWN

[ ]  Magentos / Alternators .... OFF

[ ]  Battery Master .... OFF

[ ]  Brake & Flight Control Lock .... SET

Limitations & Speeds (KIAS)

[ ]  Oil Temperatur min. for Take-Off .... 100°F

[ ]  Oil Temperatur max. allowable .... 240°F

[ ]  Oil Temperatur Cruise .... 170-200°F

[ ]  Cylinder Heat Temp in Flight .... MAX. 420°F

[ ]  Cylinder Heat Temp max. allowable .... 460°F

[ ]  T.I.T. in Flight .... MIN. 1500°F, MAX. 1750°F

[ ]  T.I.T. max. allowable for 30 sec. .... 1800°F

SPEEDS KIAS

[ ]  V1 (decision speed at half TODA) .... 55

[ ]  Vrot .... 77

[ ]  Vx .... 90

[ ]  Vy .... 110

[ ]  Vlgear up .... 130

[ ]  Vlgear down .... 170

[ ]  Vle .... 200

[ ]  Vflap down 10° .... 170

[ ]  Vde-ice .... 183

[ ]  VA max (4100lbs/1860kg) .... 135

[ ]  VA min (2450lbs/1110kg) .... 103

[ ]  Vglide .... 90

[ ]  norm.cruise TAS x 70% = if gear down cruise

[ ]  Vs (clean) .... 69

[ ]  Vso (landing) .... 58

ANGLE OF BANK (VS (VSO))

[ ]  10° .... 70 (60)

[ ]  30° .... 76 (66)

[ ]  45° .... 82 (75)

[ ]  60° .... 98 (86)

[ ]  BEW .... 1324kg (2918lbs)

[ ]  MTOW .... 1860kg (4100lbs)

[ ]  ZFW .... 1772kg (3900lbs)

[ ]  MLW .... 1772kg (3900lbs)

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-46 Malibu/Daniel_Draxler_PA46_cheklist1b.txt

Rights: licensed

Checklist hash: 8eae39a855f5aed782f66ee8fc03d5ebc8462058db98569f79e28dd52a8dbbd1

Verify against the aircraft's own approved documentation before use.