Piper PA-28-181 Archer III — Emergency Procedures

NOT FOR FLIGHT — source fidelity: not_applicable | operational review: none | completeness: partial | 2 unresolved safety defect(s)

safety_defect: Contains no airspeeds, power settings, or other numeric limits — must come from aircraft POH.

safety_defect: Emergency procedures must be studied, briefed, and practiced. This file is an aide-memoire. Aviate first, then navigate and communicate.

content_gap: Equipment variations across this production run may affect available systems during emergencies. Verify all procedures against the specific aircraft POH before flight.

Engine Failure on Takeoff Roll (Before Lift-Off)  [EMERGENCY]

[ ]  Throttle [MEMORY] .... CLOSE IMMEDIATELY

[ ]  Brakes [MEMORY] .... APPLY FIRMLY — STOP ON REMAINING RUNWAY

[ ]  Mixture [MEMORY] .... IDLE CUT-OFF

[ ]  Ignition [MEMORY] .... OFF

[ ]  Fuel selector [MEMORY] .... OFF

[ ]  Master switch [MEMORY] .... OFF

[ ]  Evacuate aircraft if fire or fuel spill present .... AS REQUIRED

Engine Failure After Lift-Off  [EMERGENCY]

[ ]  Best glide airspeed — pitch immediately to conserve altitude and extend glide range [MEMORY] .... ESTABLISH PER POH

[ ]  Landing area — select best available surface straight ahead or with the smallest practical turn; avoid low turns to return to runway [MEMORY] .... SELECTED

[ ]  Throttle [MEMORY] .... CLOSE

[ ]  Flaps — adjust for selected landing surface; partial flap may allow steeper descent to a short field [MEMORY] .... AS REQUIRED FOR TERRAIN

[ ]  Mixture [MEMORY] .... IDLE CUT-OFF BEFORE TOUCHDOWN

[ ]  Ignition [MEMORY] .... OFF BEFORE TOUCHDOWN

[ ]  Master switch [MEMORY] .... OFF BEFORE TOUCHDOWN

Engine Failure in Flight  [EMERGENCY]

[ ]  Best glide airspeed — establish immediately to maximize range and time available [MEMORY] .... ESTABLISH PER POH

[ ]  Landing area — identify best available field or surface within glide range [MEMORY] .... SELECTED

[ ]  Carburetor heat [MEMORY] .... ON — apply immediately; carb ice may have caused the failure

[ ]  Fuel selector — switch to the tank with more fuel; verify selector is on a tank [MEMORY] .... FULLEST TANK

[ ]  Mixture [MEMORY] .... RICH

[ ]  Throttle [MEMORY] .... ADVANCE — check if partial power is available

[ ]  Primer — ensure closed and locked; an unlocked primer can lean the mixture .... IN AND LOCKED

[ ]  Ignition — cycle through LEFT and RIGHT to identify a failed magneto or broken lead .... CHECK BOTH

[ ]  Restart attempt — if engine fails to start and forced landing is committed, execute forced landing checklist .... ATTEMPT OR COMMIT TO FORCED LANDING

[ ]  Mayday call — declare emergency with position, altitude, and intentions .... 121.5 MHZ OR LAST ATC FREQUENCY

Engine Fire in Flight  [EMERGENCY]

[ ]  Mixture [MEMORY] .... IDLE CUT-OFF — starve the engine of fuel immediately

[ ]  Fuel selector [MEMORY] .... OFF

[ ]  Throttle [MEMORY] .... FULL OPEN — draw fuel remaining in the induction system away from the fire

[ ]  Ignition [MEMORY] .... OFF

[ ]  Master switch [MEMORY] .... OFF

[ ]  Heater and vents — close all fresh-air vents and cabin heat to reduce airflow feeding the fire [MEMORY] .... CLOSED

[ ]  Airspeed — increase to a speed that may help extinguish the fire by slipstream; do not exceed Vne [MEMORY] .... INCREASE WITHIN LIMITS

[ ]  Forced landing — execute forced landing procedure immediately; do not delay for a restart [MEMORY] .... INITIATE FORCED LANDING

Engine Fire on Ground (During or After Start)  [EMERGENCY]

[ ]  Starter — continue cranking if fire is in the induction system; cranking may draw fire back through the intake [MEMORY] .... CONTINUE CRANKING (induction fire)

[ ]  Throttle — advance to assist drawing fuel and fire into the engine if induction fire [MEMORY] .... ADVANCE (induction fire only)

[ ]  Mixture [MEMORY] .... IDLE CUT-OFF

[ ]  Fuel selector [MEMORY] .... OFF

[ ]  Ignition [MEMORY] .... OFF

[ ]  Master switch [MEMORY] .... OFF

[ ]  Evacuate — exit all occupants immediately; move upwind away from the aircraft [MEMORY] .... EVACUATE IMMEDIATELY

[ ]  Fire extinguisher — use only if fire is small, accessible, and escape route is clear .... AS ABLE; CALL FIRE SERVICES

Electrical Fire / Smoke in Cockpit  [EMERGENCY]

[ ]  Master switch [MEMORY] .... OFF — remove all electrical power

[ ]  Avionics master switch [MEMORY] .... OFF

[ ]  All electrical switches — lights, fan, pitot heat [MEMORY] .... OFF

[ ]  Vents — open to vent smoke from cockpit; fresh air assists in clearing fumes [MEMORY] .... OPEN VENTS, CLOSE HEAT

[ ]  Fire extinguisher — discharge toward source of smoke if identified and accessible .... AS REQUIRED

[ ]  Land as soon as practicable — smoke or fumes indicate a potential for rapid fire spread; do not delay [MEMORY] .... DIVERT AND LAND

[ ]  Electrical switches — after smoke clears, restore individual switches one at a time to identify the faulty circuit before landing .... RESTORE SELECTIVELY AS NEEDED

[ ]  Circuit breakers — do not reset a tripped breaker in flight unless essential for safety; a tripped breaker indicates a fault .... DO NOT RESET UNLESS ESSENTIAL

Loss of Oil Pressure  [EMERGENCY]

[ ]  Oil pressure gauge — verify indication; cross-check oil temperature for confirming trend [MEMORY] .... VERIFY; CROSS-CHECK OIL TEMP

[ ]  Throttle — reduce power to minimum needed to maintain flight; lower power reduces engine stress [MEMORY] .... REDUCE TO MINIMUM REQUIRED

[ ]  Landing area — identify a suitable field or airport within close range; engine seizure may be imminent [MEMORY] .... IDENTIFY NEAREST SUITABLE AREA

[ ]  Mayday call — declare emergency with position, nature of malfunction, and intentions .... DECLARE EMERGENCY

[ ]  Land as soon as possible — if pressure is zero or temperature is rising abnormally, do not delay; engine seizure can occur without warning [MEMORY] .... LAND IMMEDIATELY

[ ]  After landing — do not run the engine; inspect for oil loss before any further operation .... SHUT DOWN AND INSPECT

Carburetor Ice  [ABNORMAL]

[ ]  Carburetor heat — apply FULL ON at first sign of unexplained RPM loss or engine roughness in cruise or descent [MEMORY] .... FULL ON

[ ]  RPM — monitor for initial further drop as ice melts and passes through the induction system, followed by recovery .... EXPECT BRIEF DROP THEN RECOVERY

[ ]  Throttle — avoid large changes while carburetor heat is applied; heat reduces available power .... STABLE THROTTLE

[ ]  Carburetor heat — leave ON until RPM recovers and stabilizes above original indication; then return to COLD .... LEAVE ON UNTIL RECOVERED; THEN COLD

[ ]  If RPM does not recover — land as soon as practicable; severe ice accumulation may not fully clear with heat alone [MEMORY] .... DIVERT IF NO RECOVERY

[ ]  Icing conditions — if conditions are conducive to carburetor ice (temperature between approximately 20 and 70 F with visible moisture or high humidity), apply carburetor heat periodically as a preventive measure .... APPLY PERIODICALLY IN ICING CONDITIONS

Forced Landing (Power-Off)  [EMERGENCY]

[ ]  Best glide airspeed — establish and maintain throughout the approach; do not let speed decay or build excessively [MEMORY] .... ESTABLISH AND MAINTAIN PER POH

[ ]  Landing area — select field considering length, surface, slope, obstacles, and wind direction; commit early [MEMORY] .... SELECTED AND COMMITTED

[ ]  Mayday call — transmit position, altitude, nature of emergency, and intended landing area .... 121.5 MHZ OR LAST ATC FREQUENCY

[ ]  Transponder .... 7700

[ ]  Fuel selector [MEMORY] .... OFF

[ ]  Mixture [MEMORY] .... IDLE CUT-OFF

[ ]  Ignition [MEMORY] .... OFF at short final, prior to flare

[ ]  Master switch [MEMORY] .... OFF at short final, prior to flare

[ ]  Seat belts and shoulder harnesses [MEMORY] .... CONFIRM TIGHT ON ALL OCCUPANTS

[ ]  Doors — unlatch before touchdown to prevent jamming in impact; hold door slightly ajar [MEMORY] .... UNLATCHED BEFORE TOUCHDOWN

[ ]  Flaps — extend on final as needed to control descent angle and landing speed per POH .... AS REQUIRED FOR FIELD AND CONDITIONS

[ ]  After landing — evacuate aircraft promptly and move upwind to prevent injury from possible fire .... EVACUATE AND MOVE CLEAR

NOT FOR FLIGHT — source fidelity: not_applicable | operational review: none | completeness: partial | 2 unresolved safety defect(s)

Source: authored — no source document

Rights: original_expression CC-BY-4.0

Checklist hash: 93016fd46fb28dd9014bf960a54a4fe5f618c0e97c137dcd93b452fe3157ea57

Verify against the aircraft's own approved documentation before use.