PZL 150A Koliber — Ariel_Batista_PZL_Koliber_150a_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Light Signals" item count differs between passes (10 vs 6); verify nothing was dropped.

Preflight Inspection

COCKPIT

[ ]  Canopy

[ ]  Aircraft docs (AROW)

[ ]  Weight & Balance

[ ]  Control wheel lock .... Remove

[ ]  Hobbs/Tach

[ ]  Magneto SW/Ignition .... Off

[ ]  Avionics Power Switch .... Off

[ ]  Master Switch .... On

[ ]  Fuel quantity indicators

[ ]  Flaps .... Extend

[ ]  Trimming tab .... “T-O” Position

[ ]  Fuel valve .... On fullest tank

[ ]  Stall warning

[ ]  Master Switch .... Off

LEFT WING

[ ]  Wing tie-down .... Disconnect

[ ]  Flaps

[ ]  Aileron

[ ]  Pitot tube

[ ]  Fuel tank vent open

[ ]  Slats

[ ]  Fuel Quantity

[ ]  Fuel Filler Plug

[ ]  Fuel tank sump .... Sample

[ ]  Main wheel tire

[ ]  Brakes

[ ]  Landing/Taxi Light(s)

NOSE

[ ]  Engine oil level

[ ]  Fuel strainer .... Sample

[ ]  Propeller and spinner

[ ]  Alternator belt

[ ]  Air intake

[ ]  Carburetor air filter

[ ]  Nose wheel strut & tire

RIGHT WING

[ ]  Wing tie-down .... Disconnect

[ ]  Aileron

[ ]  Flaps

[ ]  Fuel Quantity

[ ]  Fuel Filler Plug

[ ]  Fuel tank sump .... Sample

[ ]  Slats

[ ]  Main wheel tire

[ ]  Brakes

FUSELAGE AND EMPENAGE

[ ]  Rivets

[ ]  Rudder Cables Both sides

[ ]  Tail Tie-Down .... Disconnect

[ ]  Control surfaces

[ ]  Trim Tab

[ ]  Antennas

Before starting engine

[ ]  Preflight inspection

[ ]  Passenger Briefing

[ ]  Seats, belts .... Adjust & Lock

[ ]  Brakes

[ ]  Fuel Selector Valve .... Fullest Tank

[ ]  Avionics Power Switch .... Off

[ ]  Electrical Equip

[ ]  Circuit breakers

Starting Engine

[ ]  Mixture .... Rich (IN)

[ ]  Carburetor heat .... Cold (IN)

[ ]  Battery & Alternator SW .... On

[ ]  Fuel Pump .... On

[ ]  Anti-Collision light .... On

[ ]  Prime

[ ]  Throttle .... Open 1/4 inch

[ ]  Propeller area

[ ]  Starter .... Engage

[ ]  Engine .... 800-1000 rpm

[ ]  Oil Pressure

Before Taxi

[ ]  Avionics Master Switch .... On

[ ]  Fuel Pump .... Off

[ ]  Turn and Bank .... On

[ ]  Lights

[ ]  Radios .... Frequencies Set

[ ]  Brake

[ ]  Primer

Taxi

[ ]  Elevator .... Neutral or Aft.

[ ]  Flaps .... Up

[ ]  Flight & Engine Inst.

Engine Run-Up

[ ]  Parking brake .... Set

[ ]  Mixture .... Rich

[ ]  Throttle .... 1700 RPM

[ ]  Magnetos

[ ]  Carburetor Heat

[ ]  Engine Inst & Ammeter

[ ]  Fuel Pump

[ ]  Suction Gauge

[ ]  Throttle .... 800-1000

Before Takeoff

[ ]  Flight Controls

[ ]  Canopy .... Closed

[ ]  Emergency Trim S/W

[ ]  Electric Trim

[ ]  Flaps

[ ]  Encoder Switch .... On

[ ]  Transponder

[ ]  Avionics

[ ]  Lights

Takeoff Normal

[ ]  Flaps .... Up

[ ]  Throttle .... Full Open

[ ]  Rotate .... 45 – 50 Kts.

[ ]  Initial Climb .... 62 Kts.

[ ]  Passing 500 FT .... Fuel Pump Off

T/O Short & Soft

[ ]  Flaps .... Full

[ ]  Control Stick .... Full Back

[ ]  Throttle .... Full Open

[ ]  Control Stick .... Ease Forward

[ ]  Initial Climb .... 50-55 Kts.

[ ]  Passing 100 FT .... 61 Kts.

[ ]  Flaps .... Retract Slowly

[ ]  Passing 500 FT .... Fuel Pump Off

Climb

[ ]  Power .... Full Open

[ ]  Engine Temp

[ ]  Best Angle of Climb Vx

[ ]  Best Rate of Climb Vy

[ ]  Lights

Cruise

[ ]  Power

[ ]  Mixture .... Adjust

[ ]  Trim

[ ]  Fuel Tanks

NOTE: Switch tanks with Pumps on

Descent

[ ]  Power

[ ]  Mixture .... Rich

[ ]  Carb Heat

Before Landing

[ ]  Belts & Harnesses

[ ]  Flight instruments

[ ]  Radios

[ ]  ATIS

[ ]  Fuel Pump .... On

[ ]  Fuel Tanks

NOTE: Switch tanks with Pumps on

[ ]  Carb Heat .... On (Out)

[ ]  Mixture .... Rich

[ ]  Landing light .... On

[ ]  Airspeed .... 65-75 KIAS (Flaps Up)

NOTE: 60-70 KIAS (Flaps Dn)

After landing

[ ]  Flaps .... Up

[ ]  Carb Heat .... Cold (In)

[ ]  Transponder .... Standby

[ ]  Landing light .... Off

Parking

[ ]  Avionics .... Off

[ ]  Electrical .... Off

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... Cut-off

[ ]  Ignition switch .... Off

[ ]  Master switch .... Off

Securing the aircraft

[ ]  Control Lock .... Install

[ ]  Hobbs/Tach

[ ]  Canopy

[ ]  Tie-downs

Go-around

[ ]  Throttle .... Full

[ ]  Carb Heat .... Cold (In)

[ ]  Flaps .... 20(

[ ]  Climb .... 55 KIAS

[ ]  Flaps .... 10(

[ ]  Climb .... 60 KIAS

[ ]  Flaps .... Up

[ ]  Climb .... 79 KIAS

Engine failure

TAKEOFF

[ ]  Throttle .... Idle

[ ]  Brakes .... Apply

[ ]  Flaps .... Retract

[ ]  Mixture .... IDLE cut-off

[ ]  Ignition .... Off

[ ]  Master switch .... Off

Engine failure

AFTER TAKEOFF

[ ]  Airspeed .... 65 KIAS (flaps UP)

NOTE: 60 KIAS (flaps DN)

[ ]  Mixture .... Idle Cut-off

[ ]  Fuel shutoff valve .... Off (Out)

[ ]  Ignition .... Off

[ ]  Flaps

[ ]  Master switch .... Off

[ ]  Cabin Doors .... Unlatch

[ ]  Land .... Straight Ahead

Engine failure

DURING FLIGHT

[ ]  Airspeed .... 65 KIAS

LOOK FOR A FIELD

[ ]  Fuel shutoff valve .... On (In)

[ ]  Fuel selector valve .... Both

[ ]  Aux Fuel Pump Switch .... On

[ ]  Primer .... In & Locked

[ ]  Mixture .... Rich

[ ]  Ignition .... BOTH (or START)

[ ]  Airspeed .... 65 KIAS (flaps UP)

NOTE: 60 KIAS (flaps DN)

[ ]  Mixture .... Idle Cut-off

[ ]  Fuel shutoff valve .... Off (Out)

[ ]  Ignition .... Off

[ ]  Flaps

[ ]  Mayday .... Transmit 121.5

[ ]  Mayday .... Squawk 7700

[ ]  Master switch .... Off

[ ]  Touchdown .... Tail Low

Carburetor Icing  [ABNORMAL]

[ ]  Throttle .... Full

[ ]  Carb Heat .... On (Out)

[ ]  Mixture .... Adjust

Engine Roughness  [ABNORMAL]

[ ]  Magnetos

[ ]  Mixture .... Lean as necessary

Engine Fire during start  [EMERGENCY]

[ ]  Crank

[ ]  Power .... 1700 RPM (2 min)

[ ]  Engine

Engine Fire during flight  [EMERGENCY]

[ ]  Mixture .... Idle Cut-off

[ ]  Fuel shutoff valve .... Off (Out)

[ ]  Master Switch .... Off

[ ]  Boost Pump .... Off

[ ]  Cabin Heat/Air .... Off

[ ]  Airspeed .... 100 KIAS

Electrical failure  [ABNORMAL]

[ ]  Load meter

[ ]  Alternator .... Off

REDUCE LOAD TO MINIMUM

[ ]  Breaker/alt

[ ]  Alternator .... On

IF STILL NO POWER:

[ ]  Alternator .... Off

REDUCE LOAD AND LAND

Electrical overload  [ABNORMAL]

[ ]  Master Switch .... Off

[ ]  Master Switch .... On

[ ]  Over-voltage light .... Off

WARNING: or TERMINATE flight ASAP

Spin Recovery  [EMERGENCY]

[ ]  Ailerons .... NEUTRAL

[ ]  Throttle .... IDLE

[ ]  Rudder .... Full opposite

[ ]  Control wheel .... Full forward

[ ]  Rudder control/wheel .... Neutral

[ ]  Pitch .... Level

Light Signals

[ ]  Steady Green — On Ground

[ ]  Steady Green — In Flight

[ ]  Flashing Green — On Ground

[ ]  Flashing Green — In Flight

[ ]  Steady Red — On Ground

[ ]  Steady Red — In Flight

[ ]  Flashing Red — On Ground

[ ]  Flashing Red — In Flight

[ ]  Flashing white — On Ground

[ ]  Red/Green alternating

Comm Freq

[ ]  ATIS

[ ]  Ground

[ ]  Tower

[ ]  Club

[ ]  Fuel

V-Speeds (KIAS)

[ ]  Rotate Vr

[ ]  Normal Climb Vclm

[ ]  Max angle Vx (Sea Level)

[ ]  Max angle Vx (10,000 ft)

[ ]  Climb rate Vy (Sea Level)

[ ]  Climb rate Vy (10,000 ft)

[ ]  Maneuver Va

[ ]  Flaps Vfe

[ ]  Normal max Vno

[ ]  Never exceed Vne

[ ]  Stall (clean) Vs

[ ]  Stall (land) Vso

[ ]  Final Approach Flaps

[ ]  Final Approach No flaps

[ ]  Best Glide

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: PZL/150A Koliber/Ariel_Batista_PZL_Koliber_150a_Checklist.txt

Rights: licensed

Checklist hash: 02430d7a7d38ab93e36facf150583b876f2fdf85363890a302f9af5d4034ce37

Verify against the aircraft's own approved documentation before use.