Quad City Ultralight Aircraft Challenger — Douglas_White_Quad_City_Ultralight_Aircraft_Challenger_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Aircraft Weight and Balance" item count differs between passes (11 vs 10); verify nothing was dropped.

content_gap: Section "Preflight Inspection" item count differs between passes (45 vs 50); verify nothing was dropped.

Preflight Inspection

COCKPIT

[ ]  Ignition .... OFF

[ ]  Master Switch .... ON

[ ]  Fuel Gauges .... CHECK

[ ]  Master Switch .... OFF

[ ]  Primary Flight Controls .... CHECK

[ ]  Flaps .... CHECK

[ ]  Baggage .... STOWED

[ ]  Required Papers .... ON BOARD

[ ]  Parking Brake .... ON/As Required

NOSE SECTION

[ ]  General Condition .... CHECK

[ ]  Nose Gear Tire .... CHECK

[ ]  Nose Wheel Strut .... Proper Inflation (3”)

[ ]  Windshield .... CLEAN

LEFT WING

[ ]  Chock and Tie Down .... REMOVED

[ ]  Surface Condition .... CHECK

[ ]  Wing Tip .... CHECK

[ ]  Lights .... CHECK

[ ]  Flaperon and Hinges .... CHECK/Neutral

[ ]  Landing Gear and Tire .... CHECK

[ ]  Brake Block and Disk .... CHECK

FUSELAGE (LEFT SIDE)

[ ]  General Condition .... CHECK

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Cap .... CLOSE and SECURE

[ ]  Antennas .... CHECK

[ ]  Prop and Spinner .... CHECK

[ ]  Air Filter .... CHECK

[ ]  Spark Plugs & Electrical wiring .... CHECK

[ ]  Engine Fuel Lines .... CHECK

[ ]  Carburetors .... CHECK Float Chambers

[ ]  Fan Belt .... CHECK For Tightness

[ ]  Motor Mounts .... CHECK For Cracked Rubber

[ ]  Fuel Tank Sump .... CHECK For Water

RIGHT WING

[ ]  Check as Left Wing

EMPENNAGE

[ ]  General Condition .... CHECK

[ ]  Hinges and Attachments .... CHECK

[ ]  Rudder & Elevator Rods .... CHECK

FUSELAGE (RIGHT SIDE)

[ ]  General Condition .... CHECK

[ ]  Exhaust &Springs .... CHECK/SECURE

Before Starting Engine

[ ]  Cockpit Doors .... AS REQUIRED

[ ]  Seats .... ADJUSTED AND LOCKED

[ ]  Seat Belts & Harnesses .... FASTENED

[ ]  Parking Brake .... SET

Starting Engine (COLD)

[ ]  Prime .... 5 PUMPS

[ ]  Throttle .... Advance 1/2

[ ]  Master Switch .... ON

[ ]  Choke .... OFF

[ ]  Area and Propeller .... “CLEAR”

[ ]  Starter .... ENGAGE

[ ]  Throttle .... ADVANCE SLIGHTLY

Starting Engine (Hot)

[ ]  Prime .... 3-4 PUMPS

[ ]  Throttle .... Advance 1/2

[ ]  Master Switch .... ON

[ ]  Choke .... OFF

[ ]  Area and Propeller .... “CLEAR”

[ ]  Starter .... ENGAGE

[ ]  Throttle .... ADJUST

Starting Engine (Flooded)

[ ]  Throttle .... OPEN FULL

[ ]  Master Switch .... ON

[ ]  Choke .... OFF

[ ]  Area and Propeller .... “CLEAR”

[ ]  Starter .... ENGAGE

[ ]  Throttle (warm up) .... 2000 to 3000 RPM

[ ]  Idle .... 2000 RPM

Before Taxi

[ ]  Intercom .... HEADSETS CHECKED

[ ]  Radios .... ON

[ ]  Taxi Area .... CLEAR

[ ]  Brakes .... RELEASED/CHECK

[ ]  Steering .... CHECK

Run Up

[ ]  Brakes .... ON

[ ]  Throttle .... 6600 RPM

[ ]  Magnetos .... 175 Max. Drop/ 50 Diff.

[ ]  EGT .... CHECK

[ ]  CHT .... CHECK

[ ]  Throttle .... IDLE

Before Takeoff

[ ]  Master Switch .... CHECK ON

[ ]  Flight Instruments .... CHECK

[ ]  Fuel Quantity .... CHECK

[ ]  Choke .... OFF

[ ]  Engine Gauges .... CHECK

[ ]  Belts/Harness .... FASTENED

[ ]  Empty Seat Belt .... FASTENED

[ ]  Flaperons .... NEUTRAL

[ ]  Controls .... FREE

[ ]  Doors .... LATCHED (If Attached)

Takeoff

[ ]  Accelerate (6800 RPM) .... 45 MPH

[ ]  Back Pressure to Rotate to Climb Attitude

[ ]  Climb Speed (6600 RPM) .... 55 MPH

Short Field Takeoff, Obstacle Clearance

[ ]  Accelerate .... 45 MPH

[ ]  Back Pressure to Rotate to Climb Attitude

[ ]  Accelerate (until over obstacle) .... 50 MPH

[ ]  Accelerate (after over obstacle) .... 55 MPH

Short Field Takeoff, No Obstacle

[ ]  Accelerate .... 45 MPH

[ ]  Back Pressure to Rotate to Climb Attitude

[ ]  Accelerate (after breaking ground) .... 55 MPH

Soft Field Takeoff, With or Without Obstacle

[ ]  Accelerate and lift off nose as soon as possible.

[ ]  Lift off at slowest possible airspeed.

[ ]  Accelerate just above ground to 50 MPH to climb past obstacle height (if present).

[ ]  Accelerate (after over obstacle) .... 55 MPH

Climb

[ ]  VX – Best Angle of Climb Speed .... 50 MPH

[ ]  VY – Best Rate of Climb Speed .... 55 MPH

Cruise

[ ]  Normal Max. Power .... 6600 RPM

[ ]  Cruise Power .... 5700 RPM

Approach and Landing

[ ]  Belts/Harness .... FASTEN

[ ]  Final Approach Speed .... 55 MPH

[ ]  Landing Speed .... 45 MPH

Engine Shutdown

[ ]  Radios .... OFF

[ ]  Magnetos .... CHECK GROUND

[ ]  Throttle .... FULL AFT

[ ]  Choke .... FULL ON

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

[ ]  Parking Brake .... SET

[ ]  Control Stick .... SECURED WITH BELT

[ ]  Flaperons .... NEUTRAL

[ ]  Wheel Chocks .... IN PLACE

[ ]  Tie Downs .... SECURE

Aircraft Performance Speeds

[ ]  VS0 – Landing configuration stall at gross weight, power off .... 35 MPH

[ ]  VS1 – Cruise configuration stall at gross weight, power off .... 35 MPH

[ ]  Best Glide .... 55 MPH

[ ]  VREF (Short/Soft) .... 55 MPH

[ ]  VX – Best Angle of Climb Speed .... 50 MPH

[ ]  VY – Best Rate of Climb Speed .... 55 MPH

[ ]  Cruise Speed .... 75-85 MPH

[ ]  Top Speed .... 90-100 MPH

[ ]  VNO – Red Line .... 110 MPH

[ ]  VNE –– Never Exceed .... 120 MPH

[ ]  VL/D .... 55 MPH

[ ]  Max. X-wind .... 30 MPH

Aircraft Specifications

[ ]  Maximum Gross Weight .... 1041 LBS

[ ]  Basic Empty Weight (N548SL) .... 451 LBS

[ ]  Useful Load (N548S) .... 500 LBS

[ ]  Max. Front Seat .... 250 LBS

[ ]  Min. Front Seat .... 170 LBS

[ ]  Fuel Capacity (useable) .... 15 GALS

[ ]  Fuel Consumption .... 4.5 GPH

[ ]  Cruise Speed .... 85 MPH

Aircraft Weight and Balance

QUAD CITY CHALLENGER | WEIGHT | ARM | MOMENT

2: N548SL

[ ]  Basic Empty Weight .... 451 | 104 | 47016

[ ]  Pilot .... 250 | 50 | 12500

[ ]  Co-Pilot .... 180 | 77 | 13860

[ ]  Fuel Gal. (15 Max) .... 90 | 103 | 9270

[ ]  Total (M.G.W.= 951 lbs) .... 971 | 85.1 | 82646

[ ]  Fuel (10 gal) .... 60 | 103 | 6180

[ ]  Total (M.G.W.= 951 lbs) .... 941 | 84.5 | 79556

CAUTION: Acceptable moment arm for takeoff weight and balance is 80 to 90 inches

CAUTION: The Challenger generally stalls with wings level, nose level, but altitude descent of 500 FPM. Listen for a propeller “Growl”. Practice stalls and slow flight at 2000AGL

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Quad City Ultralight Aircraft/Challenger/Douglas_White_Quad_City_Ultralight_Aircraft_Challenger_Checklist.txt

Rights: licensed

Checklist hash: 99c0547bee9a06298992170ff8e1f4e199c9d0b73bb05392f30b14ca037c5557

Verify against the aircraft's own approved documentation before use.