Robinson R22 — michael_smith_r22_eng_start_shutdown_cklistDS

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE STARTING ENGINE

[ ]  Seat belts .... Fastened

[ ]  Fuel shut-off valve .... On

[ ]  Cyclic/collective friction .... Off

[ ]  Cyclic, collective, pedals .... Full travel free

[ ]  Throttle .... Full travel free

[ ]  Collective .... Full down, friction on

[ ]  Cyclic neutral .... Friction on

[ ]  Pedals .... Neutral

[ ]  Governor .... Off

[ ]  Circuit breakers .... In

[ ]  Carb heat .... Off

[ ]  Mixture .... Full rich

[ ]  Mixture guard .... Installed

[ ]  All switches/avionics .... Off

[ ]  Clutch .... Disengaged

[ ]  Altimeter .... Set

[ ]  Rotor brake .... Disengaged

[ ]  Map light .... Off

STARTING ENGINE & RUNUP (headset-off one ear)

[ ]  Master switch .... On

[ ]  Throttle twists for priming .... As required

[ ]  Throttle .... Closed

[ ]  Area .... Clear

[ ]  Strobe light .... On

CAUTION: CAUTION – Be sure rotor blades are approximately level to avoid possible tailcone strike.

[ ]  Ignition switch .... Start, then both

[ ]  Started-On light .... Out

[ ]  Set idle speed .... 55%

[ ]  Clutch switch .... Engaged (no delay)

[ ]  Alternator switch .... On (no delay)

[ ]  Blades turning .... Less than 5 seconds

[ ]  Oil pressure in 30 sec. .... 25 psi minimum

[ ]  Avionics, headsets .... On

[ ]  Wait for clutch light .... Out

[ ]  Warm-up RPM .... 70 – 75%

[ ]  Engine gages .... Green

[ ]  Warning lights .... Out

[ ]  Mag drop @ 75% RPM .... 7% max in 2 sec.

[ ]  Carb heat check .... CAT rise/drop

[ ]  Sprag clutch check from 75% RPM .... Needles split

[ ]  Doors .... Closed & latched

[ ]  Limit MAP chart .... Check

[ ]  Cyclic/collective friction .... Off

[ ]  Governor on .... RPM 102-104%

[ ]  Lift collective slightly, reduce RPM .... Horn/light at 97% rotor RPM

CAUTION: CAUTION – Avoid continuous operation at rotor speed of 60-70% to minimize tail resonance.

CAUTION: CAUTION – On slippery surfaces, be prepared to counter nose right rotation with left pedal as governor increases RPM.

NOTE: NOTE – During run-up & shutdown, pilot should uncover right ear, open right door, and listen for unusual bearing noise. Failing bearing swill produce an audible whine or growl well before final failure.

SHUTDOWN PROCEDURE

[ ]  Collective down .... Friction on

[ ]  Cyclic/pedals neutral .... Friction on

[ ]  Governor .... Off

[ ]  Idle at 70-80% .... CHT drop (~4 minutes)

[ ]  Throttle .... Closed

[ ]  Clutch switch .... Disengaged

[ ]  Wait 30 sec .... Pull idle cut-off

[ ]  Mixture guard .... Back on mixture

[ ]  Wait 30 sec .... Apply rotor brake

[ ]  Clutch light off .... All switches off (battery last)

CAUTION: CAUTION – Do not slow rotor by raising collective during shutdown. Blades may flap and strike tailcone.

NOTE: Notes:

MAXIMUM CONTINUOUS POWER

PRESS ALT-FT

OAT - C°

-20

-10

0

10

20

30

40

SL

21.5

21.8

22.1

22.3

22.6

22.9

23.2

2000

21.1

21.4

21.6

21.9

22.2

22.5

22.8

4000

20.7

21.0

21.2

21.5

21.8

22.0

22.3

6000

20.3

20.6

20.8

21.1

21.3

21.6

21.9

8000

19.9

20.2

20.4

20.7

20.9

FULL THROTTLE

NOTE: FOR MAX TAKEOFF POWER (5 MIN), ADD 0.9 IN. HG

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Robinson/R22/michael_smith_r22_eng_start_shutdown_cklistDS.txt

Rights: licensed

Checklist hash: cd08c93db3c4b03e390748cba1bdabd1d423982f6c9b679d8dc57ba7c3a394f2

Verify against the aircraft's own approved documentation before use.