Robinson R22 — michael_smith_r22_preflight_checklist_ds

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Cowl Door / Engine Right Side

[ ]  Check maintenance records, check general condition - leaks, discolorization dents, chafing, nicks, corrosion or cracks.

[ ]  Verify Telatemps show no unexplained temperature increases during prior flight.

COWL DOOR

[ ]  Master switch .... On

[ ]  Oil pressure, alternator, governor lights, low fuel .... On

[ ]  Warning light test switches .... Push to test

[ ]  Fuel quantity

[ ]  Rotor brake .... Actuation normal

[ ]  Master switch .... Off

[ ]  Aux fuel tank quantity

[ ]  Fuel filler cap .... Tight

[ ]  Aux fuel tank .... No leaks

[ ]  Aux fuel drain .... Sample

[ ]  Gearbox oil .... Full, no leaks

[ ]  Gearbox Telatemp .... Normal

[ ]  Flex coupling .... No cracks, nuts tight

[ ]  Yoke flanges .... No cracks

[ ]  Sprag clutch .... No leaks

[ ]  Static source .... Clear

[ ]  Control rod ends .... Free without looseness

[ ]  Steel tube frame .... No cracks

[ ]  All fasteners .... Tight

[ ]  Tail rotor control .... No interference

[ ]  Cowl door .... Latched

ENGINE RIGHT SIDE

[ ]  Carb air ducts .... Secure

[ ]  Carb heat scoop .... Secure

[ ]  Engine sheet metal .... No cracks

[ ]  Electrical terminals .... Tight

[ ]  Fuel line .... No leaks

[ ]  Oil cooler door

[ ]  Oil lines .... No leaks or chafing

[ ]  Exhaust system .... No cracks

[ ]  Engine general condition

[ ]  V-belt condition

[ ]  V-belt slack

[ ]  Sprag clutch .... No leaks

[ ]  Upper bearing .... No leaks

[ ]  Telatemp – upper bearing .... Normal

[ ]  Lower sheave groove wear .... Smooth & uniform

[ ]  Flex coupling .... No cracks, nuts tight

[ ]  Yoke flanges .... No cracks

[ ]  Steel tube frame .... No cracks

[ ]  Tail rotor control .... No interference

Engine - Rear / Empennage / Tail Rotor / Tailcone

ENGINE - REAR

[ ]  Cooling fan nut .... Pin in line w/ marks

[ ]  Cooling fan .... No cracks

[ ]  Fan scroll .... No cracks

[ ]  Telatemps – lower bearing .... Normal

[ ]  Lower bearing .... No leaks

EMPENNAGE

[ ]  Tail surfaces .... No cracks

[ ]  Fasteners .... Tight

[ ]  Position light

TAIL ROTOR

[ ]  Gearbox Telatemp .... Normal

[ ]  Gearbox .... Oil full, no leaks

[ ]  Blades .... Clean & no damage/cracks

[ ]  Rod ends .... Free without looseness

[ ]  Pitch link jam nuts .... Tight

[ ]  Teeter bearings

[ ]  Teeter bearing bolt .... Does not rotate

[ ]  Control bellcrank .... Free without looseness

TAILCONE

[ ]  Rivets .... Tight

[ ]  Skins .... No cracks or dents

[ ]  Strobe light condition

[ ]  Antenna

[ ]  Attachment bolts .... Tight

Engine - Left Side / Fuel Tank (Main) / Main Rotor

ENGINE - LEFT SIDE

[ ]  Engine sheet metal .... No cracks

[ ]  Exhaust system .... No cracks

[ ]  Engine oil .... 4-6 qts

[ ]  Fuel lines .... No leaks

[ ]  Gascolator drain .... Sample

[ ]  Throttle linkage .... Operable

[ ]  Battery & relay .... Secure

[ ]  Alternator belt tension

[ ]  Steel tube frame .... No cracks

[ ]  Engine general condition

FUEL TANK (MAIN)

[ ]  Leakage .... None

[ ]  Quantity

[ ]  Filler cap .... Tight

[ ]  Drain .... Sample

MAIN ROTOR

CAUTION: CAUTION – Do not pull rotor blades down as damage may occur. To lower one blade, push opposite blade up.

[ ]  Blades .... Clean & no damage/cracks

[ ]  Pitch change boots .... No leakage

[ ]  Main hinge bolts .... Cotter pins installed

[ ]  All rod ends .... Free without looseness

[ ]  Pitch link jam nuts .... Tight

[ ]  Pitch link safety wire .... Secure

[ ]  All fasteners .... Tight

[ ]  Swashplate scissors .... No excessive looseness

[ ]  Turn rotors 90° and level

Fuselage Left Side / Nose Section / Fuselage Right Side / Cabin Interior

FUSELAGE LEFT SIDE

[ ]  Baggage compartment

[ ]  Removable controls .... Secure if installed

[ ]  Collective control .... Clear

[ ]  Seat Belt .... Check condition & fastened

[ ]  Door .... Unlocked & latched

[ ]  Door hinge safety pin .... Installed

[ ]  Landing gear

[ ]  Ground handling wheel .... Removed

[ ]  Position light

NOSE SECTION

[ ]  Pitot tube .... Clear

[ ]  Windshield condition & cleanliness

[ ]  Fresh air vent .... Clear

[ ]  Landing lights

FUSELAGE RIGHT SIDE

[ ]  Landing gear

[ ]  Ground handling wheel .... Removed

[ ]  Position light

[ ]  Door hinge safety pin .... Installed

[ ]  Baggage compartment

CABIN INTERIOR

[ ]  Loose articles .... Removed or stowed

[ ]  Seat belts

[ ]  Instruments, switches, & controls

CAUTION: CAUTION – Removable controls should be removed if person in left seat is not a rated helicopter pilot.

CAUTION: CAUTION – When flying solo, fill left baggage compartment to capacity before using right baggage compartment. Avoid placing objects in compartments that might injure occupants if seat collapses during hard landing..

CAUTION: CAUTION – Shorter pilots may require cushion to obtain full travel of all controls. When using cushion, verify aft cyclic travel is not restricted.

NOTE: Notes: STAND BACK AND LOOK AT ENTIRE AREA!

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Robinson/R22/michael_smith_r22_preflight_checklist_ds.txt

Rights: licensed

Checklist hash: 1afad957dfe2cea89af990d7ae226dd37eedf0e83f3db0bbe181d7daf53a9723

Verify against the aircraft's own approved documentation before use.