Saab 2000 — MAX_Saab2000_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Approach Checklist" item count differs between passes (13 vs 15); verify nothing was dropped.

content_gap: Section "Landing Checklist" item count differs between passes (6 vs 16); verify nothing was dropped.

Pre-Start Checklist

[ ]  Parking Brakes .... SET

[ ]  Battery Switch .... OFF

[ ]  Alt Switch .... OFF

[ ]  Generator Switches .... OFF

[ ]  Throttle .... IDLE

[ ]  Landing Gear Lever .... CHECK DOWN

[ ]  Propeller .... HIGH RPM

[ ]  Fuel Cond. Lever .... FUEL CUTOFF (Ctrl-Shift-F1)

[ ]  Flaps .... UP

[ ]  Battery Switch .... ON

[ ]  Fuel Quantity .... CHECK

[ ]  Flight controls .... CHECK

[ ]  Check Weather .... (ATIS, Flight Services)

[ ]  De-Ice .... TEST/CHECK

[ ]  Request Clearance

[ ]  Transponder .... STANDBY

[ ]  Beacon .... ON

Startup Checklist

[ ]  Engine and Propeller Area .... CLEAR

[ ]  Throttle .... IDLE

[ ]  Fuel Flow .... ON (Ctrl-Shift-F4)

[ ]  Right Ignition and Engine Start Switch .... ON (hold until engine starts)

[ ]  Right ITT and Turbine PWR .... MONITOR

[ ]  Right Generator .... ON

[ ]  Left Ignition and Engine Start Switch .... ON (hold until engine starts)

[ ]  Left ITT and Turbine PWR .... MONITOR

[ ]  Left Generator .... ON

[ ]  Feather/Unfeather Props .... CHECK once

[ ]  Engine Instruments .... CHECK

Before Taxi Checklist

[ ]  Nav Lights .... ON

[ ]  Taxi Lights .... ON

[ ]  Heading Indicator/Altimeter .... SET

[ ]  Instruments .... NORMAL OPERATION

[ ]  Radios and Avionics .... CHECKED and SET

[ ]  Autopilot .... SET and OFF

[ ]  F/D .... ON

[ ]  Yaw Damper .... ON

[ ]  Request Taxi Clearance

Taxi Checklist

[ ]  Parking Brake .... RELEASE

[ ]  Brakes .... CHECK during taxi

[ ]  Directional Gyro .... PROPER IND. during turns

[ ]  Turn Coordinator .... PROPER IND. during turns

[ ]  Artificial Horizon .... ERECT during turns

[ ]  Taxi to assigned runway .... SPEED Max. 20 knots

Before Take-off Checklist

[ ]  Parking Brake .... SET

[ ]  Fuel Quantity .... CHECK

[ ]  Throttle .... IDLE

[ ]  Elevator Trim .... SET for takeoff

[ ]  Flaps .... Pos 1

[ ]  Flight Controls .... FREE AND CORRECT

[ ]  Engine/Flight Instruments .... CHECK

[ ]  Radios and Avionics .... SET

[ ]  Landing Lights .... ON

[ ]  Strobe Light .... ON

[ ]  Pitot Heat .... ON

[ ]  De-Ice .... AS REQUIRED

[ ]  Annunciator Lights .... CHECK

[ ]  Transponder .... ON

[ ]  Request Takeoff Clearance

Take-off Checklist

[ ]  Smoothly increase thrust to .... FULL

[ ]  Brakes .... RELEASE

[ ]  V1 = Vr = .... 110 KIAS

[ ]  Pitch .... 8-10 degrees

[ ]  V2 = .... 120 KIAS

[ ]  At Positive Climb Rate Touch Brakes

[ ]  Landing Gear .... RETRACT

[ ]  Flaps .... RETRACT

[ ]  Annunciator Lights/Eng. Instruments .... CHECK

Climb-out Checklist

[ ]  Autopilot .... CHECK and SET

[ ]  Taxi Lights .... OFF

[ ]  Landing Lights .... OFF

[ ]  Airspeed .... 205 KIAS / 1800 fpm 85%

[ ]  above 12,000 ft .... 200 KIAS / 1700 fpm 85%

[ ]  above 18,000 ft .... 180 KIAS / 1500 fpm 80%

[ ]  Engine Instruments .... MONITOR

[ ]  ATC .... AS REQUIRED

[ ]  At Transition Altitude (FL180) set Altimeter to .... 29,92" (1013mb)

Cruise Checklist

[ ]  Accelerate to cruise speed at 6,000 ft .... 240 KIAS 80%

[ ]  at 9,000 ft .... 240 KIAS 80%

[ ]  at 12,000 ft .... 230 KIAS 80%

[ ]  at 18,000 ft .... 220 KIAS 80%

[ ]  at 24,000 ft .... 220 KIAS 80%

[ ]  Service Ceiling .... FL310

[ ]  Engine+Instruments .... CHECK

[ ]  Fuel Quantity .... CHECK

[ ]  Radios .... TUNED and SET

[ ]  Autopilot .... CHECK and SET

[ ]  Lights .... AS REQUIRED

Descent Checklist

[ ]  Atis/Airport Information .... CHECK

[ ]  Altimeter .... CHECK

[ ]  Radios .... SET

[ ]  Descent Speed .... 240 KIAS 60%

[ ]  Descent Rate .... -1800 fpm

[ ]  Flaps / Landing Gear .... CHECK UP

[ ]  At Transition Altitude (FL180) reset Altimeter to local

[ ]  Check Weather .... (ATIS, Flight Services)

Approach Checklist

[ ]  Localizer Level Flight : Landing Lights .... ON

[ ]  Speed: Establish .... 180 KIAS 37%

[ ]  Flaps .... Pos 1

[ ]  Speed: Establish .... 160 KIAS 40%

[ ]  Flaps .... Pos 2

[ ]  Speed: Establish .... 140 KIAS 37%

[ ]  Landing Gear .... DOWN

[ ]  Turning toward runway: set flaps .... Pos 3 or FULL DOWN

[ ]  Final Glideslope Descent : Speed: Establish .... 130 KIAS 18%

[ ]  Descent Rate .... -xxx fpm

[ ]  Elevator Trim .... AS DESIRED

[ ]  Parking Brake .... VERIFY OFF

[ ]  De-Ice .... AS REQUIRED

Landing Checklist

[ ]  Landing Gear .... CHECK DOWN, 3 GREEN

[ ]  Autopilot .... OFF

[ ]  Landing Speed .... 120 KIAS

[ ]  Touchdown .... MAIN WHEELS FIRST

[ ]  Landing Roll .... LOWER NOSE WHEEL

[ ]  After touchdown Apply Reverse Thrust, at 60 kts: Cancel Reverse Thrust

Taxi To Ramp

[ ]  Strobe Light .... OFF

[ ]  Taxi Lights .... ON

[ ]  Landing Lights .... OFF

[ ]  Flaps .... RETRACT

[ ]  De-Ice .... OFF

[ ]  Transponder .... 1200

[ ]  Speed .... Max. 20 knots

[ ]  Elevator Trim .... TAKEOFF SETTING

[ ]  Avionics/Radios .... AS REQUIRED

Shutdown Checklist

[ ]  Parking Brake .... SET

[ ]  Throttle .... IDLE

[ ]  F/D .... OFF

[ ]  Taxi Lights .... OFF

[ ]  Pitot Heat .... OFF

[ ]  Nav Lights .... OFF

[ ]  Fuel Cond. Lever .... FUEL CUTOFF (Ctrl-Shift-F1)

[ ]  Beacon .... OFF

[ ]  Generators .... OFF

[ ]  Battery Switch .... OFF

Securing Aircraft

[ ]  Parking Brake .... Verify SET

[ ]  Throttle .... Verify IDLE

[ ]  All Switches .... Verify OFF

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Saab/2000/MAX_Saab2000_Checklist.txt

Rights: licensed

Checklist hash: ca100c5cd8cd7d324c155d4cc5dc6141cff5c2c5bfbbc8178bd51986ed9673d7

Verify against the aircraft's own approved documentation before use.