Saab 340 — MAX_Fs2ks340_Check

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

ENGINE STARTUP

[ ]  Park Brakes .... SET

[ ]  Battery switch .... ON

[ ]  Strobe and Beacon Lights .... ON

[ ]  Click on automatic startup engine button or CTRL+E .... PERFORM

AFTER ENGINE STARTUP

[ ]  Startup Engine Time .... LOGGED

[ ]  Engine power .... IDLE

[ ]  Low Idle .... CHECKED

[ ]  Fuel/Weight .... CHECKED AND LOGGED

[ ]  Avionic Master .... ON

[ ]  ATIS .... LOGGED

[ ]  Navaids .... SET

[ ]  AP Settings .... SET

[ ]  AP MAIN .... OFF

[ ]  Pitot Heat .... ON

[ ]  NAV Lights .... AS NEEDED

BEFORE TAKEOFF

[ ]  Flaps .... 7° (1ST POSITION)

[ ]  Prop RPM .... HIGH

[ ]  Trim position .... CHECKED

[ ]  AP MAIN and Flight Director .... ARMED

[ ]  Anti-Ice .... AS NEEDED

[ ]  Altimeters .... SET

[ ]  Landing Lights .... ON

[ ]  Transponder .... CODE SET

TAKEOFF AND CLIMB

[ ]  Thrust .... MAX POWER OR 95%

[ ]  Release Brakes

[ ]  Engine parameters .... CHECK

[ ]  Rotate at MTOW .... 115 KTS

[ ]  Pitch attitude after T/O .... 10°

[ ]  Retract landing gear with positive ROC

[ ]  At 500 ft and 130 kts IAS — Flaps .... UP

[ ]  Let accelerate to 160 kts IAS

[ ]  Reduce TORQUE .... 90-95%

[ ]  Engage AP with an initial ROC .... 2000 FPM

[ ]  Maintain until cruise altitude .... 160 KTS IAS AND 1500-2500 FPM ROC

CRUISE

[ ]  Reduce Prop RPM (ctrl-F2) .... 1650 RPM

[ ]  Reduce TORQUE .... 75-80%

NOTE: Usual cruise altitude is FL150-FL200

NOTE: Standard cruise speed is 280 kts TAS

DESCENT

[ ]  Establish a descent rate .... 1,500 FPM

[ ]  Maintain during descent .... 240 KTS IAS

[ ]  Landing Lights below 10,000 ft .... ON

APPROACH AND LANDING

[ ]  PROP RPM (ctrl-F4) .... HIGH

[ ]  Level approach flight (20-25 NM) clean configuration .... 210 KTS

[ ]  Glide slope alive — reduce speed and set Flaps 1 (7°) .... 170 KTS

[ ]  Passing the OM or 1500 ft — Gear Down, Flaps 3 (20°), Speed (Vref+5) .... 125 KTS

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Saab/340/MAX_Fs2ks340_Check.txt

Rights: licensed

Checklist hash: f9d76bec5c5a618689519676ef64af13273966c9fb0870384b12c2629f566bed

Verify against the aircraft's own approved documentation before use.