Schweizer SGS 2-33 — Eric_and_Ann_sgs_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

SGS 2-33 Flight Card

[ ]  Vne=98  Va=65

[ ]  W=51' L=30' H=10' ASP=18

[ ]  Slip=45-50

[ ]  G=20:1

[ ]  MPH

[ ]  Load limits -2.56<g<4.67

[ ]  Dual           0º

[ ]  Solo            0º

[ ]  45º      60º

[ ]  45º      60º

[ ]  L/D           50

[ ]  L/D            45       54

[ ]  60       70

[ ]  64

[ ]  M/Sink     42       50

[ ]  M/Sink      38       45

[ ]  60

[ ]  54

[ ]  Stall*        33

[ ]  Stall*         31       37

[ ]  39       47

[ ]  44

TAKEOFF

LAND

[ ]  Altimeter + radio .... SET

[ ]  Undercarriage .... CHECK

[ ]  Belts  + passenger brief .... BRIEFED

[ ]  Speed    50(47solo) + ½ w .... CHECK

[ ]  Control     trim off + Xwind .... CHECK

[ ]  Trim .... CHECK

[ ]  Cable .... CHECK

[ ]  Airbrakes .... CHECK

[ ]  Canopy .... CHECK

[ ]  Look out!    radio call .... PERFORM

[ ]  Dive Brakes .... CHECK

[ ]  Landing .... PERFORM

[ ]  I P-

[ ]  Direction of Wind .... CHECK

[ ]  1000'-800'

[ ]  Emergency Takeoff. forward to right .... PERFORM

[ ]  Abeam T P-     600'-500'

[ ]  200< forward to right .... PERFORM

[ ]  Base-

[ ]  200> 50mph 45ºbank .... PERFORM

[ ]  500'-400'

[ ]  180º turn to runway .... PERFORM

[ ]  Final-

[ ]  300'-200'

[ ]  Flair 5'-3'

[ ]  Vref 43 (41solo)

[ ]  Slack .... Yaw Away!

[ ]  Release rope .... Clear R & L  90ºR turn + check

[ ]  Stall* .... Test speed into and with wind

[ ]  Spin .... Neutral stick, top rudder, nose down

[ ]  Landing Stall .... Top rudder! Nose Down! Quickly!

[ ]  Speed to Fly     L/D + ½ HeadW        TailW   L/D

[ ]  ↔ M/S

WEIGHT  ARM  MOMENT

[ ]  SGS 2-33      612          x     96.12    58825

[ ]  F Pilot           _______   x     43.8    _________

[ ]  R Pilot          _______   x    74.7    _________

[ ]  Ballast          19.5 (op.)  x    14.7    286.65

[ ]  Total             _______   __________/W

[ ]  Max            1040lbs    CG 78.2↔86.1

Preflight

[ ]  N1178S

PREFLIGHT PREP:

[ ]  Open canopy pull tow hook release .... RELEASE

[ ]  Canopy Closed .... CHECK

[ ]  Remove Aileron locks .... REMOVE

[ ]  Lower Tail and retie .... LOWER/RETIE

[ ]  Remove rudder lock .... REMOVE

[ ]  Untie wing tips .... UNTIE

FRONT COCKPIT:

[ ]  Canopy - Condition & Secure .... CHECK

[ ]  ARROW .... CHECK

[ ]  Instruments and radio .... CHECK

[ ]  Controls - Free and correct .... CHECK

[ ]  Spoilers - check and leave open .... CHECK/OPEN

[ ]  Trim - operational .... CHECK

[ ]  Cables & Bolts - secure .... CHECK

[ ]  Belts and Harness .... CHECK

[ ]  Rudder Pedals - Adjust/check .... ADJUST/CHECK

[ ]  Ballast - Secure or remove .... SECURE/REMOVE

[ ]  Canopy Closed & Locked .... CLOSED/LOCKED

REAR COCKPIT + FUSELAGE INTERIOR:

[ ]  Canopy/Door - Condition .... CHECK

[ ]  Seat Cushions (Secure for solo) .... SECURE

[ ]  Belts & Harness .... CHECK

[ ]  Loose Equipment - Remove/Stow .... REMOVE/STOW

[ ]  Cables & Bolts .... CHECK

[ ]  Wing Connections .... CHECK

[ ]  Control Connections .... CHECK

[ ]  Cotter Pins - 6 .... CHECK

[ ]  Linkage .... CHECK

[ ]  Canopy Closed & Locked .... CLOSED/LOCKED

RIGHT WING:

[ ]  Struts & Bolts & Pins .... CHECK

[ ]  Leading Edge .... CHECK

[ ]  Wing Surface .... CHECK

[ ]  Outrigger Wheel .... CHECK

[ ]  Aileron .... CHECK

[ ]  Spoiler .... CHECK

[ ]  Trailing Edge .... CHECK

TAIL:

[ ]  Right Fuselage Skin .... CHECK

[ ]  Vertical Stabilizer .... CHECK

[ ]  Rudder and Hinge pins .... CHECK

[ ]  Horizontal Stabilizer .... CHECK

[ ]  Elevator and Hinge pins .... CHECK

[ ]  Control Cables/Push Rods .... CHECK

[ ]  Untie tail .... UNTIE

[ ]  Tail Fairing & Wheel .... CHECK

[ ]  Inspection Plates .... CHECK

[ ]  Left Fuselage Skin .... CHECK

LEFT WING:

[ ]  Left Wing: Repeat as per right wing in reverse 7-1 .... CHECK

NOSE SECTION:

[ ]  Pitot Static System .... CHECK

[ ]  Static Port - Clear .... CHECK

[ ]  Yaw String .... CHECK

[ ]  Tow hook release, skid, and wheel .... CHECK

[ ]  Fabric/Glass .... CHECK

WALK AROUND FINAL CHECK

[ ]  IMSAFE + Passenger Brief .... PERFORM

[ ]  Untie remaining ropes and move glider to holding area .... PERFORM

TOW PLANE/CREW:

[ ]  Instructions, Altitude & Direction .... BRIEFED

[ ]  Signals and Radio checks .... CHECK

[ ]  Check Tow Rope .... CHECK

RUNWAY:

[ ]  Traffic check and radio call .... PERFORM

[ ]  Position tow plane and glider for tow .... POSITION

[ ]  Tow release checks .... CHECK

[ ]  ABCs .... CHECK

NOTE: Use of this check list is solely at "Users risk". Authors assume no liability for any information thus contained.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Schweizer/SGS 2-33/Eric_and_Ann_sgs_checklist.txt

Rights: licensed

Checklist hash: 17331d49f1092b900cf2b83a48c3d73751de07fa70c4ef7b22c7d9f1c84a8023

Verify against the aircraft's own approved documentation before use.