Schweizer SGS 2-33A — Normal Procedures and Launch Emergencies

NOT FOR FLIGHT — source fidelity: not_applicable | operational review: none | completeness: partial | 1 unresolved safety defect(s)

safety_defect: No airspeeds or weight limits. Club and site procedures for signals, release points and pattern entry vary and override anything here.

content_gap: Aerotow only. No winch or auto-tow launch procedures, and no spin recovery section.

Daily and Preflight Inspection

[ ]  Canopy and latches .... CONDITION, SECURE

[ ]  Controls .... FREE, FULL TRAVEL, CORRECT SENSE

[ ]  Control connections and safeties .... CHECKED

[ ]  Spoilers .... OPERATE, LOCK CLOSED

[ ]  Tow release .... OPERATES AND RESETS

[ ]  Wings, fuselage, tail surfaces .... NO DAMAGE, FABRIC INTACT

[ ]  Tailwheel or skid, main wheel and tire .... CONDITION

[ ]  Pitot and static .... UNOBSTRUCTED

[ ]  Ballast and placards .... CORRECT FOR OCCUPANTS

Pre-Launch

[ ]  Straps .... TIGHT, ALL OCCUPANTS

[ ]  Instruments .... ALTIMETER SET, ZEROED AS REQUIRED

[ ]  Controls .... FREE, CORRECT

[ ]  Canopy .... CLOSED AND LOCKED

[ ]  Trim .... SET

[ ]  Spoilers .... CLOSED AND LOCKED

[ ]  Tow rope and weak link .... CORRECT, ATTACHED

[ ]  Wind and traffic .... CHECKED

[ ]  Launch emergency plan .... BRIEFED ALOUD

State the plan for a rope break below 200 feet, between 200 feet and pattern altitude, and above pattern altitude, and the landing option for each.

[ ]  Wing runner and tow pilot .... READY, SIGNAL GIVEN

Rope Break Below 200 Feet  [EMERGENCY]

Tow rope breaks or tow is released below approximately 200 feet AGL

WARNING: Do not attempt a turn back to the runway from low altitude. Land ahead, accepting whatever is in front of the glider.

[ ]  Nose [MEMORY] .... LOWER TO APPROACH ATTITUDE

[ ]  Tow release [MEMORY] .... PULL, TWICE

[ ]  Spoilers [MEMORY] .... AS REQUIRED

[ ]  Land straight ahead

Rope Break At or Above Pattern Altitude  [ABNORMAL]

Tow failure with sufficient height to fly a pattern

[ ]  Tow release [MEMORY] .... PULL, TWICE

[ ]  Airspeed .... ESTABLISH BEST GLIDE

[ ]  Landing area .... SELECTED

[ ]  Pattern .... FLOWN, ADJUSTED FOR HEIGHT

GO TO: Continue with the normal approach and landing checklist.

Approach and Landing

[ ]  Straps .... TIGHT

[ ]  Landing area .... CONFIRMED CLEAR

[ ]  Airspeed .... APPROACH SPEED SET FOR CONDITIONS

[ ]  Spoilers .... USED TO CONTROL GLIDE PATH

NOTE: Plan to land long rather than short. Undershooting is not recoverable in a glider.

After Landing

[ ]  Maintain wings level with aileron until the glider stops

[ ]  Spoilers .... OPEN

[ ]  Canopy .... AS REQUIRED

[ ]  Clear the runway and report clear

NOT FOR FLIGHT — source fidelity: not_applicable | operational review: none | completeness: partial | 1 unresolved safety defect(s)

Source: authored — no source document

Rights: original_expression CC-BY-4.0

Checklist hash: 5afe01fd8549bcfcf8b46b64dc02d841954b06c295936b228a03151bd157ca2f

Verify against the aircraft's own approved documentation before use.