Socata TB10 Tobago — Richard_Haines_Socata_tb10_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "11. Top of Descent Checks" item count differs between passes (8 vs 9); verify nothing was dropped.

content_gap: Section "12. D/wind Chk (BUMFFICHIW)" item count differs between passes (11 vs 12); verify nothing was dropped.

content_gap: Section "2. Before Start Checklist" item count differs between passes (3 vs 8); verify nothing was dropped.

content_gap: Section "6. Power/Run up Checklist" item count differs between passes (13 vs 14); verify nothing was dropped.

content_gap: Section "8. Take Off Checks" item count differs between passes (9 vs 10); verify nothing was dropped.

1. Cockpit checklist

[ ]  A/C documents .... ON-BOARD/CHKD

[ ]  Preflight /Logbook .... COMPLETED

[ ]  Ice/Weather .... REMOVED/CHECKED

[ ]  Control Lock/Tie dwn .... REMOVED

[ ]  Passenger Breifing .... GIVEN

[ ]  Doors/Seats/Belts .... LOCKED/CHKD

[ ]  All Switches/Radio .... OFF

[ ]  Brakes(park/foot) .... SET/ON

[ ]  Cockpit Checks Completed

2. Before Start Checklist

[ ]  Controls .... CHKD FULL & FREE

[ ]  Master Switch .... ON/CHECK HOBS

[ ]  Fuses .... CHKD IN

2a. Start up Clearance only

[ ]  Radio/Tower Freq .... ON/SET/CONTACT

[ ]  Start up Clearance .... RECEIVED

[ ]  Radio Master .... OFF

[ ]  Before Start Checks Complete

3. Starting Engine(master on)(COLD)

[ ]  Carb Heat .... COLD

[ ]  Propeller(blue) .... FWD

[ ]  Mixture .... RICH

[ ]  Fuel Tank(front panel) .... LOWEST TANK

[ ]  Fuel Pump(butt 1) .... ON/CHK PRESS

[ ]  Throttle Friction .... SET

[ ]  Throttle .... 2 PUMPS 1/4 OPEN

[ ]  Before Start .... CLEAR ALL ROUND

[ ]  Magnetos/Starter .... BOTH/ENGAGE

[ ]  Throttle .... 1000RPM(warm)

[ ]  Oil Pressure .... CHKD

[ ]  Engine Temp .... RISING

[ ]  Fuel Pump(butt 1) .... OFF

[ ]  Flaps .... UP

3a. Starting Engine (HOT)

[ ]  As above but .... NO Throttle pump

[ ]  Starting Engine Complete

4. After Start Checklist

[ ]  Generator(G) .... ON/CHKD

[ ]  Turn coordin(butt 3) .... ON

[ ]  Beacon/Nav lights .... ON

[ ]  Magnetos .... CHECK QUICKLY

[ ]  Ammeter/Suction .... CHGING/REGIST

[ ]  Compass/DI .... SET X-CHKD

[ ]  Radio nav/Autopilot .... SET/TEST/CHKD

[ ]  Radios/Tower .... ON/SET/CONTACT

[ ]  Altimeters .... SET QFE/QNH

[ ]  Transponder .... 7000 ON SBY

[ ]  CHECK TIME BRAKES OFF & NOTE

[ ]  FLIGHT PLAN OPENED ????

[ ]  After Start Checks complete

5. Taxi Checklist

[ ]  Control Column .... FULL BACK

[ ]  Brakes/rudder .... CHKD

[ ]  Turn indic/bal ball .... CHKD

[ ]  DI/Compass/AI .... CHKD

[ ]  Taxi check complete

6. Power/Run up Checklist

[ ]  Wheels/Action .... STRAIGHT/ LOOKOUT

[ ]  Brakes(foot/pking) .... FULL/HOLDING

[ ]  Engine Temp/Press .... GREEN ARC

[ ]  Fuel .... PUMP ON / FULLEST TANK

[ ]  Throttle .... 2000RPM

[ ]  Carb Heat .... CHECK

[ ]  Magnetos .... CHECK(175/50)

[ ]  Prop(Blue) .... BACK ONCE OIL

[ ]  Prop recycle .... BACK AGAIN CHECK

[ ]  Prop(controlling) .... CHECK WITH THROTTLE

[ ]  Suction/Ammeter .... CHECK

[ ]  Throttle .... IDLE CUT OUT CHECK THEN 1000RPM

[ ]  Power/Run up Check Completed

7. Before T/Off Checklist Vital Actions

[ ]  Trimmers .... SET FOR TAKE OFF

[ ]  Throttle Friction .... SET

[ ]  Mixture/Engine T&Ps .... RICH / WITHIN LIMITS

[ ]  Master/Magnetos .... MASTER ON/MAGS ON BOTH

[ ]  Pitot Heater .... AS REQUIRED

[ ]  Propeller .... FWD/FINE

[ ]  Fuel Pump .... ON

[ ]  Fuel .... FULLEST TANK

[ ]  Flaps .... T/O 10DEGS

[ ]  Instruments .... CHKD SET INC GPS

[ ]  Hatches/Harnesses .... CHKD SECURE

[ ]  Carb Heat .... RECHECK / COLD

[ ]  Controls .... FULL & FREE MOVEMENT

[ ]  Lights .... ALL ON

[ ]  Transponder .... 7000 ON ACS

[ ]  Radio Nav .... CHKD

[ ]  ATC Clearance .... RECEIVED

[ ]  Before T/Off Checks Complete

8. Take Off Checks

[ ]  Line up .... RWAY CLEAR / LOOKOUT

[ ]  Compass/DI .... X-CHKD RWAY HDING

[ ]  Brakes .... OFF

[ ]  Throttle .... HALF POWER

[ ]  Engine Temp/Press .... STEADY WITHIN LIMITS

[ ]  Throttle .... FULL POWER IF ABOVE OK

[ ]  Airspeed .... INCREASING

[ ]  Rotate .... FRONT WHEEL LIFT 50KNTS THEN MIN 65KNTS ROTATE

[ ]  Take Off Checks Complete

9. After T/Off Checklist(FLEETPI)>300'

[ ]  Flaps/Landing light .... UP / LANDING LIGHT OFF

[ ]  Engine Temp/Press .... STEADY WITHIN LIMITS

[ ]  Throttle/Prop .... 25"/2500RPM THROTTLE 1ST

[ ]  Attitude .... FOR BEST CLIMB 78KNTS

[ ]  Trimmer .... AS REQUIRED

[ ]  Altimeters .... CHKD SET QNH/QFE

[ ]  Fuel Pump .... OFF AT 1000' MIN CHK PRESS

[ ]  Icing .... CHECK SURFACES

[ ]  After T/Off Checks Complete

10. Top of Climb /Cruise Check

[ ]  Altitude .... SET / MAINTAIN

[ ]  Power/Prop .... ADJUST AS BELOW

[ ]  Trimmer .... AS REQUIRED

[ ]  Fuel .... CHANGE 15MINS

[ ]  Fuel Pump .... ON AS CHANGING

[ ]  Mixture .... ADJUST (EGTT)

NOTE: MAX 75% >1000'

[ ]  Engine .... T&Ps WITHIN LIMITS

[ ]  Instruments .... SET AS REQUIRED

[ ]  Compass/DI .... CHECK REGULARLY

[ ]  Icing .... CHECK SURFACES

[ ]  Weather .... CHECK WITH ATC

[ ]  Top of climb checks complete

11. Top of Descent Checks

[ ]  Radio / ATC .... FREQS / CONTACT

[ ]  Engine .... T&Ps WITHIN LIMITS

[ ]  Altimeter .... SET

[ ]  Radionav .... CHKD / CONFIRM

[ ]  Instrument Plates .... CHKD / CONFIRM

[ ]  Carb Heat .... HOT IF DESCENDG

[ ]  Power .... REDUCE THROTTLE TO APPROX 20"

[ ]  Top of Descent checks complete

12. D/wind Chk (BUMFFICHIW)

[ ]  ATC / Brakes .... CONTACT / OFF

[ ]  U/carriage / Speed .... FIXED / 100knots

[ ]  Master / Mags .... ON / BOTH

[ ]  Fuel pump .... ON/CHK PRESS

[ ]  Fuel Tanks .... FULLEST TANK

[ ]  Instruments .... QFE/NAV/T&Ps

[ ]  Carb heat .... CHKD

[ ]  Harnesses/Hatches .... SECURE/CLOSED

[ ]  Icing .... CHECK SURFACES

[ ]  Weather .... CHKD DIVERT IF REQUIRED

[ ]  Downwind Chk complete

13. Baseleg Checklist

[ ]  Throttle .... APPROX 15" MP

[ ]  Carb Heat .... HOT

[ ]  Flaps .... 10DEGS ONCE VFE(<95KNTS)

[ ]  Altitude .... IMC MINIMA

[ ]  Approach speed .... MIN 70 KNOTS

[ ]  Baseleg check complete

14. Finals Checks

[ ]  Landing Clearance .... RECEIVED

[ ]  Ldg Lights(whites) .... ON

[ ]  Mixture(red) .... RICH

[ ]  Propeller(Blues) .... FULLY FWD / FINE

[ ]  Throttle (black) .... SET FOR APPROACH SPEED

[ ]  Undercarriage(green) .... FIXED

[ ]  Fuel .... PUMP ON FULLEST TANK

[ ]  Carb Heat .... COLD @ 300FT

[ ]  Flaps .... FULL OVER THRESHOLD

[ ]  Finals Check Complete

15. After Landing Chks(once clear r/way)

[ ]  Carb Heat .... COLD

[ ]  Transponder .... OFF

[ ]  Fuel Pump/Pitot Ht .... OFF

[ ]  Landing Light .... OFF

[ ]  Flaps/Fuel Tanks .... UP/REFILL TO TABS

[ ]  After landing checks complete

16. Parking Checklist

[ ]  Brakes .... SET/Parking Brake ON

[ ]  CHECK TIME BRAKES ON & NOTE

[ ]  Radio Master .... OFF

[ ]  Throttle .... SET 1000 FOR 30 SECS

[ ]  Magnetos .... CHECK QUICKLY

[ ]  Throttle .... CLOSED

[ ]  Mixture .... CUT OFF

[ ]  All Switches/Fuel .... OFF / FUEL OFF SET CENTRE

[ ]  Master Switch .... OFF / CHECK HOBS

[ ]  Magnetos/Logbook .... OFF KEY OUT / COMPLETED

[ ]  Control Lock/Covers .... APPLIED / FIXED & SECURED

[ ]  Parking Checks completed

Notes and Performance Data

NOTE: NOTE:- CHECK TIME BRAKES OFF & ON & NOTE ON LOGSHEETS NOT HOBBS

NOTE: FLIGHT PLAN CLOSED ???

NOTE: FUEL (approx. 53US gallons usble. OIL-QTY-6-8Litres)

NOTE: CRUISE POWER SETTINGS — 2000ft 11oC MP IN 25/ 11.1 25/ 11.5 23.6/ 11 23.6/ 11.7 22.1/ 11.4; in.Hg/ 23.6/ 10.1 23.6/ 10.5 22.1/ 10 22.1/ 10.6 20.7/ 10.4; FUEL/HR 22.1/ 9.2 22.1/ 9.6 20.7/ 9.1 20.7/ 9.6 19.2/ 9.4; 20.7/ 8.4 20.7/ 8.7 19.2/ 8.4 19.2/ 8.8

NOTE: CRUISE POWER SETTINGS — 4000ft 7oC MP IN 23.6/ 10.4 23.6/ 10.9 23.6/ 11.4 23.6/ 12.1 22.1/ 11.8; in.Hg/ 22.1/ 9.6 22.1/ 9.9 22.1/ 10.3 22.1/ 11 20.7/ 10.7; FUEL/HR 20.7/ 8.7 20.7/ 9 20.7/ 9.7 20.7/ 9.9 19.2/ 9.7; 19.2/ 8.3 19.2/ 8.5 19.2/ 9.1

NOTE: CRUISE POWER SETTINGS — 6000ft 3oC MP IN 22.1/ 9.9 22.1/ 10.7 22.1/ 10.7 22.1/ 11.3 22.1/ 12.2; in.Hg/ 20.7/ 8.9 20.7/ 9.3 20.7/ 9.7 20.7/ 10.3 20.7/ 11.1; FUEL/HR 19.2/ 8.2 19.2/ 8.5 19.2/ 8.9 19.2/ 9.3 19.2/ 9.9

NOTE: CRUISE POWER SETTINGS — 8000ft -1oC MP IN 20.7/ 9.2 20.7/ 9.6 20.7/ 10 20.7/ 10.6 20.7/ 11.4; in.Hg/ 19.2/ 8.5 19.2/ 8.7 19.2/ 9.1 19.2/ 9.6 19.2/ 10.2; FUEL/HR 17.7/ 8.7 17.7/ 9.3

NOTE: CRUISE POWER SETTINGS — 10000ft -5oC MP IN 19.2/ 8.7 19.2/ 9 19.2/ 9.4 19.2/ 9.9 19.2/ 10.6; in.Hg/FB 17.7/ 9.5

NOTE: BOLD recommend figures head wind; BOLD recommend figures tail wind

NOTE: WEIGHTS — GROSS 2535Lbs, EMPTY 1500Lbs

NOTE: SPEEDS — KIAS / MPH: Vr rotation 65-75 / 75-90; Vy Best ROC 73 / 84; Vy 4up Best ROC 80 / 92; Vx Best AOC 67 / 78; Vfe Max Flaps 95 / 109; Va Cruise 115-122 / 140; Vno Struc CR 128 / 147; Vne Never exc 165 / 190; Ldng Normal 72-81 / 83-93; Ldng Soft/short 70 / 80; Glide BEST 86 / 99

NOTE: Add/decrease MP by 0.5 as temp varies 10°C above/below standard temp in table above

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Socata/TB10 Tobago/Richard_Haines_Socata_tb10_checklist.txt

Rights: licensed

Checklist hash: b2338c8380b104eef758893b341942f8d9d1c8c4912d80eb6069a7d97ae6dc60

Verify against the aircraft's own approved documentation before use.