Yakovlev Yak-12M — YAK_12M_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

External

[ ]  Kick the tyres, nothing about to fall off etc.

[ ]  Oil level > 8 litres.

[ ]  Fuel levels OK.

[ ]  Turn engine 9 blades; watch for the oil.

Internal / starting

[ ]  All switches off, mags off etc.

[ ]  Fuel OK.

[ ]  Air on.

[ ]  Brakes on.

[ ]  Fuel on. .... ON

[ ]  Mixture rich. .... RICH

[ ]  Close oil cooler.

[ ]  Close carb heat.

[ ]  Open Gills.

[ ]  Flaps up.

FOR COLD START;

[ ]  If you have an assistant, 9 blades, 1 prime each.

[ ]  Otherwise, 8 primes, suck in, then 2 more.

FOR WARM START,

[ ]  1 prime per 10° below 100 cylinder head temp.

[ ]  Master switch on.

[ ]  Generator on.

[ ]  DC converter.

[ ]  Starter on.

[ ]  Set throttle.

[ ]  Press starter, when engine turns correct way, mags on both.

[ ]  When engine fires, set throttle and prime as necessary.

[ ]  Set 1000 RPM. .... 1000 RPM

[ ]  Close Gills until cylinder head temp rises, then open as necessary.

[ ]  Open oil cooler as necessary.

Before Taxi

[ ]  Switches on as necessay.

[ ]  Radio on and set.

[ ]  Set DI to compass

[ ]  Set altimeter.

[ ]  Uncage artificial horizon.

[ ]  Do not taxi until oil temp in green.

Taxi

NOTE: Note; tail wheel unlocked when stick forward.

[ ]  Check brakes and steering.

[ ]  Check pneumatic pressure maintained.

[ ]  Check turn and slip move correctly.

[ ]  Check DI moves correctly.

Engine checks

[ ]  Oil temp in green.

[ ]  Cylinder head temp in green.

[ ]  Set 2000 RPM .... 2000 RPM

[ ]  Check mags.

[ ]  Exercise prop.

[ ]  Check carb heat.

[ ]  Set idle; engine continues to run.

[ ]  Set 1000 RPM. .... 1000 RPM

Pre take off vital checks

[ ]  Mags on both.

[ ]  Trim set.

[ ]  Flaps to 15° .... 15°

[ ]  Fuel on and sufficient.

[ ]  Mixture fully rich.

[ ]  Starter covered.

[ ]  Primer locked.

[ ]  Prop fully fine.

[ ]  Oil temp green.

[ ]  Cylinder head temp green.

[ ]  Set DI to compass.

[ ]  Switches on as necessary.

[ ]  Controls full and free.

[ ]  Hatches and harness’ secure.

Take off

[ ]  Set full power, then back to 7.5 boost.

[ ]  Set prop to 2000 RPM. .... 2000 RPM

[ ]  Take off with tail up or 3 point.

Climb

[ ]  boost, 2000 RPM.

[ ]  Flaps up above 300 feet.

Cruise

[ ]  boost, 1860 RPM, no flaps.

Descent

[ ]  Reduce throttle.

[ ]  Flaps extend below 140kmh

[ ]  Final approach 110kmh, Prop to fine

NOTE: Note land with stick back to ensure tail wheel locked - risk of ground loops.

Close down

[ ]  Brakes on.

[ ]  Set 1000 RPM .... 1000 RPM

[ ]  Check mags.

[ ]  Flaps up.

[ ]  Close carb heat.

[ ]  Unnecessary electrics off.

[ ]  Set idle.

[ ]  Mags off.

[ ]  Open gill flaps.

[ ]  Electrics off.

[ ]  Close pneumatic valve.

[ ]  Stick free and brakes off for manoeuvring.

[ ]  Drain water trap

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Yakovlev/Yak-12M/YAK_12M_checklist.txt

Rights: licensed

Checklist hash: a8014c5a64be2cc0348f752afc44b02dcd58b9738e8ef998f99b00046934bc12

Verify against the aircraft's own approved documentation before use.