Yakovlev Yak 52 — yak52_fltrep52

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Circuit and Approach" item count differs between passes (5 vs 4); verify nothing was dropped.

content_gap: Section "Engine Start" item count differs between passes (3 vs 4); verify nothing was dropped.

content_gap: Section "Landing" item count differs between passes (4 vs 3); verify nothing was dropped.

Cockpit Entry

[ ]  Climb onto wing using knee, then other leg, while holding canopy rail

[ ]  Walk only on anti-skid strip adjacent to fuselage

Before Start

[ ]  Check front cockpit wall chart for priming duration

[ ]  Check air reservoir pressure

NOTE: Air reservoir is good for 12 start attempts before re-charging.

Engine Start

[ ]  Start using pneumatic system

NOTE: Starting procedure varies with outside air temperature, time since last run, and oil dilution.

NOTE: When warm, the Ivchenko starts with the simplicity of a typical Continental or Lycoming.

After Start

[ ]  Check compressor re-charges air bottle after start

Taxi

[ ]  Use rudder bars for gentle turns

[ ]  Use burst of power to energize rudder when needed

[ ]  For tighter turns, use full rudder plus stick-mounted brake lever for differential braking

NOTE: With rudders neutral, braking is equal to both main wheels.

CAUTION: Take care not to reach excessive taxi speed; allow engine temperature to stabilize.

Engine Run-Up and Vital Actions

[ ]  Magnetos check

[ ]  Exercise RPM control to verify propeller governor operation

[ ]  Complete pre-takeoff vital actions using checklist

NOTE: Max power (ceaseless engine function duration) is 5 minutes.

NOTE: Throttle-engine lag (racing capacity) is 3 seconds max.

NOTE: Max overspeed when operating throttle rapidly is 109 per cent.

NOTE: Maximum allowed airspeed in level flight is conditioned by avoidance of sudden spin: 130 km/h normal flight, 170 km/h inverted. Author suspects max/min are reversed.

CAUTION: Aerobatics forbidden with less than 4.5 gallons of fuel remaining.

Takeoff

[ ]  Apply full throttle

[ ]  Expect to push on the 'wrong' foot due to engine rotation direction

[ ]  Rotate .... 120 km/h (65 kt)

NOTE: Take-off roll in ISA conditions is 590 ft at maximum weight.

After Takeoff

[ ]  Raise gear .... BELOW 200 km/h (108 kt)

[ ]  After selecting gear up, hold lever until 'three reds', then return lever to neutral

Circuit and Approach

[ ]  Lower gear .... AT 200 km/h (108 kt)

[ ]  After selecting gear down, hold lever until 'three greens', then return lever to neutral

[ ]  Extend flaps .... AT/OR BELOW 170 km/h (92 kt)

[ ]  Fly approach .... 170 km/h (92 kt) slowing to 120 km/h (65 kt) at threshold

NOTE: Without flaps, forward view from rear seat is virtually nil; a curved approach is needed.

Landing

[ ]  Keep the canopy lock pin in place in the rail hole

[ ]  Lower nose after touchdown

NOTE: High wing-loading means no tendency to float.

NOTE: With practice, landing roll can be as short as 300 m (985 ft).

Forced Landing

[ ]  Leave gear retracted for dead-stick landing

NOTE: Standard drill is to leave gear retracted to flatten the descent profile.

NOTE: Brakes remain usable with gear up.

NOTE: With gear up, the undercarriage remains partially exposed to minimize damage in a wheels-up landing.

Reference

NOTE: Performance: climb to 3,000 ft in 98 seconds; cruise 140-145 kt at 75 per cent power; fuel burn just over 13 US gal/hr at 140 kt cruise.

NOTE: Endurance in excess of 2 hours with 45 minutes reserve; maximum range 275 nm at slightly reduced cruise speed.

NOTE: G limits: unlimited aerobatics certified at max takeoff weight 1,305 kg.

NOTE: Engine: Ivchenko M-14P 9-cylinder radial, 360 hp.

NOTE: Pneumatic system used for engine starting, flaps, gear, and brakes.

NOTE: RPM gauge is calibrated in percent of maximum rather than RPM.

NOTE: Artificial horizon is inverted in presentation: top half of ball is brown, bottom half is blue.

NOTE: Rear cockpit has controls to deliberately fail certain systems in the front cockpit.

NOTE: Gear lever is pneumatic and returns to central neutral position when not required.

NOTE: Brakes are operated by a stick-mounted lever; pneumatic brake noise resembles a bus/lorry stopping.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Yakovlev/Yak 52/yak52_fltrep52.txt

Rights: licensed

Checklist hash: 0f28b1a9f2d5130aba7a6dcd55653cced8366193b967dd41ac34fcfede2fdddb

Verify against the aircraft's own approved documentation before use.