Yakovlev Yak 52 — yak52_yak52shortch

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

EXTERNAL

[ ]  Fuel quantity

[ ]  Oil Quantity, min 8 lts

[ ]  Prop alignment marks "0"

[ ]  Prop Area clear

[ ]  Cowl Shutters

[ ]  Fuselage FOD

[ ]  Oil Leaks

[ ]  Gear Struts

[ ]  Cowl Latches

[ ]  Pitot cover .... OFF

[ ]  Static Ports

[ ]  Flaps

[ ]  Drain Fuel

[ ]  Control Surfaces

[ ]  Ail Bearings

[ ]  Oil Cooler Cover

[ ]  Manifold drain tap .... shut

[ ]  Configure cockpits

REAR

[ ]  Mag Control UP (1st cabin)

[ ]  Gear .... NEUTRAL

[ ]  Flaps .... NEUTRAL

[ ]  Intsr. Switches .... DOWN

[ ]  Mags .... OFF

[ ]  Brake Override .... OFF

[ ]  Parachute Secured

[ ]  Straps Tied and Secure

FRONT

[ ]  Mags .... OFF (0)

[ ]  All Switches .... OFF

[ ]  Gear .... DOWN & Latched

[ ]  Brakes .... ON

[ ]  Main air supply .... ON

[ ]  Prime as required (cyls)

[ ]  Pull through blades as req.

WARNING: Do not touch the propeller if the engine is hot!

WARNING: TREAT PROP AS LIVE!

[ ]  Rear Mags .... 1+2

[ ]  Rear canopy .... CLOSED

STARTING

[ ]  Air supply Re-check

[ ]  Air supply .... ON

[ ]  Brakes re-check

[ ]  Brakes .... ON

[ ]  Master switch .... ON Generator ON

[ ]  Ignition .... ON Engine Instruments ON

[ ]  U/C switch .... ON

[ ]  Throttle .... SET

[ ]  Prop .... FINE

[ ]  Fire valve .... ON

[ ]  Enunciator lights .... TEST

[ ]  Carb heat .... OFF

[ ]  Shutters .... OPEN

[ ]  Prime LEFT Fuel pressure

[ ]  Prime right as req'd

[ ]  Check behind "Clear prop"

[ ]  Press starter

[ ]  Mags to BOTH (1+2)

[ ]  Prime as req'd then lock

WARNING: OIL PRESSURE!!!

[ ]  Warm up at .... 40-45%

[ ]  Cooling gills .... CLOSED Electrical systems ON

WARM UP

[ ]  Monitor temps and Pressures

[ ]  Set cooling controls as required

[ ]  Switch on and test avionics

POWER CHECKS

[ ]  Brakes .... ON

[ ]  Clear behind, T's & P's

[ ]  Shutters As required

[ ]  Power .... 70%

[ ]  Brakes holding

[ ]  Exercise Prop .... (54%)

[ ]  Mags .... (3% max drop)

[ ]  T's & P's .... CHECK

[ ]  *Ammeter Charging & 28v

[ ]  *Carb heat 1% RPM, + carb temp inc.

[ ]  Idle .... 28%

[ ]  Set .... 35%

NOTE: * (can do at 60%)

VITAL ACTIONS — WORK LEFT TO RIGHT

[ ]  Air .... ON

[ ]  Electric's .... ON

[ ]  Trimmer .... SET

[ ]  Flaps .... UP

[ ]  Prop .... FINE

[ ]  Fuel .... ON

[ ]  Lights .... PTT

[ ]  Mags .... 1+2

[ ]  Instruments .... Check

[ ]  Fuel Contents

[ ]  Primer .... Neutral

[ ]  Shutters .... Open & locked

[ ]  Carb heat .... Cold

[ ]  Controls .... Full Free

[ ]  Harnesses .... Secure

[ ]  Canopy As req.

[ ]  Strobes .... ON

[ ]  Transponder .... ON

TAKEOFF

[ ]  VR .... 100kph

[ ]  Climb .... 170 kph

[ ]  Gear .... UP

[ ]  Power to 82%, 80 cms or as req.

[ ]  T's & P's .... Check

BEFORE LANDING

[ ]  Power to 70% & 40 cms

[ ]  Abeam touchdown area

[ ]  200 kph GEAR .... DOWN, check lights and poles.

[ ]  170 kph Flaps

[ ]  Approach .... 150kph

[ ]  PUFA CHECK on finals

AFTER LANDING

[ ]  Taxi clear of r'way & STOP.

[ ]  Strobes & TX .... OFF

[ ]  FLAPS Identify and raise

[ ]  Shutters .... Open

[ ]  Converters .... OFF

SHUT DOWN

[ ]  Cool engine to 150 CHT

[ ]  Electric's .... OFF

[ ]  Mags .... OFF

[ ]  Air tap & shutters Close

[ ]  Water drain. Bleed

[ ]  Control locks. As req.

[ ]  Manifold drain .... OPEN

USE ONLY AS A GUIDELINE

NOTE: THIS IS NOT AN APPROVED CHECKLIST.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Yakovlev/Yak 52/yak52_yak52shortch.txt

Rights: licensed

Checklist hash: aa676be885ffe46c975fa4e2a664a9c709ceb07db64cddfd72f15e82d6b31f82

Verify against the aircraft's own approved documentation before use.