Yakovlev Yak 52TW — Yak52TW_W&B_steve_dalton

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Weight and Balance" item count differs between passes (43 vs 45); verify nothing was dropped.

Weight and Balance

ENTER AMOUNTS IN YELLOW CELLS

N52SD

S/N: 0112226

DATE: 18 MAY 2002

[ ]  Empty weight of Aircraft .... 2264.3 lbs.

[ ]  Empty weight of Aircraft arm .... 46.170532614936185 in

[ ]  Empty weight of Aircraft moment .... 104543.937 in-lbs.

[ ]  Oil (1.9 lb/qt) .... 0 Quarts

[ ]  Oil weight .... 0 lbs.

[ ]  Oil arm .... 4.59 in

[ ]  Oil moment .... 0 in-lbs.

[ ]  Main Fuel .... 0 Gallons

[ ]  Main Fuel weight .... 0 lbs.

[ ]  Main Fuel arm .... 45.15 in.

[ ]  Main Fuel moment .... 0 in-lbs

[ ]  Aux. Fuel .... 0 Gallons

[ ]  Aux. Fuel weight .... 0 lbs.

[ ]  Aux. Fuel arm .... 71.76 in.

[ ]  Aux. Fuel moment .... 0 in-lbs

[ ]  Smoke Oil .... 0 Gallons

[ ]  Smoke Oil weight .... 0 lbs.

[ ]  Smoke Oil arm .... 65 in.

[ ]  Smoke Oil moment .... 0 in-lbs

[ ]  Front Pilot .... 0 lbs.

[ ]  Front Pilot arm .... 48.87 in

[ ]  Front Pilot moment .... 0 in-lbs

[ ]  Rear Pilot .... 0 lbs.

[ ]  Rear Pilot arm .... 99.48 in.

[ ]  Rear Pilot moment .... 0 in-lbs

[ ]  Baggage .... 0 lbs.

[ ]  Baggage arm .... 117.79 in.

[ ]  Baggage moment .... 0 in-lbs

[ ]  Total weight .... 2264.3 lbs.

[ ]  Total arm .... 46.170532614936185 in.

[ ]  Total moment .... 104543.937 in-lbs

[ ]  Gear up? (YES or NO): .... NO

[ ]  % M.A.C. .... 0.22532956150009692

[ ]  Within Normal Limits?: .... YES

[ ]  Within Acro Limits?: .... YES

NOTE: Maximum Gross weight is 3,012 Lbs. (1,366 Kg.)

NOTE: Aerobatic C.G. Limits (17.5% to 28% M.A.C.)

NOTE: Normal C.G. Limits (17.5% to 31% M.A.C.)

NOTE: Gear extension moves the C.G. aft .5% M.A.C.

Weighing Information

ENTER WEIGHTS IN YELLOW CELLS

[ ]  Left Main weight .... 1086 lbs.

[ ]  Left Main arm .... 33.56 in.

[ ]  Left Main moment .... 36446.16 in-lbs

[ ]  Right Main weight .... 1082 lbs.

[ ]  Right Main arm .... 33.56 in.

[ ]  Right Main moment .... 36311.920000000006 in-lbs

[ ]  Tail weight .... 121 lbs.

[ ]  Tail arm .... 263.63 in.

[ ]  Tail moment .... 31899.23 in-lbs

[ ]  Oil (1.9 lb/qt) .... 13 qt.

[ ]  Oil arm .... 4.59 in

[ ]  Oil moment .... 113.373 in-lb

[ ]  Main Fuel .... 0 gal.

[ ]  Main Fuel arm .... 45.15 in.

[ ]  Main Fuel moment .... 0 in-lb

[ ]  Aux. Fuel .... 0 gal.

[ ]  Aux. Fuel arm .... 68.38 in.

[ ]  Aux. Fuel moment .... 0 in-lb

[ ]  Empty Totals weight .... 2264.3 lbs.

[ ]  Empty Totals arm .... 46.170532614936185 in.

[ ]  Empty Totals moment .... 104543.937 in-lbs

NOTE: Datum is front jacking point on firewall

NOTE: Level aircraft using the front cockpit canopy rail

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Yakovlev/Yak 52TW/Yak52TW_W&B_steve_dalton.txt

Rights: licensed

Checklist hash: 5fed48ff1c8c57b303204711136bcc248ab3a20ad0612714da0687d99e8f5194

Verify against the aircraft's own approved documentation before use.